# 3GPP TSG-SA3 Meeting #109 AdHoc-e	S3-230192

# e-meeting, 16th – 20th January, 2023

# Source:	OPPO

# Title:	Address an EN in Sol #11

# Document for:	Approval

Agenda Item:	5.9

1	Decision/action requested

This pCR proposes to address an Editor’s Note in solution #11.

2	References

N/A

3	Rationale

This contribution proposes to resolve an EN in Solution #11: Authentication mechanism selection procedure among EEC, ECS, and EES.

Editor’s Note: It if FFS to consider the security protection of selection messages between EEC and ECS.

To protect EEC’s security and mechanism chosen by EEC in the selection message, the ECS can send these messages to the EEC after the authentication and TLS connection establishment, thus the EEC can verify its security capability and chosen mechanism have not been tampered with.

4	Detailed proposal

********* Begin 1st change*********

## 6.11	Solution #11: Authentication mechanism selection procedure among EEC, ECS, and EES

### 6.11.1	Solution overview

This solution addresses security requirement for authentication mechanism selection between EEC and ECS, EEC and EES in key issue #2.2.

### 6.11.2	Solution details

The EEC should be configured with the security capability according to the local configuration (e.g., TLS with AKMA [3], TLS with GBA [4], or other TLS authentication methods). The ECS and EES should be separately configured via network management with mechanisms that are are allowed.

The EES provides the supported authentication mechanism(s) to the ECS during the EES registration procedure in clause 8.4.4.2.2 in TS 23.558[1], and the ECS stores the security capability of the registered EES.

The ECS provisions the Edge configuration information to the EEC which contains the information for establishing a connection with EESs (such as URI), in the Service provisioning procedure as specified in clause 8.3.3 of TS 23.558[1]. ECS helps the authentication mechanism selection between EEC and EES, and contains the selection result in the Edge configuration information, to establish the security connection between EEC and EES.

Figure 6.3.11.2-1: Procedure for authentication mechanism selection among EEC, EES, and ECS

Step 0: The EEC is pre-configured with or has discovered the address (e.g. URI) of the ECS.

Step 1. The EEC chooses an authentication mechanism, and sends an Authentication Mechanism Selection Request message to the ECS, including EEC security capability, the chosen authentication mechanism, and may include the UE identifier such as GPSI, connectivity information, UE location and AC profile(s) information. Step 2a. The ECS stores the security capability of EEC, and checks if it supports the authentication mechanism chosen by EEC.

Step 2b. The ECS may utilize the capabilities (e.g. UE location) of the 3GPP core network or the profile(s) provided by the EEC, to identify the EES as specified in clause 8.3.3.2 of TS 23.558[1]. With the security capability of the identified EES stored in EES registration, and the receiving security capability of the EEC in step 1, the ECS checks if the identified EES supports the authentication mechanism chosen by EEC.

Step3. The ECS sends the Authentication Mechanism Selection Request message to the identified EES, including EEC security capability, and the authentication mechanism chosen by EEC.

Editor’s Note: whether ECS should send the EEC’s security capability and EEC chosen method to EES is FFS

Step4. If ECS supports any mechanism in EEC's security capability, ECS may use the authentication mechanism EEC chooses or another mechanism in EEC's security capability (e.g., based on local policy), ECS should sends EEC the Authentication Mechanism Selection completes message including the selection result. Otherwise the ECS should reply with a failure indication.

ECS helps the identified EES to select the authentication mechanism based on the security capability of EEC and EES, and authentication mechanism chosen by EEC, and the selection result of ECS should be contained in the Edge configuration information, and provide to the EEC in the Service provisioning procedure as specified in clause 8.3.2.2 of TS 23.5558[1], for the EEC to establish security connection with the EES. If the EES could not support any mechanism in EEC's security capability, ECS sends a failure indication to EEC.

Step5. Upon receiving the Authentication Mechanism Selection Complete message from ECS with the selection result, EEC starts using the mechanism indicated in selection result to authenticate and establish TLS connection with ECS. Otherwise the authentication mechanism selection failed between EEC and ECS.

Step6. After the authentication and TLS connection establishment between EEC and ECS, the ECS sends back EEC’s security capability and chosen mechanism to the EEC, and this message can be security protected by the TLS context. Thus, the EEC can verify the message had not been tampered with.

### 6.11.3	Solution evaluation

TBD

Editor's Note: How to consider security capabilities of UEs and PLMNs in the negotiation is FFS.

Editor’s Note: it is FFS how to solve the authentication selection failure case if there do not exist the same authentication mechanisms.

********* End 1st change*********