3GPP TSG-SA3 Meeting #109AdHoc-e 	draft-S3-230066-r1 merger of S3-230066, S3-230181, S3-230281

- Electronic meeting, 16 - 20 January 2023

- Source:	Lenovo, Qualcomm?, Huawei?, Interdigital?

- Title:	Conclusion to KI#1 Direct C2 Security

- Document for:	Approval

Agenda Item:	5.17

# 1	Decision/action requested

This pCR proposes the conclusion to KI#1 on Direct C2 Security

# 2	References

[1]	3GPP TR 33.891, ‘Study on Security of Phase 2 for UAS, UAV and UAM’, Release 18.

[2]	3GPP TR 23.700-58, ‘Study of further architecture enhancements for uncrewed aerial systems and urban air mobility’, Release 18.

[3]	3GPP TS 23.256, ‘Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2’, Release 17.

[4]	3GPP TS 33.256, ‘Security aspects of Uncrewed Aerial Systems (UAS)’, Release 17.

[5]	3GPP TS 33.536, ‘Security aspects of 3GPP support for advanced Vehicle-to-Everything (V2X) services’, Release 17.

[6]	3GPP TS 33.503, ‘Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)’, Release 17.

# 3	Rationale

The conclusion to ‘Key issue #1: Direct C2 Security’ is proposed in this contribution considering the following aspects:

- TR 33.891 Solutions 1,2,3 and 4 related to KI#1 propose to establish secure PC5 unicast link for C2 communication (between UAV and UAV-C) by reusing the unicast security establishment procedure specified in eV2X [5] as baseline.

- Further according to TS 33.536 [5], Clause 5.3.3.1.4.2.3	 Security policy handling, it states the following:

‘For a NR PC5 unicast link, the UE shall be provisioned with the following:

-	The list of V2X services, e.g. PSIDs or ITS-AIDs of the V2X applications, with Geographical Area(s) and their security policy which indicates the following:

-	Signalling integrity protection: REQUIRED/PREFERRED/NOT NEEDED

-	Signalling confidentiality protection: REQUIRED/PREFERRED/NOT NEEDED

-	User plane integrity protection: REQUIRED/PREFERRED/NOT NEEDED

-	User plane confidentiality protection: REQUIRED/PREFERRED/NOT NEEDED

NOTE 1:	No integrity protection on signalling traffic enables services that do not require security.

NOTE 2:	Ensuring that only a connection with security is used for a V2X service is guaranteed if the signalling integrity security policy of at least one of the UEs for that V2X service is set to REQUIRED. It is recommended to set this security policy to REQUIRED in order to guarantee security protection.’

- According to SA2 TR 23.700-58 Clause 8.1 Conclusion for Key Issue #1 related to ‘Transport C2 communication over PC5 interface’, states the following:

The Unicast mode 5G Prose Direct Communication procedure as defined in clause 6.4.3 of TS 23.304 [8] is used as the baseline for establishing C2 communication over PC5, with the following enhancements and adaptations:

-	Prose Service info is replaced with "C2 Communication Service".

-	C2 Communication Service identifier may be pre-configured or derived from the CAA-level UAV ID.

Both UE-oriented and service-oriented unicast link establishment are supported as defined in clause 6.4.3.1 of TS 23.304 [8].

- The security policy in the above references [5] is related to V2X service, where KI#1 in [1] is about C2 service. Even though the direct communication establishment should reuse existing procedures defined for Prose [6] and V2X [5], C2 service specific information (such as security policy specific to the C2 service) should be used during the direct communication establishment.

This contribution includes merger of S3-230024 (Interdigital), S3-230066 (Lenovo), S3-230181 (Huawei), S3-230281 (Qualcomm).

# 4	Detailed proposal

SA3 is kindly requested to agree on the pCR below to TR 33.891

*****Start of Change 1*****

## 7.1	Conclusion for Key issue #1

To address ‘Key issue #1: Direct C2 Security’, following aspects are recommended for the normative work:

A secure PC5 unicast link for C2 service related communication can be established by re-using security establishment procedures defined in TS 33.536 [5] and TS 33.503[6] as baseline with the following adaptations.

- C2 service specific security policy is used for the security establishment.

- In the case that PC5 based C2 communication happens following the C2 communication authorisation procedure in TS 23.256 [4], the network support transport of USS provided security materials used for PC5 unicast security establishment.

- Further C2 service specific adaptations (e.g., usage of UAV ID such as CAA-level UAV ID) if any related to security establishment are upto the normative work.

*****End of Change 1*****