3GPP TSG-SA3 Meeting #109AdHoc-e 	draft_S3-230070-r2

- Electronic meeting, 16 - 20 January 2023

- Source:	Lenovo

- Title:	Solution to address KI#2

- Document for:	Approval

Agenda Item:	5.11

# 1	Decision/action requested

This pCR provides a Solution to address KI#2 in TR 33.884.

# 2	References

[1]	3GPP TR 33.884, ‘Study on security of application enablement aspects for subscriber-aware northbound API access (FS_SNAAPPY)’, Release 18.

[2]	3GPP TS 33.122, ‘Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs’, Release 17.

# 3	Rationale

The pCR provides a solution to address KI#2.

# 4	Detailed proposal

SA3 is kindly requested to agree on the pCR below to TR 33.884

*****Start of Change 1*****

## 6.Y	Solution #Y: Resource Owner based authorization for resource access

### 6.Y.1	Introduction

The solution addresses Key Issue #2.

The solution describes the method to receive and revoke authorization as required from an authenticated Resource owner to control access to resource(s) of a resource owner. Figure 6.Y.2-1 shows the resource authorization procedure to allow access to resources.

### 6.Y.2	Solution details

Figure 6.Y.2-1: Resource authorization procedure to allow access to resources

The steps shown in Figure 6.Y.2-1 is described as follows:

1-2. During the primary authentication procedure, the UDM may also indicate to the AUSF whether CAPIF keys need to be generated for the UE (i.e, resource owner). If the CAPIF Indication is included, the UDM may also include GPSI and CAPIF Function information (i.e., ID/address).

3. If the AUSF receives the CAPIF indication from the UDM, the AUSF and UE following a successful primary authentication can generate CAPIF Key and an ID (to identify the CAPIF Key for the UE) from the AKMA Key.

NOTE: The inputs used in the CAPIF key and ID generation can be upto the normative phase.

4a. The AUSF based on CAPIF Function information, sends resource owner authorization data notification request to the CAPIF function which can include the UE ID (i.e., GPSI), CAPIF key and the ID.

4b. The CAPIF Function stores the received information and sends a sends resource owner authorization data notification response to the AUSF.

5. The AUSF sends the CAPIF Function information and GPSI to the AMF and the AMF forwards the information to the UE over the NAS transport.

6a. The UE initiates a resource owner data notification trigger with GPSI and a freshness parameter to the CAPIF function.

Optionally, prior to step 6a, the UE may trigger resource owner registration request with the CAPIF function by sending a GPSI and then can establish a secure connection based on CAPIF Key. The CAPIF function then sends a resource owner ID and lifetime (for the resource owner registration) following a successful resource owner registration.

6b. The CAPIF function and the UE (resource owner) establishes secure connection using the resource owner key generated from the CAPIF Key, the freshness parameter and using and other inputs if requried.

7. Resource owner authorization data can be fetched or handled over the established secure connection. The resource owner authorization data fetching, and handling can be upto operator implementation.

### 6.Y.3	Evaluation

The solution addresses Key Issue #2 and enables the following:

Mutual authentication and secure session establishment between the resource owner and the CAPIF Function (i.e., an authorization function) need to be supported based on CAPIF Key derived from the UE 5G security context.

Access to resource of resource owner is allowed based on resource owner specific authorization information (i.e., user consent data) verification by the network.

Editor Note: Further evaluation is FFS.

*****End of Change 1*****