3GPP TSG-SA3 Meeting #109AdHoc-e 	draft_S3-230260-r246

- Electronic meeting, 16 - 20 January 2023

- Source:	Ericsson

- Title:	Update of evaluation to solution #2

- Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

This document proposes to update the evaluation to solution #2.

# 2	References

[1]	3GPP TR 23.700-86

[2]	3GPP TR 33.893

# 3	Rationale

This contribution proposes to update evaluation to solution #2.

# 4	Detailed proposal

**** START OF CHANGE ****

## 6.2	Solution #2: Authorization of 5GC NF for Ranging/SL positioning service exposure

### 6.2.1	Introduction

This solution addresses Key Issue #2 on Authorization for Ranging/SL Positioning service. Specifically, it addresses the second requirement in KI#2: “The 5G Ranging/SL positioning system shall be able to support the authorization of a network function for triggering Ranging/Sidelink Positioning services and obtaining the location information”.

According to TR 23.700-86 [2], 5GC NF is enabled to initiate SL positioning service to obtain the accurate location of a UE or obtain the distance between two UEs, for which the 5GC NF needs to be authorized at two levels:

- The first level of authorization is for service access, i.e. the 5GC NF should be authorized to request the SL positioning service. This level of authorization could be achieved by existing OAuth token-based authorization defined in TS 33.501 [8].

- The second level of authorization is for preserving UE privacy. This is because, even if the 5GC NF is authorized to request the SL positioning service, it does not mean that the requested service can always be exposed to the 5GC NF. For example, it is possible that the 5GC NF is allowed to request SL positioning info between UE1 and UE2, but may not be allowed to request SL positioning info between UE3 and UE4. Without further check on the authorization info of the involved UEs, there is still the risk that unauthorized SL positioning information could be exposed to the 5GC NF. Therefore, the 5GC NF needs to be further authorized on whether it is allowed to acquire position information of the involved specific UEs.

This solution introduces a method for authorizing the 5GC NF which initiates SL positioning service targeting at specific UEs, assuming the SL positioning service procedure does not rely on the existing procedure for enhanced location service.

### 6.2.2	Solution details

The solution proposes that, when the 5GC NF checks with the UDM to discover the serving AMF(s) of the UEs, it also needs to check with the UDM about authorization information of the UEs for acquiring SL positioning information of the UEs, based on e.g. the privacy related parameters in UE’s subscription data stored in the UDM/UDR.

UE1 and UE2 refer to SL Reference UE and Target UE respectively for SL Positioning service.

1.	Service authorization and policy/parameters provisioning procedure is performed between UE1/UE2 and the network respectively.

2.	The 5GC NF interacts with the UDM to check the authorization info of UE1/UE2 via Nudm_SDM_Get service operation.

NOTE:	regardless of whether UE1/UE2 are managed by different UDMs, separate Nudm_SDM_Get message needs to be sent as the API applies only for one target UE and for checking that UE’s subscription.

3.	The UDM checks the authorization info of UE1 and UE2 against their subscription data respectively based on the UE1 ID and UE2 ID.

4.	The UDM returns the authorization results of the UEs to the 5GC NF.

5.	The 5GC NF checks the authorization results of both UEs. Since SL Positioning service concerns location of the UE, it could be possible that the authorization info of the UE is location specific (e.g. the UE allows its location to be exposed in area A but does not allow its location to be exposed in area B).

If none of the UE grants or one of the UEs does not grant authorization for Ranging/SL positioning, the 5GC NF aborts the Ranging/SL positioning service.

If both UEs grant authorization without location restriction, the 5GC NF proceeds to step #11.

Conditionally, if both UEs grant authorization which is restricted in a certain area, the 5GC NF proceeds to step #6.

If the 5GC NF is the AMF of the UEs, the 5GC NF skips the steps #6~#9 and proceeds to step #10.

Figure 6.2.2-1:	Authorization of the 5GC NF for Service Exposure

6.	[Conditional] The 5GC NF sends the Nudm_ParameterProvision_Get Request to the UDM for requesting the coarse location of UE1/UE2 (e.g. TAI or Cell-ID of UE1/UE2).

NOTE:	Whether Nudm_ParameterProvision can be used to retrieve location info of UE needs to be coordinated with SA2.

7.	[Conditional] The UDM sends the Namf_Location_ProvideLocationInfo Request to the AMF.

NOTE:	regardless of whether UE1/UE2 are managed by different or same AMF(s), separate Namf_Location_ProvideLocationInfo Response messages needs to be sent to the AMF as the API applies only for one UE.

8.	[Conditional] The AMF of UE1/UE2 responds with the Namf_Location_ProvideLocationInfo Response to the UDM which contains the coarse location of UE1/UE2 (e.g. TAI or Cell-ID of UE1/UE2).

9.	[Conditional] The UDM returns the Nudm_ParameterProvision_Get Response to the 5GC NF which contains the coarse location of UE1/UE2 (e.g. TAI or Cell-ID of UE1/UE2).

10.	[Conditional] Based on the coarse location of UE1/UE2, the 5GC NF checks whether the UE1/UE2 is within the area for requesting SL Positioning service.

11. 	If both UEs grant authorization in their current locations, the 5GC NF sends the SL Positioning service request to the AMF of one of the UEs (e.g. UE1). If the 5GC NF is the AMF, this step can be skipped.

12~15.	 The rest of the SL Positioning procedure is performed between the network and the UE.

Alternatively, after the 5GC NF checks the authorization info of both UEs in step #5, if both UEs grant authorization which is however restricted in a certain area, the 5GC NF sends the Namf_Location_ProvideLocationInfo Request to the AMF directly rather than through the UDM. Then the AMF responds the Namf_Location_ProvideLocationInfo Response to the 5GC NF directly rather than through the UDM.

NOTE:	The area granularity could be detailed in the UE privacy profile for SL positioning in UE subscription.

### 6.2.3	Evaluation

This solution addresses the second requirement “The 5G Ranging/SL positioning system shall be able to support the authorization of a network function for triggering Ranging/Sidelink Positioning services and obtaining the location information” in KI#2.

On top of the existing authorization of the 5GC NF on service level, the authorization in this solution is further performed on specific UE level, which ensures the privacy of all involved UEs in a service, as Ranging/SL Positioning services request UE location information which is privacy sensitive.

This solution does no assume the availability of the architecture supporting location based services, and relies on the requesting 5GC NF itself to check with the UDM for the authorization information of the involved UEs. Therefore, mainly the requesting 5GC NF and the UDM are impacted in this solution.

This solution is not aligned with SA2 conclusions for KI#7 in SA2 TR 23.700-86 where the LCS architecture and MT-LR procedures are taken as baseline. It is unclear how the network assisted positioning procedure can work without the LMF and GMLC. Also, the Nudm_ParameterProvision_Get Request is not supposed to retrieve location data of UE as per its definition in clause 5.2.3.6 in TS 23.503.

Editor’s note: Whether 5G NF can trigger location service without GMLC needs to be coordinated with SA2.

All the steps for authorization in steps 3-59 would be new, with impact on UDM and 5GC NF.

NOTE:	Whether the authorization is based on existing UE LCS privacy profile in UDM or whether its new data or profile is to be decided in normative work,

and needs to be coordinated with SA2.

Editor’s note: Further evaluation is FFS.

**** END OF CHANGE ****