3GPP TSG-SA3 Meeting #109AdHoc-e	S3-230291r1

- Electronic meeting, 16 - 20 January 2023										revision of S3-22xxxx

- Source:	Qualcomm Incorporated

- Title:	Proposed addition to the conclusions

- Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

This contribution provides an addition to the conclusion.

# 2	References

[1]	3GPP TR 33.741 v0.4.0 (S3-224016)

# 3	Rationale

This contribution provides an addition to the conclusion. It has been agreed that the UDM can trigger a primary authentication but the UDM is not always in possession of the information to request such a triggering. For example, the AKMA function may be aware of a subscription that is using AKMA but for which no ‘recent’ primary authentication has been rum. In such a case, it is useful for a home operator to be allowed to have the AKMA function to trigger a primary authentication if the home operator so desires.

# 4	Detailed proposal

It is proposed that the following pCR is included in the TR [1].

**** START OF CHANGES ****

- 6	Conclusions

It is recommended to specify a new Home Network Triggered Authentication procedure for which the UDM is the central point that decides when to trigger the procedure, and its behaviour is controlled by the Home Network. That means that the UDM triggers interaction with the serving network to request the execution of primary authentication.

In addition, an internal NF that is authorised by the home operator can inform the UDM to trigger a primary authentication.

**** END OF CHANGES ****