- 3GPP TSG-SA3 Meeting #109adhoc-e	S3-230307

- e-Meeting, 16 – 20 January 2023

- Source:	Apple, Nokia

- Title:	New key issue on secure retrieval of UE id

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

This contribution proposes to add a new key issue in TR 33.739

# 2	References

[1] TS 23.501

[2] TS 23.502

# 3	Rationale

This contribution provides a new key issue to TR 33.739

# 4	Detailed proposal

It is suggested to approve the following changes.

***************************Start of Change ****************************

### 5.2.x	Key issue #1.x: Key issue on secure retrieval of UE id

#### 5.2.x.1	Key issue details

According to SA6 requirements and SA2 specification the Edge Application Server (EAS) is capable to retrieve UE id from a network identifier (e.g. IP address, MAC address) to be used in subsequent API calls.

More specifically, in TS 23.501 Clause 5.20 describes how NEF may determine the Permanent identifier of the UE, and also how NEF may provide an AF specific UE identifier to the AF by Nnef_UEId service.

The NEF may determine the Permanent identifier of the UE, as described in clause 4.15.3.2.13 of TS 23.502 [3], based on:

-	the address of the UE as provided by the AF; this may be an IP address or a MAC address;

-	the corresponding DNN and/or S-NSSAI information: this may have been provided by the AF or determined by the NEF based on the requesting AF; this is needed if the UE address is an IP address.

The NEF may provide an AF specific UE Identifier to the AF:

-	that has explicitly requested a translation from the address of the UE to a unique UE identifier (via Nnef_UEId service); or

-	that has implicitly requested a translation from the address of the UE to an AF specific UE Identifier by requesting external exposure about an individual UE identified by its address.

”

There are some requirements in TS 23.501 on the AF specific identifier:

“

The AF may have its own means to maintain the AF specific UE Identifier through, e.g. an AF session. After the retrieval of an AF specific UE Identifier the AF shall not keep maintaining a mapping between this identifier and the UE IP address as this mapping may change.

The AF specific UE Identifier shall not correspond to a MSISDN; it is represented as a GPSI in the form of an External Identifier. When used as an AF specific UE identifier, the External Identifier provided by the 5GCN shall be different for different AF.

”

Note worthy SA3 did not conclude the security aspects related to the UE id retrieval and whether AF spefic Id resolves all privacy issues, thus the Key Issue proposes to revisit this topic and to study how the Edge Application Server can access the Edge Enablement Server without violating UE privacy.

#### 5.2.x.2	Security threats

Without proper security mechanisms Nnef_UEId service can be abused to track UEs, thus compromising the privacy of the UE.

#### 5.2.x.3	Potential security requirement

5G system should provide adequate security mechanisms to prevent the tracking of the UE and potential privacy breaches through retrieving UE IDs

***************************End of change****************************