3GPP TSG-SA3 Meeting #109AdHoc-e 	draft_S3-230340-r2

- Electronic meeting, 16 - 20 January 2023

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Evaluation for Solution #6

- Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

This contribution proposes evaluation for solution #6 in TR 33.738. It is requested to approve this proposal.

# 2	References

# 3	Rationale

This contribution proposes the evaluation of solution #6 in TR 33.738.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.738.

***START OF CHANGES***

## 6.6	Solution #6: Anomalous NF behaviour detection by NWDAF

### 6.6.1	Introduction

This solution addresses key issue #4.

The 5GC supports distributed NF deployments in order to ensure that the NF can provide services from multiple locations. The infrastructure typically implemented on cloud platforms may be owned and operated by the PLMN operator himself, or they may be run on commercial public cloud platforms. It is always possible that the NFs behave in an anomalous manner in certain sites of such distributed infrastructure. Such anomalous behaviour can arise either due to configuration errors or internal data corruption, or due to an attack.

To allow an operator to manage its NF deployment efficiently, additional relevant log data from the NFs can be provided to the NWDAF, which can then be used by NWDAF in assisting the analysis and detection of the cause for an anomaly.

### 6.6.2	Solution details

#### 6.6.2.1 General

The OAM or some operator defined AF may subscribe/request to the NWDAF to notify/respond when it detects an abnormal behaviour for a particular NF instance (for example excessive resource usage, or consistent failure to provide its service). There can be various causes of abnormal behaviour such as an erroneous operation at a NF, an error at the side of the cloud provider where the NF is deployed, or an impending attack at a particular NF by a malicious entity in the system.

In order to analyse the root cause of the anomaly with certain degree of confidence, more information or related data can help. Thus, NWDAF requests inputs from different entities in the system (such as OAM, NRF and NFs) for detailed analysis. Security related data for analytics can also be collected by NWDAF from the NFs (or via OAM). The final output analytics is then sent to the OAM or the AF for adequate measures.

In order to facilitate the security related log data collection from NFs two different methods are proposed

NOTE: Defining the protocols used for log collection is not in the scope of this document

#### 6.6.2.2 Collection of security related log data of NFs via NFs EventExposure APIs

The procedure depicted in Figure 6.6.2.2-1 allows a consumer to request analytics from NWDAF for anomalous NF behaviour and its root cause.

Figure 6.6.2.2-1: NF anomalous behaviour analytics provided by NWDAF using security logs obtained from NFs EventExposure APIs

1.	The OAM or operator's AF sends a request/subscription to the NWDAF for NF anomalous behaviour analytics using either the Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe service operation.

2.	If the request is authorized, and in order to provide the requested analytics, the NWDAF may subscribe to OAM services to retrieve resource usage and NF resources configuration of all targeted NF instances, following the procedure specified in clause 6.2.3.2 in TS 23.288 [1].

3a.	The NWDAF subscribes to NRF to receive notification on changes, e.g., on the load and status of NF instances registered in NRF, using Nnrf_NFManagement_NFStatusSubscribe service operation for all targeted NF instances. NF instances are identified by their NF id.

3b.	NRF notifies NWDAF of changes on the load and status of the requested NF instances by using Nnrf_NFManagement_NFStatusNotify service operation.

4a.	The NWDAF subscribes or requests the additional security specific log info (as specified in the table 6.6.2.3-1) for a particular NF by invoking the Nnf_EventExposure_Subscribe service operation.

4b.	The NF then notifies the NWDAF (e.g. with the complete log report) by invoking Nnf_EventExposure_Notify service operation.

5.	The NWDAF derives the relevant analytics using the inputs provided by the OAM, NRF, and the NF (as specified in the table 6.6.2.3-2)

6.	The NWDAF provides requested NF anomalous information along with the corresponding root cause (for instance a malicious NF trying to attack other NF for denial of service, or an erroneous NF unable to provide service to other NFs) using either the Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Subscribe response, depending on the service used in step 1.

#### 6.6.2.3 Collection of security related log data of NFs via OAM

The procedure depicted in Figure 6.6.2.3-1 allows a consumer to request analytics from NWDAF for anomalous NF behaviour and its root cause using the security specific NF logs obtained from OAM.

Figure 6.6.2.3-1: NF anomalous behaviour analytics provided by NWDAF using security logs obtained from OAM

1.	The OAM or operator's AF sends a request/subscription to the NWDAF for NF anomalous behaviour analytics using either the Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe service operation.

2.	If the request is authorized, and in order to provide the requested analytics, the NWDAF may subscribe to OAM services to retrieve resource usage and NF resources configuration of all targeted NF instances, following the procedure specified in clause 6.2.3.2 in TS 23.288 [1]. The NWDAF further sends the request to the OAM to provide the security specific log info (as specified in the table 6.6.2.3-1).

3.  OAM then collects all the requested information along with the security logs from the NFs and provide it to NWDAF.

4a.	The NWDAF subscribes to NRF to receive notification on changes, e.g., on the load and status of NF instances registered in NRF, using Nnrf_NFManagement_NFStatusSubscribe service operation for all targeted NF instances. NF instances are identified by their NF id.

4b.	NRF notifies NWDAF of changes on the load and status of the requested NF instances by using Nnrf_NFManagement_NFStatusNotify service operation.

5.	The NWDAF derives the relevant analytics using the inputs provided by the OAM and NRF (as specified in the table 6.6.2.3-2)

6.	The NWDAF provides requested NF anomalous information along with the corresponding root cause (for instance a malicious NF trying to attack other NF for denial of service, or an erroneous NF unable to provide service to other NFs) using either the Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Subscribe response, depending on the service used in step 1.

Table 6.6.2.3-1 Security specific parameters provided by NFs (NF/NRF) to assist in detection

| Information | Description |
| --- | --- |
| Timestamp | A time stamp associated with the service request which was sent by the NF Service Consumer to a NF Service Producer. |
| NF Service Consumer / SCP Identifier | The consumer instance or the SCP which sends the service request along with the access token to the NF Service Producer. |
| NF Service Producer Identifier | The producer instance which receives the requests and which verifies the access token received along with the requests |
| Authorization status of NF Service Consumer | Indicated if a given NF Service Consumer is authorized to receive an access token or not, as provided by NRF. |
| Access Token Authenticity | Information such as, if access token provided is for the service request it is sent, and if it is generated for the NF Service Consumer which is requesting the service. |
| Access Token Validity | Verification result, i.e. whether the access token is valid or invalid. |
| Number of requests to access a service | Number of simultaneous requests received at the NF Service Producer for a particular time window. |
| Requested Service Name | Name of the service for which the requests had been received. |
| Service Response Confirmation | Confirmation whether the NF Service Producer was able to fulfil the service requests or not. |

Table 6.6.2.3-2 Inputs provided to NWDAF in assisting the detection of anomalous NF

| Information | Source | Description |
| --- | --- | --- |
| Security Log Data | OAM/NF | Additional security relevant log info as described in table 6.6.2.3-1. |
| NF Load | NRF | The load of specific NF instance(s) recorded in their NF profile as defined per TS 29.510 [8]. |
| NF status | NRF | The status of specific NF instance(s) recorded in their NF profile as defined per TS 29.510 [8]. |
| NF resource usage | OAM | The usage of assigned virtual resources for specific NF instance(s) (e.g., mean usage of virtual CPU, memory, disk) as defined in TS 28.552 [9] clause 5.7. |

Table 6.6.2.3-3 and Table 6.6.2.3-4 specifies the output analytics from NWDAF

Table 6.6.2.3-3: Anomalous NF behaviour statistics

| Information | Description |
| --- | --- |
| Exceptions (1..max) | List of observed exceptions |
| > Exception ID | The risk detected by NWDAF |
| > Exception category | Indication if the anomalous behaviour is an attack or geniune error |
| > Exception level | Scalar value indicating the severity of the abnormal behaviour |
| > List of target NF(s) | One or more NFs which are affected due to the anomoulous NF in the system |
| > List of anomolous NF(s) | One or more NFs which are the probable cause of the anomalous activity in the system (either because they are malicious or due to internal errors) |

Table 6.6.2.3-4: Anomalous NF behaviour predictions

| Information | Description |
| --- | --- |
| Exceptions (1..max) | List of predicted exceptions |
| > Exception ID | The risk detected by NWDAF |
| > Exception category | Indication if the anomalous behaviour is an attack or geniune error |
| > Exception level | Scalar value indicating the severity of the abnormal behaviour |
| > List of target NF(s) | One or more NFs which are affected due to the anomoulous NF in the system |
| > List of anomolous NF(s) | One or more NFs which are the probable cause of the anomalous activity in the system (either because they are malicious or due to internal errors) |
| > Confidence | Confidence of this prediction |

The inputs defined in Tables 6.6.2.3-1 and 6.6.2.3-2 can assist in the detection of anomalies, but how the anomaly is finally detected, e.g., used algorithms and associated procedures, is left to the implementation. Example of use: the authenticity of an access token sent by a NF Service Consumer, and/or the number of services of service requests being sent by the NF Service Consumer, can be instrumental in informing the system about whether a potentially erroneous or malicious NF Service Consumer is trying to access a resource for which it is not authorized, and/or trying to attack a NF Service Producer by multiple simultaneous requests.

Based on the input, an analytics function can monitor and find abnormalities in NF load or NF resource usage, which can result in an alert. Specific security log data can help to understand if this is normal behaviour or could be resulting from a NF that is behaving anomalous.

Thus, when NFs send service requests to other NFs, input data such as the number of service requests sent, the percentage of successful service requests, the percentage of successful access token verification and the serving NF load and resource usage can be used to provide training data for normal service requests reception and load.

Input data such as NF Service Consumer / SCP identifier, the NF Service Producer Identifier and the Requested Service Name can be used to identity the anomalous/erroneous NF in the system.

NOTE: The derivation of output from input depends on the algorithms used or the policy present. How to derive the output from input is up to implementation logic, which is out of scope of 3GPP.

### 6.6.3	Evaluation

The solution addresses the key issue #4 intended to study the detection of anomalous NFs by the network, by enabling the NWDAF in assisting the analytis and dectition of the cause for an anomaly in the 5G Core.

The solution proposes NWDAF to collect NF related data such as resource utilization, load information and additional security specific log data from different entities in the 5G system, such as OAM, NRF and NFs, to proceed with the detailed analysis.

The solution introduces two different methods of NF log data collection - by using the NFs EventExposure APIs, and by using OAM. However, the existing event exposure is used for UE related events, not for logs. Using the OAM method requires to first expose security logs of NFs to the OAM and then make them available for the NWDAF

.

The solution specifies parameters provided by the NFs (NF/NRF) and inputs from OAM to NWDAF to assist in the detection, as well as output analytics from NWDAF in terms of statistics and predictions.

How the anomalies are finally detected and confirmed, i.e., used algorithms and associated procedures, are left to implementation and operator decision and out of scope of this document. I.e., the performance and accuracy of the anomaly detection relies on the implementation. The NWDAF based solution assists in the detection, but cannot be considered the only source/mechanism in the final decision about the anomaly.

NOTE: How NWDAF can ensure false positive and false negative rates in case of abnormal NF behaviour detection depends on the implementation of specific algorithms and NF deployment.

************ END OF CHANGES ******************