3GPP TSG-SA3 Meeting #109AdHoc-e 	draft_S3-230344-r4

- Electronic meeting, 16 - 20 January 2023

- Source:	Nokia, Nokia Shanghai Bell, China Mobile (Suzhou) Software, Intel

- Title:	Adding conclusion on KI#3

- Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

This contribution proposes conclusion on KI#3 in TR 33.738

# 2	References

[1]	3GPP TR 33.738 “Study on security aspects of enablers for Network Automation for 5G - phase 3”

[2]	3GPP TR 23.700-81 “Study of Enablers for Network Automation for 5G System (5GS); Phase 3”

# 3	Rationale

This pCR proposes to use the ideas of solution #1, #2, #7,#10,#11.#12,#13 for protection of ML models and authorization of model retrieval to conclude KI#3.

The following contributions have been merged in this document: S3-230203, S3-230244.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.738.

***BEGIN OF First CHANGE***

# 7	Conclusions

## 7.X	Conclusion on Key Issue #3 "Security for AI/ML model storage and sharing"

For key issue#3, it is recommended to use the ideas of solution #1, #2, #7,#10,#11.#12,#13 for protection of AI/ML models and authorization of model retrieval:

-	Authorization of the model retrieval at the NRF uses OAuth 2.0 token-based authorization. The NRF uses information provided by the MTLF. The NF service consumer (NFc) (e.g., AnLF) is required to be registered in the NRF with vendor ID information in its profile.

-	The model is stored in encrypted format, unless both ADRF and MtLF belong to the same security domain and vendor.

-	As per request of Analytics Id by the NFc, the MTLF performs authorization of the corresponding model retrieval per selected model, and initiates the transfer of required cryptographic material to NFc to enable it to retrieve the model from the storage platform and decrypt the model.

NOTE: the mechanism to provide the model is to be defined by 3GPP SA2.

-	NF consumer(s) (MTLF or AnLF) uses the ADRF ML model retrieval service to retrieve the ML model or URL (where the model is stored) from ADRF. ADRF verifies that the requested AI/ML model can be retrieved by the NF consumer(s) (MTLF or AnLF).

-	Keys and algorithms to be used for end-to-end protection of ML models will be speficied during normative work.

NOTE: Key management is handled out of band and not specified.

*** END OF First CHANGE ***