- 3GPP TSG-SA3 Meeting #109Adhoc-e	drft_S3-230346-r1

- Online, 16 - 20 January 2023	revision of S3-230346

- Source:	CATT

- Title:	pCR to TR33.740 Update Solution17 for removing ENs

Agenda Item:	5.3

# 1	Decision/action requested

Approve the pCR below

# 2	References

# 3		Rationale

Part1: Remove the following EN for the following reasons:

Editor’s Note: Whether PCF can also be used to provision discovery security material is FFS.

This solution focuses on the discovery security materials exchanged between different PLMNs. In R-17 TS 33.503, interfaces for exchanging security materials between 5G DDNMFs and between 5G PKMFs are defined. If PCF is used to provision security materials, then new interface for exchanging security materials between PCFs has to be defined. We did not find stronger reasons to define new interface instead of using existing solutions. The original purpose of PCF is to provide various policies rather than security materials such as keys. Therefore, this contribution proposes to remove this EN without adding additional text.

Part2: Remove 2 ENs in the clause of evaluation.

See the changes in the clause of evaluation.

# 4	Detailed proposal

#### *** BEGIN CHANGES ***

## 6.17	Solution #17: U2U relay discovery security material retrieval and authorization across PLMNs

### 6.17.1	Introduction

This solution addresses Key Issue #1: Security for UE-to-UE Relay discovery and Key issue #3: Authorization in the UE-to-UE Relay Scenario. For Key Issue #1, this solution specifically addresses the security requirement:

The 5G System shall provide a means to securely provision the security materials for UE-to-UE Relay discovery.

This solution assumes that the discovery security materials used for direct discovery and U2U relay discovery are different. Therefore, U2U relay authorization can be executed during the U2U relay discovery key provision process. Otherwise this solution does not address the Key Issue #3.

This solution needs to extend the existing specification as follows:

-	Permit multiple roles in the restricted discovery request. (Currently, there is only one role (Announce, Monitor, Query or Response) in the restricted discovery request.)

-	Permit to return more than two sets security materials in the restricted discovery response. (Currently, the restricted discovery response can return code-sending-security-parameter, code-receiving-security-parameter or both.

This solution assumes that multiple ProSe Key Management Functions (PKMFs) are used to generate and provision discovery security materials for direct discovery and U2U relay discovery. The PKMF in this solution is also called 5G U2U Relay PKMF to distinguish it from other kinds of PKMFs. A 5G U2U Relay PKMF can provision various discovery security materials based on a 5G ProSe UE’s roles in U2U relay discovery procedures. 5G ProSe UEs will always retrieve discovery security materials from one 5G U2U Relay PKMF which address is provided by the PCF or 5G DDNMF. It is possible for a 5G U2U Relay PKMF to retrieve discovery security materials from other 5G U2U Relay PKMFs.

### 6.17.2	Solution details

Figure 6.17.2-1: U2U relay discovery security material retrieval and authorization procedure across PLMNs

Note:	This solution only covers the parameters related to security material retrieval.

0.	The 5G ProSe UE (Source UE/Target UE/Relay UE) gets the address of 5G DDNMF/5G U2U Relay PKMF from the PCF of its HPLMN, which is responsible for provisioning discovery security materials related to direct discovery and U2U relay discovery.

NOTE 1:	U2U relay discovery materials are provisioned by the 5G DDNMF or 5G U2U Relay PKMF can be determined in the conclusion phase or the normative work phase.

1.	The 5G ProSe UE establishes a secure connection with the 5G DDNMF/5G U2U Relay PKMF in its HPLMN and sends a U2U Relay Discovery Key Request to the 5G DDNMF/5G U2U Relay PKMF. The request includes User Info ID, Security capability, and UE's roles in RSC-specified relay service. The request should include RSC if discovery security materials for U2U relay discovery are required.

Security for the interface between the 5G ProSe UE and the 5G DDNMF/5G U2U Relay PKMF relies on Ua security if GBA specified in TS 33.220 [14] is used (see clause 5.2.3.4) or Ua* security if AKMA specified in TS 33.535 [15] is used (see clause 5.2.5.4).

In U2U relay discovery, a UE can be the role of Announcing UE or Monitoring UE in Model A, or the role of Discoveree UE or Discoverer UE in Model B.

2.	The 5G DDNMF/5G U2U Relay PKMF in the HPLMN may need to send an Authorization Check Request to the UDM/ProSe App Server to check whether the UE is allowed to play its declared roles. The request includes User Info ID and the role list. The request should include RSC if discovery security materials for U2U relay discovery are required.

NOTE 2:	Authorization check of U2U relay discovery is performed by the UDM or ProSe App Server can be determined in the conclusion phase or the normative work phase.

3.	The UDM/ProSe App Server checks whether the UE is allowed to play the roles it claims, and returns the allowed roles through Authorization Check Request

4.	If the security materials for the roles are not available locally, the 5G DDNMF/5G U2U Relay PKMF in the HPLMN may need to send an U2U Relay Discovery Key Request to another 5G DDNMF/5G U2U Relay PKMF in a VPLMN to obtain the security materials. The request includes User Info ID and role list. The request should include RSC if discovery security materials for U2U relay discovery are required.

5-6.	The 5G DDNMF/5G U2U Relay PKMF in the VPLMN may need to send an Authorization Check Request to the UDM/ProSe App Server to check whether the UE is allowed to play its declared roles.

7.	The 5G DDNMF/5G U2U Relay PKMF in the VPLMN gets the discovery security materials for the allowed roles in the RSC-specific U2U relay service, and returns them through U2U Relay Discovery Security Material Response. The discovery security materials related to U2U relay discovery are associated with the RSC. The 5G DDNMF/5G U2U Relay PKMF may include relay discovery security policies specifying how the provided security material is used in the U2U relay discovery procedure in the response. For example, a security policy can specify whether end-to-end protection needs to be enabled in relay discovery messages.

8.	The 5G DDNMF/5G U2U Relay PKMF in the HPLMN returns the discovery security materials obtained from the local and other PLMNs to the UE through U2U Relay Discovery Key Response. The discovery security materials related to U2U relay discovery are associated with the RSC. If there are Relay Discovery Security Policies for the RSC-specific U2U relay service, the 5G DDNMF/5G U2U Relay PKMF includes these policies in the response.

### 6.17.3	Evaluation

Editor’s Note: Whether this authorization method is applicable to the sol#1/alt#1 scenario in TR 23.700-33 is FFS.

Solution #17 addresses the third security requirement of key issue #1 and Key issue #3.

This solution allows the UE to provide multiple roles that it can play in U2U relay communication in the security material request; so that all discovered security materials can be obtained at one time.

This solution assumes that there may be multiple PKMFs/5G DDNMFs involved in a security material provision process for direct discovery, U2U relay discovery or both. This solution supports interaction between multiple PKMFs/5G DDNMFs. The address of the PKMF is provided to UE by its HPLMN.

The U2U relay authorization check is done by the PKMF/5G DDNMF during discovery security materials provision process. The PKMF/5G DDNMF checks whether a UE can play its claimed roles, and then provisions corresponding discovery security materials to the UE. This solution assumes that the discovery security materials provisioned for direct discovery and U2U relay discovery are different.

*** END OF CHANGES ***