3GPP TSG-SA3 Meeting #109AdHoc-e 	draft_S3-230388-r1

- Electronic meeting, 16 - 20 January 2023

- Source:	Xiaomi

- Title:	New Sol on authorization mechanism for partly rejected S-NSSAI

- Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

It is requested to approve the pCR for TR 33.886

# 2	References

[1]	3GPP TR 33.886 “Study on enhanced security for phase 3 network slicing”

# 3	Rationale

This contribution proposes to approve the pCR for TR 33. 886 [1].

# 4	Detailed proposal

*************** Start of the Change ****************

## 5.Y	Solution #Y: Authorization mechanism for partly rejected S-NSSAI

### 5.Y.1	Introduction

This solution addresses key issue #1.

In TR23.700-41 [3], it is concluded that partly rejected S-NSSAI is supported.

For partly rejected S-NSSAI, AMF can provide information to the UE enabling the UE to be able to register an S-NSSAI that is rejected S-NSSAI for the RA, when the UE moves to a TA supporting the S-NSSAI. The UE shall be able to request a rejected S-NSSAI, by initiating a registration update procedure, in a supported TA based on the supported/not supported TA information associated with this S-NSSAI.

In this contribution, AMF will authorize the partly rejected S-NSSAI based on the location of UE.

### 5.Y.2	Solution details

Figure 5.Y.2-X Authorization mechanism for partly rejected S-NSSAI

Step 1. To request a partly rejected network slice, UE sends target S-NSSAI to AMF.

Step 2. AMF needs to obtain the location of UE if the requested S-NSSAI is a partly rejected S-NSSAI. AMF should reject the UE when UE is not in a TA supporting that partly rejected S-NSSAI.

Step 3. If the S-NSSAI needs to be authenticated and authorized by AAA server, NSSAA proceudre should be triggered. AMF should terminate the procedure when the UE is not authorized by the AAA server.

Step 4. AMF sends registration update response to the UE.

Editor's Note: Further details interaction with NSSAA are FFS.

### 5.Y.3	Evaluation

AMF needs to obtain the location of UE if the requested S-NSSAI is a partly rejected S-NSSAI. AMF should reject the UE when UE is not in a TA supporting that partly rejected S-NSSAI.

Editor's Note: Further evaluation is FFS.

*************** End of the Change ****************