3GPP TSG-SA3 Meeting #109AdHoc-e 	S3-230019

# Electronic meeting, 16 - 20 January 2023

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Key Issue for NAS UE Registration Procedure

# Document for:	Approval

Agenda Item:	5.25

# 1	Decision/action requested

Approve the KI for Protection of NAS UE Registration Procedure

# 2	References

[1]	3GPP TR 33.700-28 V0.2.0 (2022-11): Study on security aspects of satellite access (Rel 18)

[2]		3GPP TR 33.896 V0.3.0 (2022-10): Study of Security Aspects on User Consent for 3GPP Services; Phase 2 (Rel 18)

[3]	3GPP TR 23.700-28 V1.1.1 (2022-10):  Study on Integration of satellite components in the 5G architecture; Phase 2 (Rel 18)

[4]	…

# 3	Rationale

The support of NTN Satellite discontinuous transmission requires signalling and message exchange between different network functions.

The following architecture figure is focusing on the relevant network functions.

Figure 1: NTN Discontinuous Transmission Architecture (overview)

The AMF/MME is obtaining the UE unreachability period information from the AF (via NEF/SCEF), from the UE or from the OAM (refer to [3] clause 8.1). The application function (AF) is assumed to be outside of the Trusted Domain, therefore AF network function is assumed to be untrusted, while the UE and the OAM could be trusted.

The AMF/MME configures the UEs mobility management and power saving mode (PSM) parameters and send the MICO/PSM parameters to the UE (refer to [3] clause 8.1).

The UE determines that it has to remain with no service or it has to attempt to register on available different RATs/PLMNs to receive the normal service (refer to [3] clause 7.3, R3).

Abbreviations:

AF				Application Function

AMF			Access and Mobility Management Function

MME			Mobility Management Entity

NEF			Network Exposure Function

NTN			Non-Terrestrial Network

OAM			Operation and Maintenance

PLMN(s)		Public Land Mobile Network(s)

PSM			Power Saving Mode

RAT(s)		Radio Access Technology(s)

SCEF			Service Capability Exposure Function

UE				User Equipment

# 4	Detailed proposals

******************** Start of New Text **********************************************

## 5.x	Key Issue #x: Protection of NAS UE Registration Procedure

### 5.x.1	Key issue details

The NTN in discontinuous transmission requires the exchange of signalling information. The AMF/MME will be receiving UE unavailability period information, furthermore, the AMF/MME will collect power saving mode parameters and will need to send those to the UE. The UE itself needs to register properly to a RAT/PLMN to ensure service continuity during missing satellite coverage.

A missing protection of the signalling information might lead to denial of service, due to overflooding of target network function with invalid UE signalling. This might lead to misconfiguration, because of tampered/spoofed information. This might lead to unauthorized provision of satellite access or avoidance of satellite access due to location impersonation of UE sensitive information.

The mitigation for R3 [3, clause 7.3] is implicit reducing the impact due to invalid UE signalling as described by R4 [3, clause 7.3].

### 5.x.2	Security Threats

Tampering of UE unavailability period information, PSM parameters and/or injecting of in-/valid information/parameters.

Location impersonation attack by eavesdropping of UE sensitive information during NAS UE registration request procedure.

### 5.x.3	Potential Requirements

TBD

************************** End of New Text **************************************