3GPP TSG-SA3 Meeting #109AdHoc-e 	S3-230020

# Electronic meeting, 16 - 20 January 2023

# Source:	Nokia, Nokia Shanghai Bell

# Title:	New Key Issue on Protection of 5GS SIB19

# Document for:	Approval

Agenda Item:	5.25

# 1	Decision/action requested

SA3 is kindly asked to approve the proposed new key issue in TR 33.700-28.

# 2	References

[1]        TR 33.700-28     Study on security aspects of satellite access

[2]        TR 23.700-28	Study on Integration of satellite components in the 5G architecture; Phase 2

# 3	Rationale

This pCR intends to introduce a new key issue into TR 33.700-28 [1] on protection of SIB19 (5GS) provided to the UE.

# 4	Detailed proposal

************ Start of the Change ************

## 5.X	Key issue #X: Protection of NTN assistance information

### 5.X.1	Key issue details

Satellite assistance information could be broadcasted in System information (i.e., SIB19 for 5GS and SIB31 for EPS) or only NTN configuration could be sent in RRC setup message (5GS). Satellite assistance information contains several sensitive information like reference location of serving cell, distance threshold from the serving cell reference location and it is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE states.

Coverage of satellite is in wider range or regions and mis-using any assitance information could impact the mobility of UEs in all RRC states.

### 5.X.2	Threats

If the UE receives the tampered or falsified NTN assistance information, could result in wrong time based and location-based measurements for Quasi earth fixed cell scenarios. This leads to service interruption or to stay camped on malicious terrestrial networks (Denial on non-terrestrial networks access) or obtaining UE location based on falsified location reference provided.

### 5.X.3	Potential security requirements

The 5GS/EPS shall provide a means for the UE to verify the integrity of the received UE unreachability period information.

The 5GS/EPS shall provide a means to mitigate the replay attack on UEs receiving NTN assistance data.

************ End of the Change ************