3GPP TSG-SA3 Meeting #109AdHoc-e 	S3-230168

# Electronic meeting, 16 - 20 January 2023										    Revision of S3-23xxxx

# Source:	Huawei, HiSilicon

# Title:	User consent for roaming scenarios where the data source is in vPLMN

# Document for:	Approval

Agenda Item:	5.22

1	Decision/action requested

The contribution proposes a new solution to address KI#1

2	References

3	Rationale

The solution addresses the requirement in KI#1 “User consent for roaming case in eNA”, specifically for the case that data source (e.g. input data or analytics) is in vPLMN.

The solution follows SA2’s conclusion.

4	Detailed proposal

*************** Start of 1st Change ****************

6.X	Solution #X: User consent for roaming scenarios where the data source is in vPLMN

6.X.1	Introduction

This solution addresses the requirement in KI#1 "User consent for roaming case in eNA", specifically for the case where the data source (e.g. input data or analytics) is located in the vPLMN.

As agreed in conclusion 8.3 in TR 23.700-81 [2], "an NWDAF is used as entry point to exchange analytics in roaming scenario between HPLMN and VPLMN. It authorizes the request according to operator policies and user consent and it filters the information exposed in reply to the request".

The solution analyses the two cases depending on whether the vPLMN and hPLMN belong to the same jurisdiction or not while considering the conclusion above.

6.X.2	Solution details

6.X.2.1	vPLMN and hPLMN in same jurisdiction

Figure 6.X.2.1 – 1 Example procedure for provisioning and user consent check

First, in this scenario, it is assumed that user consent for analytics is required by the local regulation.

Now since the data source is located in the vPLMN, the vNWDAF is considered to be the enforcement point according to Annex V of TS 33.501 [3]. If the vNWDAF is required by hNWADF in vPLMN to collect UE’s input data or analytics, the vNWDAF retrieves the necessary user consent parameters belonging to the vPLMN from the UDM. The vNWDAF is responsible for enforcing the vPLMN specific user consent accordingly based on the request for input data or analytics.

NOTE Y:	The vPLMN and hPLMN negotiates the specific value of purpose as well as language and human readable purpose when signing roaming contract, so that the vNWDAF and subscriber can recognize the purpose.

NOTE W: The hPLMN can acquire user consent from its subscriber when the subscriber is roaming via means out of 3GPP, e.g. SMS or Application using the negotiated contract text. The hPLMN converts the contract to vPLMN specific user consent parameters, and stores them in the UDM.

When the subscriber left the vPLMN or the consent is revoked, the UDM notifies the vNWDAF to stop data processing. The vNWDAF notifies other network entities to stop data processing.

6.X.2.2	vPLMN and hPLMN in different jurisdiction

In this scenario, since different jurisdictions may have different requirements for transfers of personal data to third countries or international organisations, it is left to operators to address the different applicable legal requirements if any for example based on SLAs or other out of band mechanisms should the user consent be required in vPLMN.

6.X.3	Evaluation

The solution addresses the requirement in KI#1 “User consent for roaming case in eNA”, specifically for the case where the data source (e.g. input data or analytics) is located in the vPLMN.

The solution is aligned with conclusion in TR 23.700-81 [2].

*************** End of 1st Change ****************