3GPP TSG-SA3 Meeting #109AdHoc-e 	S3-230244

# Electronic meeting, 16 - 20 January 2023

# Source:	Ericsson

# Title:	Adding conclusion on KI#3

# Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

This contribution proposes conclusion on KI#3 in TR 33.738.

# 2	References

[1]   3GPP TR 33.738 V0.4.0 "Study on security aspects of enablers for Network Automation for 5G - phase 3"

[2]   3GPP TR 23.700-81 V2.0.0 " Study of Enablers for Network Automation for 5G System (5GS); Phase 3"

# 3	Rationale

This pCR proposes to use the ideas of solution #7,#10,#11.#12,#13 for protection of ML models and authorization of model retrieval to conclude KI#3.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.738.

***BEGIN OF First CHANGE***

# 7	Conclusions

## 7.X	Conclusion on Key Issue #3 "Security for AI/ML model storage and sharing"

For key issue#3, it is recommended to use the ideas of solution #7,#10,#11.#12,#13 for protection of ML models and authorization of model retrieval:

-	Authorization of the model retrieval at the NRF uses token-based authorization. The NRF uses information stored by the MTLF at the NRF as per conclusions in TR 23.700-81.

-	Additionally, the MTLF performs authorization of the model retrieval per model.

-	During normative work, specify which algorithms to use for end-to-end protection of ML models. Key distribution is handled out of band and not specified.

*** END OF First CHANGE ***