3GPP TSG-SA3 Meeting #109AdHoc-e 	S3-230315

# Electronic meeting, 16 - 20 January 2023

# Source:	Ericsson

# Title:	Discussion. Key issue #12: Security in Hosted SEPP scenarios

# Document for:	Endorsement

Agenda Item:	5.24

# 1	Decision/action requested

It is proposed to endorse the proposal of this discussion paper.

# 2	References

[1]	3GPP TR 33.875: "Study on enhanced security aspects of the 5G Service Based Architecture (SBA)"

[2]	GSMA NG.113-v5.0-2: "5GS Roaming Guidelines"

[3]	3GPP S3-213806: "LS to 3GPP SA3 working group on 5GS Roaming Hubbing"

[4]	3GPP S3-221737: "LS to 3GPP SA3 on Hosted SEPP"

# 3	Rationale

This discussion paper providers additional details of the problems to solve in "Key issue #12: Security in Hosted SEPP scenarios."

It serves as background to an update of the KI#12 details (distributed separately) as well as a Solution Proposal for KI#12 (distributed separately).

# 4	Detailed proposal

It is proposed to discuss and endorse the following background and observations. The serves as background information for updates to KI#12 [1] and solution.

## 4.1 Introduction

Bilateral PLMN inter-connection scenarios are described by NG 113 v5.0 annex B [2]. SEPPs can be deployed

- Within a PLMN

- Outsourced outside PLMN

## 4.2 SEPP Deployed within the PLMN

### 4.2.1 General

SEPPs can be deployed within a PLMN ([2] annex B.4.1)

HPLMN

VPLMN

SEPP

SEPP

NF

NF

Figure 4.2.1-1 SEPP Deployed within the PLMN

Both PLMNs may have a single PLMN ID or owning multiple PLMN IDs. In case of multiple PLMN ID’s the same N32-f may be used as specified in [1] clause 5.9.3.2 "Requirements for Security Edge Protection Proxy (SEPP)"

### 4.2.2 Functions for SEPP Deployed within the PLMN

Among other the SEPP functions are

(1) Terminating the N32c/f interface and authenticate roaming partner and IPX (TLS, PRINS). [1] clause 5.9.3.2 
"The SEPP shall protect application layer control plane messages between two NFs belonging to different PLMNs or SNPNs that use the N32 interface to communicate with each other." and 
"The SEPP shall perform mutual authentication and negotiation of cipher suites with the SEPP in the roaming network."

(2) Roaming Security Functions (Topology Hiding and Firewalls) [1] clause 5.9.3.2 
"The SEPP shall perform topology hiding by limiting the internal topology information visible to external parties."

(3) Shielding PLMN external CAs towards the PMN internal NFs. [1] clause 5.9.3.2 
"The SEPP shall protect application layer control plane messages between two NFs belonging to different PLMNs or SNPNs that use the N32 interface to communicate with each other."

(4) Bridging between PMN external and internal IP addressing schemas (private, public) [1] clause 5.9.3.2 
"As a reverse proxy the SEPP shall provide a single point of access and control to internal NFs."

(5) Enforcement of market specific regulatory requirements to have a SEPP PLMN internally within HPLMN. [3] clause "1) Roaming Hub" 
"Furthermore, regulation in some countries may require that the PLMN SEPP be deployed in the PLMN and located in the country where the PLMN operates."

HPLMN

SEPP

NF

(1) (2)

(3) (4) (5)

Figure 4.2.1-2 Functions for SEPP Deployed within the PLMN

## 4.3 SEPP deployed outsourced on behalf of the PLMN

### 4.3.1 General

Hosted

HPLMN

VPLMN

SEPP

SEPP

NF

NF

SEPPs can be deployed outsourced on behalf of an PLMN to an IPX provider. It may also be deployed outsorced to an organization of an operator group handling roaming interconnect for the group ([2] annex B.4.2/B.4.3) (aka hosted SEPP)

Figure 4.3.1-1 SEPP deployed outsourced on behalf of the PLMN

In the incominmg LS from GSMA [3] on "5GS Roaming Hubbing" it is stated 
"The key issue identified by NG 5GMRR for this function is that when roaming hubs are used, the commercial agreements are not directly between the network operators, but between the roaming hub and the home and visited network operator. In this case it is inappropriate for a home network to provide the security parameters for the N32 interface as they do not have the commercial agreement or the liability.  It is also inappropriate for the roaming hub to give security details to the home network operator to use between the home network and the serving network as then the owner of the commercial contract (the roaming hub) would lose all control of that N32 interface and be unable to audit its use. This issue exists regardless of whether TLS or PRINS is used for the N32 interface.

Deploying a PLMN SEPP in the roaming hub is not possible with current R16 3GPP solution since the roaming hub would have full visibility of any direct roaming agreements (as illustrated above between PLMN 1 and PLMN 2) made which is highly undesirable.

Furthermore, regulation in some countries may require that the PLMN SEPP be deployed in the PLMN and located in the country where the PLMN operates."

Additionally in the incoming LS from GSMA [4] on "LS to 3GPP SA3 on Hosted SEPP" we find the following requirements

"5GMRR has agreed on the following requirements on Hosted SEPP:

-	NFs like AMF, NRF, SMF, UDM shall not be impacted by the decision to deploy a Hosted SEPP, i.e.,

o	A NF in the VPLMN shall discover and interact with NF(s) in the HPLMN as standardized by 3GPP.

o	A NF in the HPLMN shall interact with NF(s) in the VPLMN as standardized by 3GPP.

-	All SBI messages shall be transmitted securely between the PLMN and the Hosted SEPP deployment.

-	Both the operator edge and the edge of the hosted SEPP deployment need to be protected.

-	A roaming partner SEPP shall interact with the Hosted SEPP as standardized by 3GPP. Section 4.19 of GSMA PRD IR.67 describes the use of DNS in Peer SEPP FQDN(s) discovery including the FQDN discovery for a non-MNO SEPP. Section 5.1.2 of GSMA PRD FS.34 describes the FQDN of the non-MNO SEPP.

-	The Hosted SEPP providers shall isolate the processing for each PLMN from the processing for other PLMNs. The hosted SEPP provider shall be able to use a different TLS certificate per serving PLMN for an incoming TLS connection, as the MNC and MCC of that PLMN are encoded in the certificate. This certificate shall be dedicated to the given PLMN (i.e., shall not contain MNC/MCC combinations belonging to any other PLMN). It is assumed that the hosted SEPP is only able to obtain a certificate with an MCC/MNC combination if the affected MNO has previously authorized this."

Observations and conclusions:

- The different SEPP deployment variants have different requirements.

- The outsourced SEPP may be in another "security zone" than the other HPLMN NFs.

- In 3GPP [2] there is only one set of requirements for SEPP, i.e. for the SEPP deployed within PLMN and not outsourced.

- There seems to be a need to distinguish between SEPPs deployed in different ways, also in the 3GPP specifications.

- In case we don't make necessary distinctions between SEPP deployment variants we may later run into interoperability issues when different vendors will implement different subsets of the SEPP requirements.

Please note that we are not discussing to introduce a new NF type. The only thing we discuss is to distinguish between the one and only NF SEPP being used in different deployment variants.

### 4.3.2 Functions for SEPP Outsourced on behalf of the PLMN

In outsourced hosted SEPP deployment scenario, the hosted SEPP will still be responsible for

(1) Terminating the N32c/f interface (TLS, PRINS)

(2) Roaming Security Functions (Topology Hiding and Firewalls)

For the other functions the situation is not obvious and depends on

(3) The organization of CA between the HPLMN and IPX provider

(4) The IPX is able to handle private addresses of the Home PMN

(5) Whether the IPX is trusted to enforce market specific regulatory requirements

Hosted

HPLMN

SEPP

NF

N32

(1) (2)

(3) (4) (5)

(2)

Figure 4.3.2-1 Functions for SEPP Outsourced on behalf of the PLMN

## 4.3 SEPP deployed within PLMN on behalf of the PLMN

Hosted

HPLMN

SEPP

SCP

N32

Figure 4.3-1 Functions for SEPP Outsourced on behalf of the PLMN

Due to regulatory requirements the SEPP may be deployed at the edge of the PLMN and communicating with the HPLMN SCP. This SEPP has another set of requirements than the other SEPP deployments.

[3] clause "1) Roaming Hub" states:
"Furthermore, regulation in some countries may require that the PLMN SEPP be deployed in the PLMN and located in the country where the PLMN operates."

## 4.4 Summary of SEPP deployment variants

Figure 4.4-1 Summary of SEPP deployment variants

This leaves us with three types of SEPP deployment variants. Each will have different types of requirements.