3GPP TSG-SA3 Meeting #110Ad-Hoc-e 	draft_S3-231845-r1

- Electronic meeting, Online, 17 - 21 April 2023									(revision of S3-yyxxxx)

- Source:	ZTE

- Title:	Add conclusion for KI#3

- Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This contribution proposes to add conclusion for the key issue#3.

# 2	References

[1] 	TR 33.740  v0.6.0

# 3	Rationale

It is proposed to add some conclusion for the key issue #3.

# 4	Detailed proposal

********************Start of 1st Change******************

## 7.3	Key issue #3: Authorization in the UE-to-UE Relay Scenario

For Key Issue #3, the following statement is agreed:

The PCF based service authorisation as defined in TS 23.304 [3] is reused as the basis for normative work of service authorisation for UE-to-UE Relay (Both Layer 3 and Layer 2 UE-to-UE Relay) operation.

Authorization in the UE-to-UE Relay  (Both Layer 3 and Layer 2 UE-to-UE Relay)  Scenario is performed during discovery security material provision procedure and U2U relay discovery procedure.

During PC5 link establishment in Layer 3 and Layer 2 UE-to-UE Relay Communication, when the Layer 3 and Layer 2 UE-to-UE Relay is in 3GPP coverage, the existing procedures in clause 6.3 in TS 33.503[6] can be re-used for authorization.

Editor’s Note: Futher conclusion is FFS.

********************End of 1st Change******************