3GPP TSG-SA3 Meeting #110Ad-Hoc-e 	draft_S3-231930-r2

- Electronic meeting, Online, 17 - 21 April 2023									(revision of S3-yyxxxx)

- Source:	Huawei, HiSilicon

- Title:	Updating NOTE1

- Document for:	Approval

Agenda Item:	4.14

# 1	Decision/action requested

Approve the changes in clause 4 for inclusion in the living CR of HONTRA.

# 2	References

[1]	S3-231477	SA3#110

[2]	TS 23.502		3GPP TS 23.502: "Procedures for the 5G System".

# 3	Rationale

The changes in this contribution are based on the living CR S3-231477.

The NOTE1 can be used as general description which is more useful. Thus, turn the NOTE1 to the normative text and put it in general clause. Moreover, AAnF is added as another case besides the intworking and SoR/UPU counter wrap around.

# 4	Detailed proposal

***************** Start of Change *****************

## 6.1X	Home network triggered primary authentication procedure

### 6.1X.1	General

In case the HN and the SN support the services defined in clause 14.2.x, then the UDM is able to trigger a primary authentication procedure as described in the following clause.

Editor's Note: The impact of HONTRA on the security related service operations in clause 14 is FFS. The above statement wil be updated when the service operations are agreed.

### 6.1X.2	Security mechanisms

The UDM may initiate primary authentication based on events from other NFs, considering local policy into account as well.

Figure 6.1.x.2-1 Home Network triggered primary authentication procedure

0a.[optional] The UDM may be pre-configured with an operator policy in order to determine when to trigger a primary authentication procedure.

A prerequisite for the initiating of Home Network Triggered Primary Authentication is that the UDM already has the information about the AMF serving the UE. Otherwise, the UDM cannot contact any AMF in later steps.

Editor’s Note: Whether a new UDM service is needed is FFS, whether it is needed that a consumer NF (e.g. AUSF) triggers the UDM is FFS.

1. Based on a received event and the local operator policy, if there is no ongoing primary authentication for the UE, and if the UDM determines to trigger the primary authentication, the UDM determines the serving AMF/SEAF of the target UE.

If the UE registed from different PLMNs by both 3GPP access and non-3GPP access, the UDM shall select the AMF used by 3GPP access.

NOTE 1:	The reasons for the UDM determining that the UE needs to be authenticated can be different. For example, the UDM can determine to initiate a primary authentication when the AMF registers the UE upon the Registration procedure during the mobility from EPC or when SoR/UPU counters are about to wrap around, or when required based on authentication policy. The UDM behaviour is determined by the operator primary authentication policy which takes into account the support of certain features in the PLMN and if a time information is set by the operator to consider for primary (re-)authentication. For example, if the PLMN does not support the SoR/UPU feature, then SoR/UPU counter wrap around will not happen and soprimary authentication will not be required for this case. For the other case AAnF can consider the authentication policy available it’s side for the AKMA and related AF key lifetime setting. If an AF key expiry happens and still if there is no successful primary authentication that resuls in new AKMA key, then the AAnF can consider to request the UDM for a primary (re-)authentication.

2. The UDM sends an authentication message to the AMF/SEAF with the UE’s SUPI.

Editor’s Note: The name and the type (request/reply, subscribe notify etc) of the interaction between the AMF and UDM is FFS.

3. After receiving the authentication message from the UDM, the AMF/SEAF shall decide whether run the primary authentication procedure based on its own local authentication policy. If the AMF/SEAF cannot run a primary authentication as described in steps 4, the AMF/SEAF sends the authentication response message to the UDM with an acknowledgement which includes failure cause else it is set as success.

Editor’s Note: The serving network has its own authentication policy. Moreover it is possible that the UE cannot be reached. Therefore it is FFS whether it can start a primary authentication right away after receiving the authentication message from the UDM.

4. The AMF/SEAF starts the primary authentication procedure as defined in clause 6.2.1 of the present document.

The UDM may execute other procedures (e.g. SoR/UPU) depending on the reason that motivated the UDM triggered reauthentication procedure in step 1.

Editor’s Note: The step details such as for aspects covering operator/local configuration/policy usage, UDM logic, or trigger to UDM etc. are FFS.

***************** End of Change *****************