3GPP TSG-SA3 Meeting #110Ad-Hoc-e 	S3-231754

# Electronic meeting, Online, 17 - 21 April 2023								(revision of S3-yyxxxx)

# Source:	Ericsson

# Title:	Updated conclusion for KI#1 regarding trusted access

# Document for:	Approval

Agenda Item:	5.16

# 1	Decision/action requested

It is proposed to add the proposed conclusions into the TR 33.858 [1].

# 2	References

[1]	3GPP TR 33.858 Study on security aspects of enhanced support of Non-Public Networks phase 2

[2]	S3-231753

# 3	Rationale

As discussed in [2] the preferred way of identifying the key KTNGF in case anonymous SUCI is used for trusted access, is to use the hash of the key KTNGF.

# 4	Detailed proposal

***BEGIN CHANGES***

### 7.1.3 Conclusion for Trusted N3GPP access to SNPN

Solution #2 is selected as basis for normative work with regards to the aspects:

- 	Support for all key generating EAP-methods

- 	Support for onboarding

Solution #3 is selected as basis for normative work with regards to

- 	Support for usage of anonymous SUCI

This implies that the procedure specified in TS 33.501 [2] section 7A.2.4 will be reused for normative work with the following modifications:

- 	Support for usage of anonymous SUCI:

- 	Add possibility to send anonymous SUCI in step 5 (affecting also following steps 5-8) if the construction of SUCI as described in clause 6.12 of TS 33.501 [2] cannot be used and if the employed EAP method supports privacy.

-	In step 13, if the construction of SUCI as described in clause 6.12 of TS 33.501 cannot be used to identify the UE, then the hash of the key KTNGF is used (potentially using some additional input to be decided in the normative work). The identifier is put in the IDi payload.

- 	Support for all key generating EAP-methods: Extension of applicable authentication mechanism in step 8 to key-generating EAP authentication methods.

- 	Support for onboarding: Add possibility to send onboarding SUCI in step 5

***END CHANGES***