3GPP TSG-SA3 Meeting #110Ad-Hoc-e 	S3-231756

# Electronic meeting, Online, 17 - 21 April 2023								(revision of S3-yyxxxx)

# Source:	Ericsson

# Title:	Updated conclusion for KI#1 regarding N5CW access

# Document for:	Approval

Agenda Item:	5.16

# 1	Decision/action requested

It is proposed to add the proposed conclusions into the TR 33.858 [1].

# 2	References

[1]	3GPP TR 33.858 Study on security aspects of enhanced support of Non-Public Networks phase 2

# 3	Rationale

There are two remaining Editor’s notes regarding N5CW device access to SNPN:

Editor’s Note: Conclusions regarding the issue of key derivation for non-NAS capable devices shall be aligned with outcome of study in TR 33.887 and are FFS.

Editor’s Note: Further conclusions are FFS.

The first EN is related to the identifier problems in the existing procedures for N5CW devices, specified in clause 7A.2.4 of TS 33.501. Since the devices are non-NAS devices, they don’t hold the NAS count, which is needed to derive the key KTWIF.

Since the proposed solution is reusing the existing procedures, the solution can only be concluded if the existing procedures are also corrected. This is however out of scope of the study. It is proposed to remove the Editor’s note and correct the existing procedures in clause 7.A.2.4 of TS 33.501 as part of normative work.

The second Editor's note related to further conclusions can be removed since all aspects of the untrusted access are concluded:

- Support for all key generating EAP-methods (Solution #1 selected)

- Support for onboarding (Solution #1 selected)

- Support for usage of anonymous SUCI (Solution #1 selected)

# 4	Detailed proposal

***BEGIN CHANGES***

### 7.1.4 Conclusion for N5CW device access to SNPN

Solution #4 is selected as basis for normative work with regards to the aspects:

- 	Support for all key generating EAP-methods

- 	Support for usage of anonymous SUCI if the construction of SUCI as described in clause 6.12 of TS 33.501 [2] cannot be used and if the employed EAP method supports privacy.

- 	Support for SNPN Id (PLMN Id and NID) carried in NAI

Since Solution #4 assumes reuse of existing procedures and since there is an issue of key derivation for non-NAS capable devices (NAS count is needed to derive the key KTWIF, however N5CW devices do not hold a NAS count), the existing procedure in clause 7A.2.4. needs to be corrected.

***END CHANGES***