3GPP TSG-SA3 Meeting #110Ad-Hoc-e	S3-231803

# Electronic, Online, 17 - 21 April 2023

# Source:	Qualcomm Incorporated

# Title:	Update the evaluation of solution #9

# Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This contribution proposes to update the evaluation of solution #9 in TR 33.740.

# 2	References

[1]	TR 33.740

# 3	Rationale

This contribution proposes to update a part of evaluation in solution #9. Particularly, this contribution proposes to update the evaluation to reflect that the solution #9 addresses the security requirements of KI#1 when a U2U relay service identified with an RSC is only used for a single ProSe service. Other aspects of this solution are not evaluated in this contribution.

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TR [1].

***** START OF CHANGES *****

### 6.9.3	Evaluation

This solution addresses the Key Issue #1 when a U2U relay service identified with an RSC is only used for a single ProSe service.

This solution is aligned with the SA2’s procedure defined in TS 23.304 [8].

By using the security materials, the 5G Prose End UEs can securely discover each other via UE-to-UE Relay and the Solicitation/Response messages can be protected.

This solution fulfills the security requirements of KI#3 based on the DDNMF/PKMF’s authorization by reusing the Discovery procedure defined in clause 6.1.3.2 of TS 33.503 [6]. Once the authorization is passed, the 5G Prose End UEs and UE-to-UE Relay can perform UE-to-UE relay discovery.

This solution uses one set of discovery security material to protect the Solicitation/Response message by reusing the Discovery procedure defined in clause 6.1.3.2 of TS 33.503 [6].

Editor’s Note: Further evaluation is FFS.

***** END OF CHANGES *****