3GPP TSG-SA3 Meeting #110Ad-Hoc-e	S3-231805

# Electronic meeting, Online, 17 - 21 April 2023

# Source:	Qualcomm Incorporated

# Title:	Conclusion of KI#5

# Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This contribution proposes a conclusion of KI #5.

# 2	References

[1]	TS 33.503

[2]	TR 33.740

# 3	Rationale

This contribution proposes to add a conclusion of Key Issue #5.

Particularly, it is proposed to reuse the existing procedures for security policy provisioning and the activation of security between the UEs based on the provisioned security policies as specified in TS 33.503 [1].

In case of end-to-end security for layer-3 U2U relay, it is proposed that the end-to-end security can be activated based on the security policy configuration by PKMF. If PKMF configures security policy to activate the end-to-end security , the corresponding security materials are provisioned by PKMF.

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TR [2].

***** START OF CHANGES *****

## 7.5	Key Issue #5: Security of source and target UE communication via U2U relay

The activation of security between the UEs is based on the provisioned security policies for the specific 5G ProSe UE-to-UE Relay service as specified in TS 33.503[6].

For Layer-3 U2U relay, end-to-end security can be activated based on the security policy configuration by PKMF.

For Layer-3 U2U relay, security materials for end-to-end security are provisioned by PKMF.

***** END OF CHANGES *****