3GPP TSG-SA3 Meeting #110 Adhoc-e	S3-231821

# Online, 17 - 21 April 2023

# Source:	Philips International B.V.

# Title:	Conclusion on KI#3

# Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

SA3 is kindly asked to approve conclusions related to KI#3 in TR 33.740

# 2	References

N/A

# 3	Rationale

The contribution proposes additional conclusions related to KI#3 in TR 33.740.

# 4	Detailed proposal

*** Start of changes ***

## 7.3	Key issue #3: Authorization in the UE-to-UE Relay Scenario

For Key Issue #3, the following statement is agreed:

The PCF based service authorisation as defined in TS 23.304 [3] is reused as the basis for normative work of service authorisation for UE-to-UE Relay operation.

Authorization in the UE-to-UE Relay Scenario is performed during discovery security material provision procedure and U2U relay discovery procedure.

During PC5 link establishment in Layer 3 and Layer 2 UE-to-UE Relay Communication, when the Layer 3 and Layer 2 UE-to-UE Relay is in 3GPP coverage, the existing procedures in clause 6.3 in TS 33.503 can be re-used for authorization.

The authorization mechanism of UEs (Source, Target, and UE-to-UE relay) can be determined based on a policy depending on the 3GPP coverage status of the UE-to-UE relay.

The policy may allow using temporary credentials, e.g., passwords, when the UE-to-UE relay is out of 3GPP coverage and credentials are not available (e.g., have expired or have not been configured), to authenticate and authorize the UE-to-UE Relay communication.

Editor’s Note: Further conclusion is FFS.

*** End of changes ***