3GPP TSG-SA3 Meeting #110Ad-Hoc-e	S3-231987

# Electronic meeting, Online, 17 - 21 April 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | draft | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [draftCR] Adding a new security sevice operation provided by the UDM and AMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | HN_Auth |  |  |  |  |  | Date: |  |  | 2023-03-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The home network initiated primary authentication is a new procedure agreed in SA3#110 meeting. There is a need for a new UDM service operation and AMF service operation for triggering the primary authentication. Therefore, it is proposed to delete the respective Editor’s Note and add a new service operation from the NF to UDM and from the UDM to the AMF to initiate a primary authentication procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Nudm_HNAuthentication_Authenticate service operation is added for the NF to request the UDM to trigger the primary authentication. Namf_UEAuthentication_Authenticate service operation is added for the interaction between the AMF and the UDM to trigger the primary authentication procedure in HNTRA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Leads to an incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1X.2, 14.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ******Start of Change******

## 6.1X	Home network triggered primary authentication procedure

### 6.1X.1	General

In case the HN and the SN support the services defined in clause 14.2.x, then the UDM is able to trigger a primary authentication procedure as described in the following clause.

Editor's Note: The impact of HONTRA on the security related service operations in clause 14 is FFS. The above statement wil be updated when the service operations are agreed.

### 6.1X.2	Security mechanisms

The UDM may initiate primary authentication based on events from other NFs, considering local policy into account as well.

Figure 6.1.x.2-1 Home Network triggered primary authentication procedure

0a.[optional] The UDM may be pre-configured with an operator policy in order to determine when to trigger a primary authentication procedure.

A prerequisite for the initiating of Home Network Triggered Primary Authentication is that the UDM already has the information about the AMF serving the UE. Otherwise, the UDM cannot contact any AMF in later steps.

1. Based on a received event and the local operator policy, if there is no ongoing primary authentication for the UE, and if the UDM determines to trigger the primary authentication, the UDM determines the serving AMF/SEAF of the target UE.

Editor’s Note: The handling of multiple AMFs registered to the UDM is FFS.

NOTE 1:	The reasons for the UDM determining that the UE needs to be authenticated can be different. For example, the UDM can determine to initiate a primary authentication when the AMF registers the UE upon the Registration procedure during the mobility from EPC or when SoR/UPU counters are about to wrap around, or when required based on local policy. The UDM behaviour is determined by the primary authentication policy which takes into account the support of certain features in the PLMN. For example, if the PLMN does not support the SoR/UPU feature, then a policy to initiate HONTRA upon SoR/UPU counter wrap around will not be required.

2. The UDM sends a Namf_UEAuthentication Authenticate request  to the AMF/SEAF with the UE’s SUPI as detailed in clause 14.X.

3. After receiving the authentication message from the UDM, the AMF/SEAF shall decide whether run the primary authentication procedure based on its own local authentication policy. If the AMF/SEAF cannot run a primary authentication as described in steps 4, the AMF/SEAF sends the authentication response message to the UDM with an acknowledgement which includes failure cause else it is set as success.

Editor’s Note: The serving network has its own authentication policy. Moreover it is possible that the UE cannot be reached. Therefore it is FFS whether it can start a primary authentication right away after receiving the authentication message from the UDM.

4. The AMF/SEAF starts the primary authentication procedure as defined in clause 6.2.1 of the present document.

The UDM may execute other procedures (e.g. SoR/UPU) depending on the reason that motivated the UDM triggered reauthentication procedure in step 1.

Editor’s Note: The step details such as for aspects covering operator/local configuration/policy usage, UDM logic, or trigger to UDM etc. are FFS.

******Next Change******

## 14.2	Services provided by UDM

14.2.X	Nudm_HNAuthentication service operation

Service operation name: Nudm_HNAuthentication_Authenticate

Description: Requester NF requests the UDM to trigger the primary authentication.

Inputs, Required SUPI.

Inputs, Optional: None.

Outputs, Required:  Operation execution result indication.

Output, Optional: None.

******Next Change******

## 14.X	Services provided by AMF

#### 14.X.1.2	Namf_UEAuthentication_Authenticate

#### 14.X.1.2.1	Namf_UEAuthentication_Authenticate service operation

Service operation name: Namf_UEAuthentication_authenticate.

Description: The UDM requests the AMF to start the primary authentication procedure.

Input, Required: SUPI.

Inputs, Optional: None.

Output, Required: Operation execution result indication.

Output, Optional: None.

******End of Change******