3GPP TSG-SA3 Meeting #110 	S3-23657

# Athens, Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | 0098 | rev | 0.1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of authorization verification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Federal Office for Information Security (BSI) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS |  |  |  |  |  | Date: |  |  | 2023-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The test purpose requires verification of authentication and authorization, but the test only verifies successful authentication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add verification of user authorization. Accordingly, add successful authorization to the expected results. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency between the test purpose and the execution steps. System functions could be accessed and used without successful authorization. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.3.4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

4.2.3.4.1.1	System functions shall not be used without successful authentication and authorization.

Requirement Name: System functions shall not be used or accessed without successful authentication and authorization.

Requirement Description:

The usage of a system function without successful authentication on basis of the user identity and at least one authentication attribute (e.g. password, certificate) shall be prevented. System functions comprise, for example network services (like SSH, SFTP, Web services), local access via a management console, local usage of operating system and applications. This requirement shall also be applied to accounts that are only used for communication between systems. An exception to the authentication and authorization requirement are functions for public use such as those for a Web server on the Internet, via which information is made available to the public.

Security Objective references: tba.

Test case:

Test Name: TC_SYS_FUN_USAGE

Purpose:

To ensure that system functions shall not be used without successful authentication and authorization.

Procedure and execution steps:

Pre-Conditions:

1. The manufacturer shall supply the list of system functions which include network services, local access via a management console, local usage of operating system and applications.

2. The manufacturer shall supply the list of access entries for system functions.

Execution Steps

The accredited evaluator's test lab is required to execute the following steps:

1.	The tester verifies, based on his/her own experience, that the list is adequate.

2.	The tester verifies that the access entries to use system functions, which are listed by the manufacturer, require successful authentication on basis of the user name and at least one authentication attribute. The tester also verifies that the access entries to use system functions require authorization via an access control mechanism (e.g. Discretionary access control/Ownership/Capabilities or Mandatory access control).This applies to both system functions that are locally accessible and those that are remotely accessible via a network interface.

Expected Results:

1.	The network product does not allow access to any system function provided by the manufacturer without successful user authentication and authorization.

Expected format of evidence:

A testing report provided by the testing agency which will consist of the following information:

- 	Description of executed tests and commands

- 	Relevant output (e.g. Screenshot)

- 	Test result (Passed or not)

********** END OF CHANGE **********