| 3GPP TR ab.cde V0.1.0 (2023-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on security for N32 and SEPP hosted scenarios (FS_N32SEPP_SEC); (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Key issues	7

4.1	Key issue #1: N32 security in mediated roaming scenarios	7

4.1.1	Introduction	7

4.1.2	Key issue details	7

4.1.3	Security requirements	8

4.2	Key issue #2: Security in Hosted SEPP scenarios	8

4.2.1	Key issue details	8

4.2.1.1	Background	8

4.2.1.2	Issues to be studied	9

4.2.2	Threats	10

4.2.3	Security requirements	10

4.X	Key issue #X: tbd	10

4.X.1	Key issue details	10

4.X.2	Security threats	10

4.X.3	Potential security requirements	10

5	Solutions	10

5.0	Mapping of solutions to key issues	10

5.1	Solution #1: Hosted SEPP requirements	11

5.1.1	Introduction	11

5.1.2	Solution details	11

5.1.3	Evaluation	12

5.2	 Solution #2: PRINS for Roaming Hubs	12

5.2.1	 Introduction	12

5.2.2	 Solution details	13

5.2.2.1	 RH Proxy Resolves pSEPP Well-Known FQDN	16

5.2.3	 Evaluation	16

5.3	Solution #3: Solution on N32 security profiles	17

5.3.1	Introduction	17

5.3.2	Solution details	17

5.3.3	Evaluation	18

5.4	Solution #4: Dedicated N32 connections per PLMN in hosted SEPP deployment	19

5.4.1	Introduction	19

5.4.2	Solution details	19

5.4.3	Evaluation	19

5.Y	Solution #Y: tbd	20

5.Y.1	Introduction	20

5.Y.2	Solution details	20

5.Y.3	Evaluation	20

6	Conclusions	20

6.X	KI#X: tbd	20

6.X.1	Analysis	20

6.X.2	Conclusion	20

Annex A (informative): Change history	21

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

# 1	Scope

This clause shall start on a new page.

The present document …

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

It is preferred that the reference to 21.905 be the first in the list.

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	Key issues

## 4.1	Key issue #1: N32 security in mediated roaming scenarios

### 4.1.1	Introduction

In roaming scenarios, the communication between the visited network and the home network is sometimes mediated and routed through intermediaries. The following types of intermediaries exist.

IPX providers: While, traditionally, for some roaming relations, they simply route the traffic, for others they actively modify certain messages in order to establish or enhance interoperability.

Roaming hubs: There are two types of roaming hub, as follows.

- 	Public Roaming Hubs offer a contractual and technical framework that enables operators to avoid entering a very large number of roaming agreements with individual other operators in order to achieve a large roaming coverage. By contracting the roaming hub, the operator obtains access to a large roaming footprint, without individually contracting the operators that are mediated through the hub. Apart from mediating the signalling, roaming hubs typically also mediate the billing. GSMA introduced the concept of roaming hubs to 3GPP in LS S3-213806.

- Operator Group Roaming Hubs offer similar services as public roaming hubs, and are special in that they are used by group network operator companies that reside in the same security domain to consolidate and secure operator group roaming.

The key issue looks at N32 security in intermediary scenarios, in particular roaming hub scenarios that have not been addressed in TS 33.501 so far.

### 4.1.2	Key issue details

The GSMA is working on best practice guidance and related specifications for how operators interact with both IPX and roaming hubs. However, the pre-5G architecture of these methods uses the 4G hop-by-hop security paradigm. In 5G, SEPP communication for roaming relies on end-to-end security by design as specified in TS 33.501, i.e. messages are authenticated and integrity-protected between the SEPP of the visited network and the SEPP of the home network. This creates the need to study in more detail how the migration away from hop-by-hop security can be achieved without loss of the benefits provided by IPX providers and roaming hubs while the 5G security requirements on N32 are met.

In certain scenarios, IPX providers and roaming hubs would need to modify certain IEs. This requires the setup of appropriate modification policies between the roaming partners. Given that, according to TS 33.501,

- all attributes transferred over the N32-f interface shall be integrity protected (clause 5.9.3.3),

- a default data-type encryption policy, which provides confidentiality protection for authentication vectors, cryptographic material and location data, and optionally for SUPI, applies (clause 5.9.3.3), and

- any roaming partner-specific data-type encryption policy takes precedence over the default policy (clause 13.2.3.6),

and further given that

- it is assumed that the set of IEs to be modified depends on the deployment scenario, and

- no default modification policy has been specified,

maintaining a large set of roaming partner-specific modification policies could cause avoidable complexities. It is, therefore, unclear whether the current specification needs to be extended in order to avoid such complexities.

Furthermore, a roaming hub may need to prevent the setup of N32-c between a home network SEPP and a visited network SEPP, e.g., in case the respective operators have not established a commercial roaming relation via the roaming hub.

### 4.1.3	Security requirements

Existing requirements in TS 33.501 on the SEPP should apply in the roaming intermediary scenario unless there are explicit exceptions.

Specifically, the requirements on edge protection of the PLMN should still apply, including end-to-end security for N32-c.

Existing NFs and SCPs should be impacted as least as possible.

For the scenarios involving intermediaries (IPX providers, roaming hubs), requirements are as follows.

- The intermediaries (IPX providers, roaming hubs) shall use their own unique credentials to authenticate themselves.

## 4.2	Key issue #2: Security in Hosted SEPP scenarios

### 4.2.1	Key issue details

#### 4.2.1.1	Background

It has been discussed that SEPPs can be deployed in different ways:

-	Local SEPP: The SEPP is deployed within the PLMN. This is the deployment as specified by TS 33.501.

Figure 5.12.1-1 Local SEPP deployment

-	Outsourced SEPP: The SEPP is outsourced by the PLMN and deployed within the PLMN infrastructure and considered within the PLMN secure domain Several PLMNs which form an Operator Group can use the same outsourced SEPP. This scenario is described as an Operator Group Roaming Hub  in KI#10 and GSMA NG.113 [X].

Figure 5.12.1-2 Outsourced SEPP deployment

-	Hosted SEPP: The SEPP is outsourced by the PLMN to an external entity and deployed outside the PLMN.

Figure 5.12.1-3 Hosted SEPP deployment

The "Hosted SEPP" scenario is subject of this key issue.

#### 4.2.1.2	Issues to be studied

In a Hosted SEPP scenario it is necessary to consider different perspectives when assessing the security perimeter boundaries. From the roaming partners’s perspective, the security perimeter of the PLMN extends to the Hosted SEPP provider, as described in TS 33.501, clause 4.2.1. From the perspective of the PLMN itself, the Hosted SEPP provider operates within its own security domain, especially since the Hosted SEPP potentially serves multiple PLMNs as a Hosted SEPP provider.

Nevertheless, in the Hosted SEPP deployment scenario, the Hosted SEPP is similarly to any SEPP responsible for

- Terminating the N32c/f interface (TLS, PRINS) - N32 side

- Providing topology Hiding - N32 side

- Providing filtering criteria and capabilities – N32 side

- Filtering criteria and possiblly a firewall – Home PMN side.

The following issues are required for studying:

- Whether the external entity, e.g., the IPX, which hosts the SEPP, uses certificates from its own CA or uses certificates issued from the Home PLMN and whether the Home PLMN is willing to consider and use, e.g., the IPX's CA as trusted CA

- Whether the Hosted SEPP can handle private addresses of the Home PMN

- Whether the Hosted SEPP is trusted to enforce market specific regulatory requirements

- In addition, the connection between Hosted SEPP and the Home PMN needs protection, e.g., Topology Hiding, filtering criteria and firewall, etc.

The PKI infrastructure can be deployed in different ways for issuing the digital identities to the hosted SEPP’s PLMN facing SBI and external facing N32.

According to GSMA LS SA3-221737, a PLMN can use both Local SEPPs (managed by PLMN) and Hosted SEPPs, i.e., the PLMN has Local SEPPs each handling a set of roaming relations, while the Hosted SEPPs handle a different set of roaming relations.

### 4.2.2	Threats

In a scenario of Hosted SEPP additional risks arise. These include:

-	attacks on the traffic between PLMN and Hosted SEPP,

-	insufficient protection of the PLMN core,

-	lack of a way to attribute the cause of a security issue to a specific actor (e.g., the operator or the Hosted SEPP provider),

-	an unauthorized actor claiming to be a Hosted SEPP provider of an operator towards its roaming partners, and

-	risks due to one actor operating Hosted SEPPs for multiple PLMNs.

### 4.2.3	Security requirements

The Hosted SEPP scenario introduces security requirements for the connection between PLMNs and the Hosted  SEPP operator.

Existing requirements in TS 33.501 on the SEPP shall apply in the Hosted SEPP scenario, unless there are explicit exceptions. Specifically, edge protection requirements can differ from the existing requirements since the hosted SEPP is not deployed at the edge of the PLMN.

Existing NFs and SCPs should be impacted as least as possible.

Further, the following requirements shall apply:

- The Hosted SEPP providers shall use their own unique credentials to authenticate themselves. Moreover, it shall be possible for operators that receive signalling from a Hosted SEPP provider pertaining to a roaming partner to verify that the Hosted SEPP provider has been authorised by the roaming partner.

- The connection between the PLMN and the Hosted SEPP provider shall be confidentiality, integrity, and replay protected.

- The solution should enable the Hosted SEPP providers to operate SEPPs for multiple PLMNs in a way that isolates the SEPP instances operated for different PLMNs from each other for security reasons. Specifically, if one of the instances gets corrupted or otherwise malfunctions, other instances should remain unaffected as much as possible.

## 4.X	Key issue #X: tbd

### 4.X.1	Key issue details

### 4.X.2	Security threats

### 4.X.3	Potential security requirements

# 5	Solutions

## 5.0	Mapping of solutions to key issues

Table 5.0-1: Mapping of solutions to key issues

| Solutions | Key Issues |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | #X |  |  |  |  |  |  |  |  |  |  |  |
| #Y: |  |  |  |  |  |  |  |  |  |  |  |  |

## 5.1	Solution #1: Hosted SEPP requirements

### 5.1.1	Introduction

This solution addresses key issue #2. It provides input for text that needs to be adapted for clarification of a deployment option for SEPP of a PLMN, if hosted by an entity external to the PLMN.

### 5.1.2	Solution details

This solution is addressing the Hosted SEPP as described in KI#2.

If the Hosted SEPP provider is contracted by several PLMNs, it hosts several distinct Hosted SEPP instances for the different PLMNs.

A PLMN can have a Local SEPP (managed by the PLMN) and a Hosted SEPP (instance), both handling disjoint sets of roaming relations.

It is proposed to add the following definitions to 33.501:

“Hosted SEPP: A SEPP that is hosted and operated by a provider outside the PLMN, for example an IPX provider. From the perspective of roaming partners,  PLMN trust extends to the Hosted SEPP instance representing this PLMN. A Hosted SEPP provider can operate Hosted SEPPs for multiple PLMNs.

Local SEPP: A SEPP of a PLMN that is operated by an MNO.

The Hosted SEPP serves a set of roaming relations that is disjoint from the set of roaming relations served by the Local SEPP.”

It is further proposed to have an addition to the general requirements in 33.501 clause 5.9.3 (Requirements for e2e core network interconnection security) as follows:

“From the roaming partner’s point of view, a Hosted SEPP shall behave in the same way as the SEPP.

All SBI messages transmitted between the PLMN and the Hosted SEPP deployment shall be confidentiality, integrity, and replay protected.

A Hosted SEPP provider shall isolate the processing for each PLMN from the processing for other PLMNs.

A Hosted SEPP provider shall use a different TLS certificate per PLMN, as the MNC and MCC of that PLMN are encoded in the certificate (as specified in TS 23.003, Table 6.1.3c.3-1). This certificate shall be issued to the Hosted SEPP provider under its own unique name, and hence shall contain both the PLMN-ID and its own unique identifier.

A Hosted SEPP shall only be able to obtain a certificate with an MCC/MNC combination if the affected MNO has previously authorized this.”

As roaming with hosted SEPPs potentially increases the number of root CAs in the overall system, the need to support certificate pinning also increases. Having in mind the trust establishment mechanism communicated by the GSMA in and acknowledged by 3GPP in SA3-223910, it is proposed to add the following requirement to 33.501.

“The SEPP shall support Certificate Pinning in the following way: It shall maintain a set of lists containing root CA certificates, as well as a mapping of PLMN-IDs to this set. The mapping associates each PLMN-ID with a given root CA certificate list. During N32-c connection setup, the SEPP shall map the PLMN-ID of the remote SEPP leaf (server or client) certificate to the associated root CA certificate list for the purposes of certificate chain verification. Only the root CA certificates in the associated list shall be treated as trusted during certificate chain verification. If the remote SEPP certificate contains multiple PLMN-IDs that are mapped to different root CA certificate lists, then that certificate shall be rejected.”

Finally, it is proposed to have an additional NOTE referring to Hosted SEPP concept in 33.501 clause 13.1.2 (Protection between SEPPs) as follows:

“If a Hosted SEPP is deployed as the edge of one PLMN and in the same security zone, the same protection requirements apply to this Hosted SEPP as they apply to a SEPP in general. Furthermore, the Hosted-SEPP-specific protection requirements in clause 5.9.3 apply.”

### 5.1.3	Evaluation

Whether to use a Hosted SEPP is a business decision. Nevertheless, it is important to provide a minimal set of requirements covering this deployment option.

Guidelines for naming are provided for Hosted SEPP by GSMA already and 3GPP TS 33.310 may need to be checked for consistency with respect to new security requirements. Such requirements may need to be communicated to GSMA as well.

NOTE: Further evaluation would be necessary, but the study has been stopped. The related key issue is followed up in a separate study.

*********** NEXT CHANGE

## 5.2		Solution #2: PRINS for Roaming Hubs

### 5.2.1		Introduction

This solution addresses key issue #1.

There may be one or two roaming hubs involved in the communication between two PMNs. The roaming hubs require the ability to allow roaming relations between these PMNs via these roaming hubs. Furthermore, roaming hubs with financial liability require the ability to shape roaming traffic between the PMNs in order to control how much roaming traffic they will have to pay for.

PRINS can be used to achieve this, allowing and disallowing establishment of N32c through standard HTTP proxy functionality. PRINS can also be used to dynamically introduce a traffic shaper under control of the RH into the data plane.

### 5.2.2		Solution details

Figure 5.2.2-1: Establishment of N32c via roaming hubs

0:	The cSEPP is configured to make use of RH1

1:	After receiving an initial request, the cSEPP may set up a TLS connection with RH1's proxy. The cSEPP then sends a CONNECT command to RH1's proxy indicating the destination to be pSEPP. If cSEPP does not have access to the pSEPP FQDN, the cSEPP follows the procedure in 6.20.2.x to find the pSEPP FQDN.

2:	RH1's proxy shall verify that cSEPP is allowed to set up a roaming relation with pSEPP. If not, the flow continues with the error message of step 6. Otherwise, it continues with step 3.

3:	If pSEPP requires another roaming hub to be reached, RH1's proxy sets up a TLS connection with RH2's proxy. R1's proxy then sends a CONNECT command to RH2's proxy indicating the destination to be pSEPP. RH2's proxy shall verify that RH1 is allowed to set up a roaming relation with pSEPP. If not, the flow continues with the error message of step 5. Otherwise, it continues with step 4.

4:	If RH2 is present, RH2's proxy, otherwise RH1's proxy shall set up a TCP connection to pSEPP. This connection may be protected by TLS.

5:	If successful, if RH2 is present, RH2 shall return success with code 200.

6:	If successful, RH1 shall return success with code 200.

If successful, pSEPP and cSEPP shall establish a TLS connection via the proxy/proxies for N32c and establish an N32f context, selecting PRINS as the protection mechanism.

Figure 5.2.2-2: N32f via roaming hubs

1:	cSEPP may either intiate a new TLS connection to RH1's proxy, or reuse the TLS connection from setting up N32c. In any case, communication between cSEPP and RH1's proxy shall be TLS protected.

2: 	After receiving a request from an NF, the cSEPP shall encapsulate the request as defined in TS29.573 [9] subclause 5.3.2.3 and send the reformatted message to RH1's proxy. The URL to send the request to shall be located on RH1's proxy, thus RH1's proxy is able to read the request.

3:	If another RH is present, RH1's proxy shall forward the message to RH2's proxy, including a via header and an asserted identity header asserting the identity of the cSEPPs PMN. If RH2 is not present, then this request shall be sent directly to pSEPP.

4:	If another RH is present, RH2's proxy shall forward the request to pSEPP, including a via header giving both RH proxies. An asserted identity header is not necessary as pSEPP can identify the source PMN from the N32f context.

5:	pSEPP shall decapsulate the request, forward it to the pNF and receive the response.

6:	pSEPP shall reformat the response and forward to RH2 (if present) or to RH1 (if RH1 is not present).

7:	If RH2 is present, then RH2's proxy shall forward  the response to RH1's proxy.

8: 	RH1's proxy shall forward the response to cSEPP.

9:	cSEPP shall decapsulate the response, forward it to the pNF

All communication shall be TLS protected on the hops between cSEPP and RH1's proxy, RH1's proxy and RH2's proxy, RH2's proxy and pSEPP.

RH1's proxy and RH2's proxy both may include modifications, or return errors in case of roaming policy violations.

The following two messages sequence charts give examples:

Figure 5.2.2-3: Roaming hub inserting a traffic shaper

To insert a traffic shaper into the roaming user data plane, the RH may include a modifications block that rewrites the N9 endpoints IP address and TEID for the visited networks UPF in message 3, and another modifications block that rewrites the N9 endpoints IP address and TEID for the visited networks UPF in message 7. As these modifications blocks are signed, the modifications are attributable.

Figure 5.2.2-4: Roaming rejecting a session creation request

A roaming hub may also reject a PDU session setup request by returning an error in message 3. This error is forwarded to the vSMF in message 4. This may be triggered by some information in the request visible to the RH. If different SUPIs require different handling, then of course, SUPI shall not be encrypted in the request.

The examples can be extended to include two roaming hubs and potentially even two traffic shapers.

### 5.2.2.1		RH Proxy Resolves pSEPP Well-Known FQDN

When the cSEPP does not have access to the pSEPP FQDN, the cSEPP follow the following procedure to get the pSEPP specific FQDN:

1.	The cSEPP is configured to make use of RH1.

2.	After receiving an initial request, the cSEPP may set up a TLS connection with RH1's proxy. The cSEPP then sends a CONNECT command to RH1's proxy indicating the destination to be the pSEPP well-known FQDN as described in [x].

3.	RH1's proxy shall verify that cSEPP is allowed to set up a roaming relation with pSEPP based on the pSEPP well-know FQDN. If the cSEPP is allowed to setup a romaing relationship with pSEPP, the RH1's HTTP proxy shall resolve the pSEPP well-known FQDN to a specific pSEPP FQDN in the HPLMN following GSMA PRDs [10] and [11].

4.	The RH shall respond by a 307 HTTP/2 response with the Location response header containing the https scheme, the discovered pSEPP FQDN, and the discovered n32-c port, if non-standard port is discovered.

5.	cSEPP shall validate that the received pSEPP FQDN belongs to the HPLMN that the cSEPP is trying to reach.

### 5.2.3		Evaluation

The solution allows a RH to allow or deny roaming traffic between operators based on the contractual relations with the RH. In addition, it allows traffic shaping of the user plane roaming traffic between two PMNs making use of roaming hubs by dynamically introducing a traffic shaper.

The solution exposes user plane traffic to roaming hub's traffic shapers (when present). However, at both PMNs, the presence of the traffic shapers is visible in the modifications blocks. Furthermore, modifications performed by the roaming hubs are attributable.

NOTE: Further evaluation would be necessary, but the study has been stopped. The related key issue is followed up in a separate study.

*********** NEXT CHANGE

## 5.3	Solution #3: Solution on N32 security profiles

### 5.3.1	Introduction

This solution is addressing aspects of KI#1, in the attempt to improve practicability of the N32 PRINS solution for IPX or roaming hub as intermediaries. It proposes to introduce profiles for the N32 security solution.

The following requirements on roaming via N32 are defined in 33.501:

- The solution shall support application layer mechanisms for addition, deletion and modification of message elements by intermediate nodes except for specific message elements described in the present document.

NOTE: Typical example for such a case is IPX providers modifying messages for routing purposes.

- The solution shall provide confidentiality and/or integrity end-to-end between source and destination network for specific message elements identified in the present document. For this requirement to be fulfilled, the SEPP – cf [2], clause 6.2.17 shall be present at the edge of the source and destination networks dedicated to handling e2e Core Network Interconnection Security. The confidentiality and/or integrity for the message elements is provided between two SEPPs of the source and destination PLMN–.

- The destination network shall be able to determine the authenticity of the source network that sent the specific message elements protected according to the preceding bullet. For this requirement to be fulfilled, it shall suffice that a SEPP in the destination network that is dedicated to handling e2e Core Network Interconnection Security can determine the authenticity of the source network.

- The solution should have minimal impact and additions to 3GPP-defined network elements.

- The solution should be using standard security protocols.

- The solution shall cover interfaces used for roaming purposes.

- The solution should take into account considerations on performance and overhead.

- The solution shall cover prevention of replay attacks.

- The solution shall cover algorithm negotiation and prevention of bidding down attacks.

- The solution should take into account operational aspects of key management.

3GPP has defined two security mechanisms for 5G Roaming Security, specifically for N32-f interface protection: TLS (Transport Layer Security) and PRINS (ALS: Application Layer Security).

Different stakeholders voiced that they may require the flexibility provided by 3GPP specs, selecting between transport layer security and application layer security, based on the interconnection schema or business needs.

This solution provides answers to the question for simplification. One of the arguments given was that the security model needs to avoid cumbersome negotiation procedures, which however in PRINS are relevant, as security configuration parameters and protected information elements need to be agreed between roaming partners and provided to IPX providers.

### 5.3.2	Solution details

To facilitate and simplify the deployment and operation of PRINS as one N32 security solution, it is proposed to introduce security profiles.

N32-c negotiation for PRINS is enhanced to allow selecting the existing scheme (for backward compatibility and high security requirements voiced in discussions) or selecting one or several security profiles.

- Only by selecting "full PRINS“, negotiation of a cipher suite and exchange of modification and encryption policies is needed (current schema).

- If a pre-defined profile, e.g. "profile A" or "profile B", is chosen, a pre-defined security profile will be negotiated between SEPPs, and IPX can be instructed equally.

- "integrity-only PRINS" could be then one option, which in current understanding means, that JSON objects are created without encryption policies but integrity protected.

NOTE: integrity-only PRINS may however not be preferable, since AVs and authorization tokens need protection

With this information, during N32-c handshake, if the PRINS enhanced profile, e.g., “B”, is chosen, then both SEPPs (VPLMN and HPLMN) know how to handle the communication on the N32-f interface and the intermediary IPX providers as well. I.e., a profile indicator during N32-c negotiation phase can be propagated as an indication of the selected PRINS profile to the IPX; since only PRINS can be chosen, N32-f will always be based on application layer.

| Enumaration value | Description |
| --- | --- |
| TLS | TLS security |
| PRINS - Profile full - Profile integrity-only - Profile A - Profile B - … - Operator defined profile | Protocol for N32 Interconnect security with subcategories to indicate full usage of PRINS, with integrity protection onl or specific profiles |

Figure 5.3.2-1: Example of N32 security profile

If PRINS with "full PRINS" is chosen, configuration parameters can still be negotiated/exchanged, which keeps market open to those, really wanting this high security option

a.	Modification policy. A modification policy indicates which IEs can be modified by an IPX provider of the sending SEPP.

b.	Data-type encryption policy. A data-type encryption policy indicates which types of data will be encrypted by the sending SEPP.

c.	Cipher suites for confidentiality and integrity protection, when application layer security is used to protect HTTP messages between them.

d.	N32-f context ID. The N32-f context ID identifies the set of security related configuration parameters applicable to a protected message received from a SEPP in a different PLMN.

If PRINS with any other profile is chosen, the following configuration parameters need to be negotiated/exchanged and profiles need to be defined.

- A PRINS profile indicating a predefined set of one or more of the above policies.

NOTE: Data type encryption policy for integrity-only PRINS profile: this policy will not specify any data type to be confidentiality protected; Modification policy for integrity-only PRINS profile: this policy will not specify any IE subject to be modifiable. Still, integrity protection is provided.

### 5.3.3	Evaluation

TBD

*********** NEXT CHANGE

## 5.4	Solution #4: Dedicated N32 connections per PLMN in hosted SEPP deployment

### 5.4.1	Introduction

This solution addresses key issue #12. This solution describes the requirements for allowing several PLMNs to be customer of a SEPP provider and use the hosted SEPP functionality.

### 5.4.2	Solution details

The connection between the customer PLMN and the Hosted SEPP provider is protected in terms of confidentiality, integrity, and against replays by means of a secure channel. Several options exist for this secure channel, for example NDSIP (TLS, IPsec).

To fulfil the isolation requirement, the Hosted SEPP provider operates a dedicated SEPP instance for each PLMN it serves. This includes the usage, for each customer PLMN, of at least one separate N32c (and N32f) connection(s) towards the roaming partners. That is, it is prohibited to mix the signalling of multiple customer PLMNs over a single N32 connection due to the isolation requirement. Moreover, the TLS certificates used for N32 connections contain both the PLMN ID(s) of the customer PLMN and the own unique identifier of the hosted SEPP operator.

NOTE: The details of identifying the subject in the TLS certificate should be defined by GSMA.

It is further required that the secure channel endpoint (e.g., NDSIP endpoint) at the PLMN acts as a firewall that drops SEPP traffic that does not conform to configured filtering rules.

### 5.4.3	Evaluation

As the N32-c connections of a customer PLMN terminates at the hosted SEPP, the hosted SEPP represents the endpoint of the security relation with that PLMN’s roaming partners.

Compared to the setting with a local, i.e., non-hosted SEPP, the attack surface in the setting of a hosted SEPP is increased because

- the Hosted SEPP provider is put into a position where it can inject signalling associated with one of its customers into a connection associated with another, and

- the source of unwanted traffic cannot be immediately and uniquely attributed as being injected by the Hosted SEPP provider or one of its customers' PLMNs (such attribution works only in a mediated way, i.e., requiring the hosted SEPP operator to cooperate in tracing back the source of signalling)

It is not possible to fully eliminate these shortcomings, because the operator of the hosted SEPP is explicitly trusted to represent the PLMN on the N32 interface as its network edge. These shortcomings can, however, be addressed by a robust governance framework which

- specifies a strict naming scheme that uniquely identifies the operator of the hosted SEPP

- defines conditions under which the hosted SEPP operators are granted the necessary TLS certificates

- installs monitoring mechanisms with rapid reaction, e.g., TLS certificate revocation makes use of the isolation requirement to enable roaming partners to selectively disable N32 connections mediated by the hosted SEPP provider without risking a situation of "overblocking".

The above mitigations fall within the realm of GSMA.

## 5.Y	Solution #Y: tbd

### 5.Y.1	Introduction

### 5.Y.2	Solution details

### 5.Y.3	Evaluation

# 6	Conclusions

## 6.X	KI#X: tbd

### 6.X.1	Analysis

### 6.X.2	Conclusion

Annex A (informative):
Change history

| Date | Meeting | TDoc | Subject/Comment | New version |
| --- | --- | --- | --- | --- |
| 2023-02 | SA3#110 | S3-230654 | Skeleton of TR eSBA SEC | 0.0.0 |
|  |  | S3-23XXXX | S3-23abcd Shifting KIs and solutions to new TR | 0.1.0 |