3GPP TSG-SA3 Meeting #110 	S3-230674

Athens, Greece, 20 - 24 February 2023

Title:	draft_ Reply LS R2-2213337 LS on security for selective SCG activation

Response to:	R2-2213337

Release:	Rel-18

Work Item:	NR_mob_enh2-Core

Source:	SA3

To:	RAN2

Cc:

Contact person:	Stawros Orkopoulos

stawros dot orkopoulos at nokia dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	no attachments

# 1	Overall description

SA3 thanks RAN2 for their incoming LS on security for selective SCG activation.

SA3 discussed the security issues for the security of selective SCG activation and arrived at following principles to be followed:

1. The same counter value or S-KgNB in repeated connections with a secondary cell cannot be reused. To avoid this reuse, counter value needs to be unique, per cell per connection. If the RRC Reconfiguration message is not repeated for every connection establishment, to maintain the above security principle, the UE should be configured by the MN with a monotonically increasing Counter.

2. To align with existing Dual connectivity solution, target SN should be provided with the computed S-KgNB by the MN, only after the UE established the connection with it, not before the UE is connected to it.

3. Any Count mis-synchronisations (detected by the failure of security between the UE and the SN), can be handled as an error condition and re-synched over the RRC connection between the MN and the UE.

4. For the Count wrap around situations, current DC solution, KgNB re-keying is retained.

# 2	Actions

To  RAN2

ACTION: 	SA3 requests RAN2 to take a note of the above security principles while developing specifications for this feature. SA3 will specify an additional sub-clause in TS 33.501 under 6.10 for the security of selective SCG activation.

# 3	Dates of next TSG SA WG 3 meetings

SA3#110Bis-e	17 - 21 April 2023	Electronic meeting

SA3#111	22 - 26 May 2023		EU (TBD)