3GPP TSG-SA3 Meeting #110 	S3-230687

# Athens, Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | 0102 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of auto-launch verification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Federal Office for Information Security (BSI), Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSCAS_5G |  |  |  |  |  | Date: |  |  | 2023-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | If a network product is equipped with more than one type of physical port for removable media devices, each of the ports could be handled by different controllers configured with different settings. Therefore, every available type of physical port that is externally accessible should be tested. It cannot be ensured that any removable media device triggers an automatic launch, e.g. a blank or zeroed disk. The removable device should be in the form that it actually triggers an automatic launch. For verifiability and reproducibility reasons, the required evidences should be as concrete as possible. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add execution step to test every available type of physical port for removable media devices. Add execution step to ensure that the removable media device triggers an automatic launch. Add execution step to verify that there is no automatic launch when a removable media device is inserted. Add an example to concretize the required evidences. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Any type of physical port that is not tested could automatically launch the content of an inserted media device. The test can be passed by executing it with a media device that cannot trigger an automatic launch due to improper format. Bad verifiability and reproducibility due to ambiguous wording of the expected evidences. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.3.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

##### 4.3.3.1.3	No automatic launch of removable media

Requirement Name: No automatic launch of removable media

Requirement Description:

The network product shall not automatically launch any application when removable media device such as CD-, DVD-, USB-Sticks or USB-Storage drive is connected. If the operating system supports an automatic launch, it shall be deactivated unless it is required to support availability requirements.

Test Case:

Test Name: TC_NO_AUTO_LAUNCH_OF_REMOVABLE_MEDIA

Purpose:

To verify that the network product does not launch any applications automatically when a removable media device is connected. Any such feature should be deactivated.

Procedure and execution steps:

Pre-Condition

If the network product is provisioned with the necessary physical ports/drives (CD/DVD drive, USB port, etc.) then the test case applies.

Execution Steps

1.	The tester log in the network product.

2.	For all available physical ports which are externally accessible:

a.	The tester prepares a removable media device (e.g. CD, DVD, USB-Sticks and/or USB-Storage drives) that contain any kind of autostart file suitable for this port type.

b.	The tester inserts the prepared media device into the network product under test.

3.	The tester verifies that the media device is not automatically mounted and there is no automatic application launch triggered by its insertion.

Expected Results:

The network product does not launch any applications to open the contents in the removable media device.

In Linux® machines, the removable media device is not automatically mounted in the filesystem.

Expected format of evidence:

Evidence can be presented in the form of logs/screenshot/screen-capture on how the network product responds when any removable media device is attached to it (e.g. the output log of the UNIX® mount command before and after insertion of the removable media device).

********** END OF CHANGE **********