3GPP TSG-SA3 Meeting #109 	S3-230702

# Athens,Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1529 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | HNTRA procedure for SoR case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | HN_Auth |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As agreed in the conclusion of TR 33.741, HNTRA procedure is to be defined for different use cases. It is recommended to specify a new Home Network Triggered Authentication procedure for which the UDM is the central point that decides when to trigger the procedure, and its behaviour is controlled by the Home Network. That means that the UDM triggers interaction with the serving network to request the execution of primary authentication. So, we are adding it for SoR wrap around case in this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 6.14.2.3 is updated for SoR wraparound case |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned with the TR conclusion |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.14.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************ START OF CHANGES

#### 6.14.2.3	SoR Counter

The AUSF and the UE shall associate a 16-bit counter, CounterSoR, with the key KAUSF.

The UE shall initialize the CounterSoR to 0x00 0x00 when the newly derived KAUSF is  stored (see clause 6.2.2.2). The UE shall store the SoR counter. If the USIM supports both 5G parameters storage and 5G parameters extended storage, then CounterSoR shall be stored in the USIM. Otherwise, CounterSoR shall be stored in the non-volatile memory of the ME

To generate the SoR-MAC-IAUSF, the AUSF shall use the CounterSoR. The CounterSoR shall be incremented by the AUSF for every new computation of the SoR-MAC-IAUSF. The CounterSoR is used as freshness input into SoR-MAC-IAUSF and SoR-MAC-IUE derivations as described in the Annex A.17 and Annex A.18 respectively, to mitigate the replay attack. The AUSF shall send the value of the CounterSoR (used to generate the SoR-MAC-IAUSF) along with the SoR-MAC-IAUSF to the UE. The UE shall only accept CounterSoR value that is greater than stored CounterSoR value. The UE shall store the received CounterSoR, only if the verification of the received SoR-MAC-IAUSF is successful. The UE shall use the stored CounterSoR received from the HPLMN, when deriving the SoR-MAC-IUE for the SoR acknowledgement.

The AUSF and the UE shall maintain the CounterSoR for lifetime of the KAUSF.

The AUSF that supports the control plane solution for steering of roaming shall initialize the CounterSoR to 0x00 0x01 when the newly derived KAUSF is stored (see clause 6.2.2.1). The AUSF shall set the CounterSoR to 0x00 0x02 after the first calculated SoR-MAC-IAUSF, and monotonically increment it for each additional calculated SoR-MAC-IAUSF. The SoR Counter value of 0x00 0x00 shall not be used to calculate the SoR-MAC-IAUSF and SoR-MAC-IUE.

The AUSF shall suspend the SoR protection service for the UE, if the CounterSoR associated with the KAUSF of the UE, is about to wrap around. When a fresh KAUSF is generated for the UE, the CounterSoR at the AUSF is reset to 0x00 0x01 as defined above and the AUSF shall resume the SoR protection service for the UE. Alternatively, when UDM requests AUSF for SoR protection via the Nausf_SoRProtection procedure, the AUSF may detect a SoR wrap around case in advance considering the maximum value of the counters is fixed at 65536 and shall not suspend the SoR protection service for the UE. Please refer to Figure 6.14.2.3-1 for more details.

.

Figure 6.14.2.3-1: Procedure for reauthentication during the SoR counter wrap around

1.	The UE is authenticated and registered at the 5GC.

2.	The UDM decides to perform the SoR procedure. Therefore, the UDM invokes the Nausf_SoRProtection procedure. The AUSF tries to increment the CounterSoR and figures out that the CounterSoR is about to wrap around or will reach the max value at the next trigger. Therefore, AUSF shall provide an indication of "CounterSoR Reaching Max value" to UDM along with the successful result of the Nausf_SoRProtection procedure. This alerts the UDM that the current SoR procedure is fine, but the next follow-up SoR procedure update will cause a counter's wraparound failure if authentication is not performed. The UDM shall store the received indication.

3. The UDM completes the SoR procedure as defined in section 6.14.2.2.

4. If "CounterSoR Reaching Max value" indication is received in step 2, the UDM sends a notification to the AMF with SUPI and the "reauthentication required" flag set to true.

5. The AMF shall respond to UDM about the confirmation of the message received.

7. Based on the received indication, the AMF shall start the primary reauthentication procedure as defined in section 6.1.2, 6.1.3 and 6.1.4.

8. After Key KAUSF is generated, the CounterSoR shall be reset. Therefore, any further SoR trigger at the UDM will be successful.

************ END OF CHANGES