3GPP TSG-SA3 Meeting #109 	S3-230708

# Athens,Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1533 | rev | - | Current version: | 17.08.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement in UPU procedure to protect UPU header-sol1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Please refer to the discussion paper S3-230707 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Providing additional IE containing protected UPU Data + Header to UE along with existing IE where only UPU data is protected. - Old UE not supporting the new IE shall continue as is without any change. They will simply drop the new IE - New UE (enhanced with new solution) shall use the new IE where UPU header is protected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UPU header is not protected. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.15.2.1, 14.1.4, A.19a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************ START OF CHANGES

#### 6.15.2.1	Procedure for UE Parameters Update

The UDM may decide to perform UE parameters update anytime after the UE has been successfully authenticated and registered to the 5G system. The security procedure for the UE parameters update is described below in figure 6.15.2.1-1:

Figure 6.15.2.1-1: Procedure for UE Parameters Update

1)	The UDM decides to perform the UE Parameters Update (UPU) using the control plane procedure while the UE is registered to the 5G system. If the final consumer of any of the UE parameters to be updated (e.g., the updated Routing ID Data) is the USIM, the UDM shall protect these parameters using a secured packet mechanism (see 3GPP TS 31.115 [65]) to update the parameters stored on the USIM. The UDM shall then prepare the UE Parameters Update Data (UPU Data) by including the parameters protected by the secured packet, if any, as well as any UE parameters for which final consumer is the ME (see TS 24.501 [35]).

2-3)	The UDM shall invoke Nausf_UPUProtection service operation message by including the UPU Data to the AUSF to get UPU-MAC-IAUSF and CounterUPU as specified in sub-clause 14.1.4 of this document. The UDM shall select the AUSF that holds the latest KAUSF of the UE.

If the UDM decided that the UE is to acknowledge the successful security check of the received UE Parameters Update Data, then the UDM shall also include Enhanced UPU data where it set the corresponding indication in the UE Parameters Update Data (see TS 24.501 [35]) and include the ACK Indication in the Nausf_UPUProtection service operation message to signal that it also needs the expected UPU-XMAC-IUE, as specified in sub-clause 14.1.4 of this document.

Note: When ACK is required to be included, then both UPU data and corresponding MAC/XMAC and EnhancedUPU data and corresponding MAC/XMAC are included in the message.

If a UE supports the enhanced UPU data then the UE shall process only enhanced UPU data and discard the UPU data. If UE does not support the enhancement, then UE shall process only the UPU data.

The details of the CounterUPU is specified in sub-clause 6.15.2.2 of this document. The inclusion of UE Parameters Update Data in the calculation of UPU-MAC-IAUSF/EnhancedUPU-MAC-IAUSF allows the UE to verify that it has not been tampered by any intermediary. The expected UPU-XMAC-IUE allows the UDM to verify that the UE received the UE Parameters Update Data correctly.

4)	The UDM shall invoke Nudm_SDM_Notification service operation, which includes the UPU transparent container if the AMF supports UPU transparent container, or includes individual IEs comprising the UE Parameters Update Data, UPU-MAC-IAUSF, EnhancedUPU-MAC-IAUSF CounterUPU within the Access and Mobility Subscription data. If the UDM requests an acknowledgement, it shall temporarily store the expected UPU-XMAC-IUE.

5)	Upon receiving the Nudm_SDM_Notification message, the AMF shall send a DL NAS Transport message to the served UE. The AMF shall include in the DL NAS Transport message the transparent container if received from the UDM in step 4. Otherwise, if the UDM provided individual IEs in step 4, then the AMF shall construct a UPU transparent container.

6)	 If UE does not support Enhanced UPU and receiving the DL NAS Transport message, the UE shall calculate the UPU-MAC-IAUSF in the same way as the AUSF (as specified in Annex A.19) on the received UE Parameters Update Data  and the CounterUPU and verify whether it matches the UPU-MAC-IAUSF value received within the UPU transparent container in the DL NAS Transport message. If UE supports Enhanced UPU and receiving the DL NAS Transport message, the UE shall calculate the EnhancedUPU-MAC-IAUSF in the same way as the AUSF (as specified in Annex A.19a) on the received Enhanced UE Parameters Update Data  and the CounterUPU and verify whether it matches the EnhancedUPU-MAC-IAUSF value received within the UPU transparent container in the DL NAS Transport message.

If the verification of UPU-MAC-IAUSF or EnhancedUPU-MAC-IAUSF is successful and the UPU Data contains any parameters that is protected by secured packet (see 3GPP TS 31.115 [65]), the ME shall forward the secured packet to the USIM using procedures in 3GPP TS 31.111 [66]. If the verification of UPU-MAC-IAUSF or EnhancedUPU-MAC-IAUSF is successful and the UPU Data contains any parameters that is not protected by secure packet, the ME shall update its stored parameters with the received parameters in UDM Updata Data.

7) 	If the UDM has requested an acknowledgement from the UE and the UE has successfully verified and updated the UE Parameters Update Data provided by the UDM, then the UE shall send the UL NAS Transport message to the serving AMF. The UE shall generate the UPU-MAC-IUE  as specified in Annex A.20 and include the generated UPU-MAC-IUE in a transparent container in the UL NAS Transport message.

8)	If a transparent container with the UPU-MAC-IUE was received in the UL NAS Transport message, the AMF shall send a Nudm_SDM_Info request message with the transparent container to the UDM.

9)	If the UDM indicated that the UE is to acknowledge the successful security check of the received UE Parameters Update Data, then the UDM shall compare the received UPU-MAC-IUE with the expected UPU-XMAC-IUE that the UDM stored temporarily in step 4.

************ NEXT CHANGES************

### 14.1.4	Nausf_UPUProtection service

The following table illustrates the security related services for UE Parameters Update that AUSF provides.

Table 14.1.4-1: NF services for UE Parameters Update provided by AUSF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nausf_UPUProtection | Protect | Request/Response | UDM |

Service operation name: Nausf_UPUProtection.

Description: The AUSF calculates the UPU-MAC-IAUSF  and/or EnhancedUPU-MAC-IAUSF as specified in the Annex A.19/A.19a of this document using UE specific home key (KAUSF) along with the UE Parameters Update Data  received from the requester NF (see clause A.19/A.19a) and delivers the UPU-MAC-IAUSF, EnhancedUPU-MAC-IAUSF and CounterUPU to the requester NF. If the ACK Indication input is present, then the AUSF shall compute the UPU-XMAC-IUE  and return the computed UPU-XMAC-IUE in the response. The details of the UE Parameters Update Data is specified in TS 24.501 [35] .

Input, Required: Requester ID, SUPI, service name, UE Parameters Update Data .

Input, Optional: ACK Indication.

Output, Required: UPU-MAC-IAUSF, EnhancedUPU-MAC-IAUSF, CounterUPU or error (counter_wrap).

Output, Optional: UPU-XMAC-IUE, (if the ACK Indication input is present, then the UPU-XMAC-IUE  shall be computed and returned).

.

************ NEXT CHANGES************

# A.19a	EnhancedUPU-MAC-IAUSF generation function

When deriving a EnhancedUPU-MAC-IAUSF from KAUSF, the following parameters shall be used to form the input S to the KDF.

-	FC = 0x7B,

-	P0 = UE Parameters Update Data

-	L0 = length of UE Parameters Update Data

- 	P1 = CounterUPU

-	L1 = length of CounterUPU

-	P2 = UPU  header

-	L2 = length of UPU header

The input key Key shall be KAUSF.

The EnhancedUPU-MAC-IAUSF is identified with the 128 least significant bits of the output of the KDF.

************ END OF CHANGES