3GPP TSG-SA3 Meeting #110 	S3-230750

# Athens, Greece, 20 - 24 February 2023	revision of S3-yyxxxx

# Source:	Interdigital

# Title:	Discussion on Lawful Interception and emergency services support for 5G ProSe Layer-3 UE-to-Network Relay

# Document for:	Discussion and endorsement

Agenda Item:	4.10

# 1	Decision/action requested

This contribution discusses LI support for 5G ProSe UE-to-Network Relay

# 2	References

[1] 3GPP TS 33.107: "3G security; Lawful interception architecture and functions"

[2] 3GPP TS 33.127: "Lawful Interception (LI) architecture and functions"

[3]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[4] 3GPP TS 33.503: "Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)"

[5] 3GPP TR 33.740: " Study on security aspects of Proximity Based Services (ProSe) in 5G System (5GS) phase 2"

# 3	Rationale

This contribution discusses the issue of Lawful Interception (LI) and emergency services support with 5G ProSe UE-to-Network Relay and proposals for agreement.

# 4	Discussion

## 4.1. Target identities for Lawful Interception (LI)

TS 33.107 [1] specifies the target identities used by LI in EPS, in particular with respect to ProSe UE-to-NW Relay events such as ProSe Remote UE Report. Possible target identities are: MSISDN, IMSI, IMEI (see TS 33.107 [1], clause 17.3.5).

TS 33.127 [2] specifies the target identities used by LI in 5GS, in particular at AMF or SMF. Possible target identities are: SUPI, PEI, GPSI. It is assumed that LI for 5G ProSe will be specified in TS 33.127 (similarly to ProSe for EPS in TS 33.107).

Observation#1: LI requires permanent UE identities as target identities (e.g., SUPI, PEI, GPSI).

## 4.2. ProSe Remote UE identification with 5G ProSe UE-to-Network relay

TS 24.301 [3] (clause 9.9.4.20) specifies that the Remote User ID in the Remote UE report is set to a permanent UE identity (e.g., IMSI, MSISDN, IMEI).

TS 33.503 [4] (clauses 6.3.3.2.2, 6.3.3.3.2) however specifies that the Remote User ID in the Remote UE report is set to a temporary UE identity (i.e., respectively UP-PRUK ID, CP-PRUK ID).

Observation#2: In EPS, it is possible for MME, S-GW and PDN-GW to detect and notify LI function that a Remote UE identified as target of interception has been connected or disconnected from the ProSe UE-to-NW Relay, based on the permanent identity of Remote UE provided in the Remote UE Report.

Observation#3: In current ProSe UE-to-NW Relay security procedures in TS 33.503, it is not possible for SMF (or AMF) to detect and notify LI function that a Remote UE that is a target of interception has been connected or disconnected from the ProSe UE-to-NW Relay, as only a temporary identity (i.e., PRUK ID) of the Remote UE is provided in the Remote UE Report.

Observation#4: How to obtain a permanent identity of Remote UE during ProSe Remote UE Report procedure is required for LI support, but currently not specified in TS 33.503.

## 4.2. Remote UE identity for emergency services

Similarly, to LI, Remote UE SUPI is required for emergency services. Remote UE SUPI resolution is assumed to be supported as per TR 33.740 [5], sol#27:

NOTE:	It’s assumed that the SMF is able to resolve a PRUK ID into a SUPI.

Observation#5: Remote UE SUPI is required for the support of emergency services provided via 5G ProSe UE-to-Network Relay.

# 5	Detailed proposal

It is proposed to agree on the draftCRs S3-230751 and S3-230752 for the resolution of the permanent identity of Remote UE in 5GC during ProSe Remote UE Report procedure to comply with LI target identities and emergency services requirement.