3GPP TSG-SA3 Meeting #110 	S3-230751

# Athens, Greece, 20 - 24 February 2023	revision of S3-yyxxxx

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.503 | CR | draft-CR | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolution of Remote UE permanent identity in Remote UE Report procedure (UP) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | PROSESA |  |  |  |  |  | Date: |  |  | 2023-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | During the Remote UE Report procedure the 5G ProSe UE-to-Network Relay serving PLMN only obtains the PRUK ID of the 5G ProSe Remote UE which is a temporary identity which is not suitable as a target identity for Lawful Intercept function or emergency services.. The 5GC needs to obtain a permanent identity (e.g., SUPI) of the 5G ProSe Remote UE (as in EPS) to be able to comply with LI target identity or emergency services requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolution of 5G ProSe Remote UE permanent identity using PKMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unable to comply with LI and emergency services requirements for 5G ProSe Remote UE target identities. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3.2, 7.2.1, 7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

##### 6.3.3.2.X	Remote UE Report

This clause describes the Remote UE Report procedure following the establishment of a PC5 link between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay as described in 6.3.3.2.2. The Remote UE Report procedure is initiated by the 5G Layer-3 ProSe UE-to-Network Relay to notify the network that a 5G ProSe Remote UE is connected or disconnected from the 5G ProSe Layer-3 UE-to-Network Relay. The procedure includes how the 5G ProSe Remote UE permanent identity is resolved using the PKMF of the 5G ProSe Remote UE.

Figure 6.3.3.2.x.1-1: Procedure for Remote UE Report via 5G ProSe Layer-3 UE-to-Network Relay (Control Plane)

1.	The 5G ProSe Remote UE establishes a PC5 link with the 5G ProSe UE-to-Network Relay as described in clause 6.3.3.2.2.

2.	The 5G ProSe Layer-3 UE-to-Network Relay sends a Remote UE Report message to the SMF for the PDU session associated with the 5G ProSe Layer-3 UE-to-Network Relay. The 5G ProSe Layer-3 UE-to-Network Relay shall include the UP-PRUK ID as the Remote User ID.

3a.	If the mapping of the Remote User ID and the 5G ProSe Remote UE's SUPI is not available in the SMF of the 5G ProSe UE-to-Network Relay, the SMF of the 5G ProSe UE-to-Network Relay shall discover the PKMF of the 5G ProSe Layer-3 UE-to-Network Relay based on the Remote User ID and send to the PKMF of the 5G ProSe Layer-3 UE-to-Network Relay a Npkmf_RemoteUE_ID_Get Request message, including the Remote User ID of the 5G ProSe Remote UE in the message. The PKMF of 5G ProSe UE-to-Network Relay sends the Npkmf_RemoteUE_ID_Get Request message to the PKMF of the 5G ProSe Remote UE.

3b.	The PKMF of the 5G ProSe Remote UE shall verify the Remote User ID is valid and the PKMF of the 5G ProSe UE-to-Network Relay is authorized to get the SUPI of the 5G ProSe Remote UE. The PKMF of the 5G ProSe Remote UE shall send to the PKMF of the 5G ProSe UE-to-Network Relay a Npkmf_RemoteUE_ID_Get Response message, including the SUPI of the 5G ProSe Remote UE in the message. The PKMF of 5G ProSe UE-to-Network Relay sends the Npkmf_RemoteUE_ID_Get Request message to the SMF. The SMF shall store the Remote User ID, the SUPI of the 5G ProSe Remote UE and the Remote UE info in the 5G ProSe Layer-3 UE-to-Network Relay's SM context for the PDU Session associated with the 5G ProSe Layer-3 UE-to-Network Relay.

4.	The SMF sends Remote UE Report Ack message to the 5G ProSe Layer-3 UE-to-Network Relay including the UP-PRUK ID of the Remote UE.

***** Next change *****

# 7	5G ProSe services

## 7.1	General

This clause provides the specification of the SBA services defined for 5G ProSe.

## 7.2	5G PKMF Services

### 7.2.1	General

The 5G PKMF supports the key request from another 5G PKMF in another PLMN via the new service operation Npkmf_PKMFKeyRequest_ProseKey. The 5G PKMF provides Remote UE ID for a given Remote User ID.

The following table shows the services exposed by 5G PKMF supporting 5G ProSe.

Table 7.2.1-1: 5G ProSe Services provided by 5G PKMF

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Npkmf_PKMFKeyRequest | ProseKey | Request/Response | 5G PKMF |
| Npkmf_RemoteUE_ID | Npkmf_RemoteUE_ID_Get | Request/Response | SMF, 5G PKMF |

***** Next change *****

### 7.2.X	Npkmf_RemoteUE_ID service

##### 7.2.X.1 	Npkmf_RemoteUE_ID_Get service operation

Service operation name: Npkmf_RemoteUE_ID_Get

Description: Provides Remote UE ID for a given Remote User ID.

Input, Required: Remote User ID.

Input, Optional: None.

Output, Required: SUPI.

***** End of changes *****