3GPP TSG-SA3 Meeting #110 	S3-230752

# Athens, Greece, 20 - 24 February 2023	revision of S3-yyxxxx

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.503 | CR | draft-CR | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolution of Remote UE permanent identity in Remote UE Report procedure (CP) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | PROSESA |  |  |  |  |  | Date: |  |  | 2023-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | During the Remote UE Report procedure the 5G ProSe UE-to-Network Relay serving PLMN only obtains the PRUK ID of the 5G ProSe Remote UE which is a temporary identity is not suitable as a target identity for Lawful Intercept function or emergency services. The 5GC needs to obtain a permanent identity (e.g., SUPI) of the 5G ProSe Remote UE (as in EPS) to be able to comply with LI target identity or emergency services requirements. One of the two proposed possible options to select from: resolution of SUPI requested by (A) SMF (preferred) (B) AMF |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolution of 5G ProSe Remote UE permanent identity using PAnF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unable to comply with LI and emergency services requirements for 5G ProSe Remote UE target identities. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3.3, 7.3.2, 7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change (Option A: SMF requests SUPI resolution from PAnF) *****

##### 6.3.3.3.X	Remote UE Report

This clause describes the Remote UE Report procedure following the establishment of a PC5 link between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay as described in 6.3.3.3.2. The Remote UE Report procedure is initiated by the 5G Layer-3 ProSe UE-to-Network Relay to notify the network that a 5G ProSe Remote UE is connected or disconnected from the 5G ProSe Layer-3 UE-to-Network Relay. The procedure includes how the 5G ProSe Remote UE permanent identity is resolved by the PAnF of the 5G ProSe Remote UE.

Figure 6.3.3.3.x.1-1: Procedure for Remote UE Report via 5G ProSe Layer-3 UE-to-Network Relay (Control Plane)

- The 5G ProSe Remote UE establishes a PC5 link with the 5G ProSe UE-to-Network Relay as described in clause 6.3.3.3.2.

- The 5G ProSe Layer-3 UE-to-Network Relay sends a Remote UE Report message to the SMF for the PDU session associated with the 5G ProSe Layer-3 UE-to-Network Relay. The 5G ProSe Layer-3 UE-to-Network Relay shall include the CP-PRUK ID as the Remote User ID.

- If the mapping of the Remote User ID and the 5G ProSe Remote UE's SUPI is not available in the SMF of the 5G ProSe UE-to-Network Relay, the SMF of the 5G ProSe UE-to-Network Relay shall discover the PAnF of the 5G ProSe Remote UE based on the Remote User ID and send to the PAnF of the 5G ProSe Remote UE a CP-PRUK_ID in Npanf_Get Request message, including the Remote User ID of the 5G ProSe Remote UE in the message.

The PAnF of the 5G ProSe Remote UE shall verify the Remote User ID is valid and the SMF of the 5G ProSe UE-to-Network Relay is authorized to get the SUPI of the 5G ProSe Remote UE. The PAnF of the 5G ProSe Remote UE shall send to the SMF a Npanf_Get Respone message., including the SUPI of the 5G ProSe Remote UE in the message. The SMF shall store the Remote User ID, the SUPI of the 5G ProSe Remote UE and the Remote UE info in the 5G ProSe Layer-3 UE-to-Network Relay's SM context for the PDU Session associated with the 5G ProSe Layer-3 UE-to-Network Relay.

- The SMF sends Remote UE Report Ack message to the 5G ProSe Layer-3 UE-to-Network Relay including the CP-PRUK ID of the Remote UE.

##### ***** First change (Option B: AMF requests SUPI resolution from PAnF via AUSF) *****

##### 6.3.3.3.X	Remote UE Report

This clause describes the Remote UE Report procedure following the establishment of a PC5 link between the 5G ProSe Remote UE and the 5G ProSe Layer-3 UE-to-Network Relay as described in 6.3.3.3.2. The Remote UE Report procedure is initiated by the 5G ProSe Layer-3 UE-to-Network Relay to notify the network that a 5G ProSe Remote UE is connected or disconnected from the 5G ProSe Layer-3 UE-to-Network Relay. The procedure includes how the 5G ProSe Remote UE permanent identity is resolved by the PAnF of the 5G ProSe Remote UE.

Figure 6.3.3.3.x.1-1: Procedure for Remote UE Report via 5G ProSe Layer-3 UE-to-Network Relay (Control Plane)

- The 5G ProSe Remote UE establishes a PC5 link with the 5G ProSe UE-to-Network Relay as described in clause 6.3.3.3.2.

- The 5G ProSe Layer-3 UE-to-Network Relay sends a Remote UE Report message to the SMF for the PDU session associated with the 5G ProSe Layer-3 UE-to-Network Relay. The 5G ProSe Layer-3 UE-to-Network Relay shall include the CP-PRUK ID as the Remote User ID.

NOTE 1:	In this option support for multiple Remote User IDs in Remote UE Report is not supported. Each Remote User ID need to be reported using separate Remote UE Report message (similarly to the case of encrypted IMSI remote UE identities, in TS 24.301, clause 6.6.3.2).

- The AMF shall select AUSF based on CP-PRUK ID and forwards the CP-PRUK ID to the AUSF in Nausf_UEAuthentication_ProseGet Request message. The AUSF shall select PAnF based on CP-PRUK ID and forwards the CP-PRUK ID to the PAnF in Npanf_Get Request message. The PAnF shall retrieve the Remote UE's SUPI from the ProSe context based on CP-PRUK ID and send the Remote UE's SUPI to the AUSF in the PAnF in Npanf_Get Respone message. The AUSF shall forward Remote UE's SUPI to the AMF in Nausf_UEAuthentication_ProseGet Response message.

NOTE 2:	As the AMF does not keep a mapping of the Remote User ID and the 5G ProSe Remote UE's SUPI (as done by the SMF), the AMF needs to request 5G ProSe Remote UE SUPI for each Remote UE Report message received, including when the 5G ProSe Remote UE disconnects from the 5G ProSe Layer-3 UE-to-Network Relay.

- The AMF shall forward the received SUPI and the Remote UE Report message to the SMF in Nsmf_PDUSession_UpdateSMContext message. The SMF shall store the Remote User ID, the SUPI of the 5G ProSe Remote UE and the Remote UE info in the 5G ProSe Layer-3 UE-to-Network Relay's SM context for the PDU Session associated with the 5G ProSe Layer-3 UE-to-Network Relay.

- The SMF sends Remote UE Report Ack message to the 5G ProSe Layer-3 UE-to-Network Relay including the CP-PRUK ID of the Remote UE.

***** Next change *****

#### 7.3.2.y	 Nausf_UEAuthentication_ProseGet service operation

Service operation name: Nausf_UEAuthentication_ProseGet.

Description: Provides the 5G ProSe Remote UE's SUPI.

Input, Required: CP-PRUK ID.

Input, Optional: None.

Output, Required: 5G ProSe Remote UE's SUPI.

Output, Optional: None.

***** Next change *****

### 7.5.z 	Npanf_get service

#### 7.5.z.1	Npanf_Get service operation

Service operation name: Npanf_Get.

Description: The NF consumer requests Remote UE's SUPI from the PAnF.

Input, Required: CP-PRUK ID.

Input, Optional: None.

Output, Required: Remote UE's SUPI.

Output, Optional: None.

***** End of changes *****