3GPP TSG-SA3 Meeting #110	S3-230813

# Athens, Greece, 20 - 24 February 2023

# Source:	Qualcomm Incorporated

# Title:	Update the evaluation of solution #8

# Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This contribution proposes to update the evaluation of solution #8 in TR 33.740.

# 2	References

[1]	TR 33.740 v0.5.0

# 3	Rationale

This contribution proposes to update a part of evaluation in solution #8. Particularly, this contribution proposes to update the evaluation to reflect that the solution #8 addresses the security requirements of KI#1 when a U2U relay service identified with an RSC is only used for a single ProSe service. Other aspects of this solution are not evaluated in this contribution.

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TR [1].

***** START OF CHANGES *****

### 6.8.3	Evaluation

This solution addresses the Key Issue #1 when a U2U relay service identified with an RSC is only used for a single ProSe service.

This solution is aligned with the SA2’s procedure defined in TS 23.304 [8].

By using the security materials, the 5G Prose End UEs can securely discover each other via UE-to-UE Relay and the Announcement message can be protected.

This solution fulfills the security requirements of KI#3 based on the DDNMF/PKMF’s authorization by reusing the Discovery procedure defined in clause 6.1.3.2 of TS 33.503 [6]. Once the authorization is passed, the 5G Prose End UEs and UE-to-UE Relay can perform UE-to-UE relay discovery.

Editor’s Note :	Further evaluation is FFS.

***** END OF CHANGES *****