3GPP TSG-SA3 Meeting #110 	S3-230897

# Athens, Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.222 | CR | draftCR | rev | <-> | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Shared key-based UE authentication with certificate-based AF authentication to TS 33.535 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AKMA_GBA_DTLS |  |  |  |  |  | Date: |  |  | 2023-2-6 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In order to complete the work on the DTLS for AKMA Ua* protocol, it is required to specify the DTLS profile for Shared key-based UE authentication with certificate-based AF authentication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add DTLS profile for Shared key-based UE authentication with certificate-based AF authentication |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete work for the DTLS for AKMA Ua* protocol |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2,Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** Start of Changes****

Annex X (normative): 
DTLS profiles for  AKMA Ua*

X.1	DTLS based protocols

X.1.1	General

This Annex covers the aspects specific to the DTLS profiles for AKMA Ua*.

X.1.2	Shared key-based UE authentication with certificate-based AF authentication

This section explains how the procedures specified in Annex B have to be changed when CoAP and DTLS is used between a UE and a AF.

1)	When the CoAP client starts communication via Ua reference point with the AF, it shall establish a DTLS tunnel with the AF. The AF is authenticated to the CoAP client by means of a public key certificate. The CoAP client shall verify that the server certificate corresponds to the FQDN of the AF it established the tunnel with. No client authentication is performed as part of DTLS (no client certificate necessary).

2)	The CoAP client sends an CoAP request to the AF inside the DTLS tunnel. The CoAP client shall indicate to the AF that AKMA-based authentication is supported by adding one constant string(s)  "3gpp-akma" to the  payload. The UE shall send the hostname of the AF in "Option" field.

3)	In response to the CoAP request received from CoAP client over the Ua* reference point,  The AF first verifies that the payload received in the CoAP request in step 2. If the AF selects AKMA for deriving the key, then the AF shall include the "3GPP-bootstrapping-akma" ,  and the FQDN of the AF in the payload field.

4)	On receipt of the response from the AF, the CoAP client shall verify that the FQDN in the payload field corresponds to the FQDN of the AF it established the DTLS connection with. On failure the CoAP client shall terminate the DTLS connection with the AF.

5)	In the following request to AF the CoAP client sends a message with POST method. And in the payload field, A-TID as key id. KAF is used as the key to compute an authentication code. If CBOR  is used, IETF RFC 8152[xx] specified how to compute and verify the MAC.

6)	On receipt of this request the AF shall get the KAF from AAnF. And the AF shall verify the authentication code received from step 5.

7)	After the completion of step 6), UE and AF are mutually authenticated as the DTLS tunnel endpoints.

X.1.3	Shared key-based mutual authentication between UE and AF

**** End of Change 1****

**** Start of Change 2****

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[3]	3GPP TS 23.501: "System Architecture for the 5G System".

[4]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[5]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[6]	IETF RFC 7542: "The Network Access Identifier".

[7]	3GPP TS 33.222: " Generic Authentication Architecture (GAA); Access to network application functions using HypertextTransfer Protocol over Transport Layer Security (HTTPS)".

[8]	Void

[9]	3GPP TS 23.003: "Numbering, addressing and identification".

[10]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[11]	3GPP TS 29.503: "5G System; Unified Data Management Services ".

[xx]	IETF RFC 8152: "CBOR Object Signing and Encryption (COSE)"

**** End of Changes****