3GPP TSG-SA3 Meeting #110 	S3-230898

# Athens, Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.222 | CR | draftCR | rev | <-> | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Shared key-based mutual authentication between UE and NAF to TS 33.222 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AKMA_GBA_DTLS |  |  |  |  |  | Date: |  |  | 2023-2-6 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In order to complete the work on the DTLS for GBA Ua protocol, it is required to specify the DTLS profile for shared key-based mutual authentication between UE and NAF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add DTLS profile for shared key-based mutual authentication between UE and NAF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete work for the DTLS for GBA Ua protocol |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** Start of Changes****

Annex X (normative): 
DTLS profiles for  GBA Ua

X.1	DTLS based protocols

X.1.1	General

This Annex covers the aspects specific to the DTLS profiles for GBA Ua.

X.1.2	Shared key-based UE authentication with certificate-based AF authentication

X.1.3	Shared key-based mutual authentication between UE and AF

X.1.3.1	General

The TLS profile specified in TS 33.210 [13] clause 6.2 apply to DTLS 1.2 and DTLS 1.3 as well.

X.1.3.2	Procedures for DTLS 1.2

The procedures  given in clause 5.4.0.1 of TS 33.222 for TLS 1.2 is also applicable for DTLS 1.2, with the following changes.

At step 3, If the CoAP+DTLS client resides in the ME, Ks_(ext)_NAF shall be used as the GBA shared key. If the CoAP+DTLS client resides in the UICC, Ks_int_NAF shall be used as the GBA shared key. In the selection of the GBA mode by the UE, AKA-based modes shall take priority over GBA_Digest.

The UE derives the DTLS premaster secret from the NAF specific key (Ks_(ext)_NAF if the initiating CoAP+DTLS client resides on the ME or Ks_int_NAF if the initiating CoAP+DTLS client resides on the UICC).

X.1.3.3	Procedures for DTLS 1.3

The procedures given in clause 5.4.0.2 of TS 33.222 for TLS 1.3 is also applicable for DTLS 1.3, with the following changes.

At step 1, If the CoAP+DTLS client resides in the ME, Ks_(ext)_NAF shall be used as the GBA shared key. If the CoAP+DTLS client resides in the UICC, Ks_int_NAF shall be used as the GBA shared key. In the selection of the GBA mode by the UE, AKA-based modes shall take priority over GBA_Digest.

The UE derives the DTLS premaster secret from the NAF specific key (Ks_(ext)_NAF if the initiating CoAP+DTLS client resides on the ME or Ks_int_NAF if the initiating CoAP+DTLS client resides on the UICC).

**** End of Changes****