3GPP TSG-SA3 Meeting #110	S3-230924

# Athens, Greece. 20th February - 24 February 2023

# Source:	Intel

# Title:	Conclusion for KI#3: Removal of EN related to Authorization

# Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

This contribution updates the conclusion on KI#3 in TR 33.738.

# 2	References

[1]   3GPP TR 33.738 V0.4.0 "Study on security aspects of enablers for Network Automation for 5G - phase 3"

[2]   3GPP TR 23.700-81 V2.0.0 " Study of Enablers for Network Automation for 5G System (5GS); Phase 3"

# 3	Rationale

This pCR proposes a way forward to resolve the EN in conclusion for KI#3:

Editor's Note: The granularity of the authorization at the MTLF is ffs.

Today, NRF sends an access token and MTLF ID using existing procedures per TS 23.288. When an NF Service Consumer requests an access token for the Nnwdaf_MLModelProvision_Subscribe to retrieve AI/ML models, the NRF grants the access token for the NF type of the NF Service Consumer or the instance ID of the NF Service Consumer. The NF Service consumer uses Nnwdaf_MLModelProvision service operation to receive ML model ID based on the analytics ID and ADRF ID to retrieve ML model.  MTLF verifies the token.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.738.

*****START CHANGE*****

# 7	Conclusions

## 7.3	Conclusion on Key Issue #3 "Security for AI/ML model storage and sharing"

The conclusions for KI#3 are:

-	Authorization of the model retrieval at the NRF uses OAuth 2.0 token-based authorization. The NRF uses information provided by the MTLF.

Editor's Note : The requirement for NF service consumers (NFc) to be registered in the NRF as part of the authorization procedure is ffs.

Editor's Note : The requirement for the AI/ML model to be stored in encrypted format and corresponding key management aspects are ffs.

-	As per the request of Analytics Id by the NFc, the MTLF performs authorization of the corresponding model retrieval per selected model.

NOTE: the model delivery procedure is to be defined by 3GPP SA2.

- MTLF provides a further granularity of authorization to access the ML model from ADRF to NF service consumers based on ML model ID.

NOTE: ML Model ID-based authorization details are specified during the normative phase.

-	ADRF verifies that the requested AI/ML model can be retrieved by the NF consumer(s) (MTLF or AnLF).

*****END CHANGE*****