# 3GPP TSG-SA3 Meeting #110	S3-230937

# Athens, Greece, 20th – 24th February, 2023

# Source:	OPPO

# Title:	Update Sol #5

# Document for:	Approval

Agenda Item:	5.20

1	Decision/action requested

This pCR proposes to update solution #5 of TR 33.898

2	References

N/A

3	Rationale

This solution proposes to update Sol #5 of TR 33.898.

To avoid the redundant user consent checking, the internal AF can be deemed as the enforcement point to check the user consent for 5GC assistance information exposure when it comes to privacy.

And it is proposed to add the evaluation for this solution.

4	Detailed proposal

********* Begin 1st change*********

## 5.5	Solution #5: Authorization for 5GC assistance information exposure to internal AF

### 5.5.1	Introduction

This solution addresses the Key issue #1 “Privacy and authorization for 5GC assistance information exposure to AF”. It is proposed to reuse the existing OAuth 2.0 based authorization of NF service access for the AF that is internal to the operator’s network.

### 5.5.2	Solution details

Figure 5.5.2-1: Authorization for 5GC assistance information exposure to internal AF

Step 1. A PDU Session between the UE and the AF may have been established.

Step 2. The AF requests the AI/ML service by sending Nnrf_AccessToken_Get Request to the NRF with the access required parameters, expected NF Service name(s), NF type, AF ID.

Step 3. The NRF authorizes the internal AF service access using OAuth 2.0 based authorization mechanism as specified in clause 13.4 of TS 33.501[4].

Step 4. If there is need to check user consent, the internal AIML AF is deemed as an enforcement point to check the user consent reusing the user consent framework defined in Annex V in TS 33.501 [4], and the AF cannot request for data processing to acquire 5GC assistance information unless user consent is granted. Otherwise, this step can be skipped.

If there is no user consent parameter in the UE context stored in AIML AF, the AF invokes Nudm_SDM_Get Request service to retrieve related user consent parameter from the UDM. The request message includes the UE ID, may include purpose of data processing and data processor ID.

Step 5. The AF sends service request to the data provider NF (e.g., NWDAF, NEF) with the access token.

Step 6. The data provider NF replies to AF with the service response, based on the outcome of the AI/ML service procedures.

### 5.5.3	Evaluation

This solution addresses KI #1: Privacy and authorization for 5GC assistance information exposure to AF.

This solution based on the existing NF service accessing authorization mechanism for the internal AF acquires 5GC assistance information. To protect of privacy information being exposed to internal AF, the AF is deemed as an enforcement point, the existing user consent checking and revocation mechanisms can be used.

Editor’s Note: The user consent checking for internal AI/ML AF is FFS.

********* End 1st change*********