3GPP TSG-SA3 Meeting #110	S3-231028

# Anthes, Greece, 20 - 24 February 2023

# Source:	Huawei, HiSilicon

# Title:	Address EN and add evaluation for Sol#26

# Document for:	Approval

Agenda Item:	5.3

1	Decision/action requested

Approve the proposal to resolve EN and to add evaluation for Sol#26 in TR 33.740

2	References

N/A

3	Rationale

The contribution proposes to address the following Editor’s Notes in solution #26 in TR 33.740:

In the 5G eV2X mechanism, UE may be provisioned with multiple Long-term credentials and UE is capable to establish different links for different V2X services. This implicitly specifies that UE knows the way to select between different root keys to use, i.e. based on the information inside the DCR message. This solution proposes a similar method to resolve the third Editor’s Note (It is FFS how to distinguish authorisation for establishing link with Relay UE or Peer UE). Due to the fact that DCR messages include different IEs for different cases (i.e. link with Relay UE or link with Peer UE) as per S2-2301479, the UEs are aware of the purpose of the link establishment. Hence the initiating UE and the peer UE distinguish the corresponding Long-term credential based on the received DCR message. Moreover, the UEs are aware of the purpose of the link establishment message, and further distinguish the authorisation with either Relay UE or End UE.

The first Editor’s Note (How Long-term credential is provisioned is FFS) is deleted by adding the following assumption and clarification:

‘The granularity of Long-term credentials (e.g. per UE and/or per ProSe service and/or per RSC) is left for implementation.’

After passing the Direct Auth and Key Establishment, only the authorised UE can have valid keys to establish the link (i.e. statically authorisation), thus the second Editor’s Note (It is FFS how the two UE’s authorise each other for direct communication) is resolved by adding the description:

‘The initiating UE and peer UE are authorised to each other after successful verification of the Direct Security Mode Command and Direct Security Mode Complete messages.’

4	Detailed proposal

***BEGINNING OF THE 1st CHANGE***

6.26	Solution #26: UE-to-UE relay PC5 connection security establishment

6.26.1	Introduction

This solution addresses Key issue #2 (Security of UE-to-UE Relay) and Key issue #3 (Authorization in the UE-to-UE Relay Scenario). This solution reuses the mechanism of unicast mode 5G ProSe Direct Communication security defined in TS 33.503 [6] to setup the security of UE-to-UE Relay Communication. This solution covers both In-Coverage and Out-of-Coverage under both L2 and L3 UE-to-UE relay scenarios.

6.26.2	Solution details

This solution reuses the mechanism of unicast mode 5G ProSe Direct Communication security defined in TS 33.503 [6] to setup the security of UE-to-UE Relay Communication. The security between UEs based on the provisioned/pre-configured Long-term credentials in the UEs (i.e. source End UE, target End UE and UE-to-UE Relay) involved in the UE-to-UE relay communication. The Long-term credentials are used to form the root of the security of the PC5 unicast link by exchanging the key_est_info during Direct Authentication and Key Establishment procedures as specified in TS 33.503 [6].

The PC5 connection security establishment procedures between the End UE (source End UE or target End UE) and UE-to-UE Relay, or between the End UEs (only for L2 case) follow the procedures below:

0. ProSe UE-to-UE relay communication security-related parameter pre-configuration and provisioning, the security-related parameter includes the Long-term credential and signalling/user plane security policies. The Long-term credentials are provisioned as specified in clause 5.3.3.1.2.1 of TS 33.536 [9]. The granularity of Long-term credentials (e.g. per UE and/or per ProSe service and/or per RSC) is left for implementation.

NOTE 1: Step 0 may happen under network coverage (e.g. provisioned by PCF).

NOTE 2: This solution assumes the UE always has valid Long-term credentials.

1. UE-to-UE discovery procedures to find a peer UE. The peer UE can be source End UE, target End UE and UE-to-UE Relay.

2.	In the case, initiating UE determines it needs to establish a PC5 connection with another UE (i.e. the peer UE), the initiating UE sends the Direct Communication Request (DCR) message and this message is received by the peer UE. Under the condition of initiating UE's signalling integrity security policy is not ‘NOT NEEDED’, the DCR shall include the Key_Est_Info. The message may also include a KNRP ID if the initiating UE has an existing KNRP for the peer UE.

3. The initiating UE and the peer UE exchanges Direct Auth and Key Establishment messages including Key_Est_Info. This is mandatory if the peer UE does not have the KNRP and KNRP ID pair indicated in step 1, and signalling integrity protection is needed.

Due to the fact that DCR messages include different IEs for different cases (i.e. link with Relay UE or link with Peer UE), the initiating UE and the peer UE distinguish the corresponding Long-term credential and authorisation purposes based on the DCR message.

Successful verification of the Direct Security Mode Command assures the initiating UE that the peer UE is authorised to establish the unicast link, successful verification of the Direct Security Mode Complete assures the peer UE that the initiating UE is authorised to establish the unicast link.

4.	In case the signalling integrity protection is needed, peer UE shall calculate KNRP and send a Direct Security Mode Command message to initiating UE, including MSB of KNRP ID.

5.	On receiving the Direct Security Mode Command, the initiating UE shall calculate KNRP and choose the LSB of KNRP ID if signalling integrity protection is needed. Initiating UE shall send a Direct Security Mode Complete message to UE_2 which shall contain the LSB of KNRP ID.

6.26.3	Evaluation

The security of the PC5 link relies on the provisioned/pre-configured security materials. Security keys for each PC5 connection can be generated without the assistance from the network. Therefore, the Source UE and the Target UE can establish the UE-to-UE connection via UE-to-UE Relay regardless of whether they are within or out of network coverage.

Due to the fact that DCR messages include different IEs for different cases (i.e. link with Relay UE or link with Peer UE), the initiating UE and the peer UE distinguish the corresponding Long-term credential and authorisation purposes based on the DCR message.

Successful verification of the Direct Security Mode Command assures the initiating UE that the peer UE is authorised to establish the unicast link, successful verification of the Direct Security Mode Complete assures the peer UE that the initiating UE is authorised to establish the unicast link.

***END OF THE CHANGES***