3GPP TSG-SA3 Meeting #110 	S3-231182

# Athens, Greece, 20 – 24 February 2023

# Source:	Philips International B.V., Apple, Deutsche Telekom

# Title:	Way forward for KI#2

# Document for:	Endorsement

Agenda Item:	5.1

# 1	Decision/action requested

This Tdoc makes observations and proposals for endorsement so that we can define and agree on a way forward to conclude KI#2 in TR 33.809.

# 2	References

# 3	Rationale

Although multiple solutions have been proposed for conclusion, by multiple companies, reaching conclusions regarding KI#2 has been difficult so far.

This Tdoc makes observations and proposals for endorsement so that we can define and agree on a way forward for the standardization of a solution addressing the requirements in KI#2.

# 4	Detailed proposal

Observation 1: There is wide company support for solutions that protect system information as defined in KI#2. This includes solutions based on digital signatures, but also other solutions such as Sol. #14 or Sol. #19.

Proposal 1: Agree to standardize a solution to protect the system information addressing KI#2.

Observation 2: Solutions based on digital signatures can protect system information when a UE is both in IDLE or CONNECTED state. However, their usage has a much bigger RAN impact. Solutions #14 and #19 have a lower RAN impact, if any, and despite not offering protection for system information in IDLE mode, they can be used to mitigate some of the issues.

Proposal 2.1: Agree to standardize a solution to protect the system information addressing KI#2 based on Solutions #14 and 19# as a short-term solution (R18).

Proposal 2.2: Take steps to analyze and standardize digital based solutions in the medium-term (>R18).

Observation 3: Key Issue #2 is about: “investigating if and how a new protection mechanism could be introduced against over-the-air attackers who broadcast rogue SI messages or replay previously captured SI messages as-is (without modification). Since SI messages are broadcast messages meant for all UEs, it is not apparent that an integrity and replay protection is strictly necessary. Nevertheless, in general, an integrity and replay protected SIs could add security value by at least making it difficult for over-the-air attackers to succeed in using a rogue SI or a previously captures SI at a later time, e.g., to lure UEs using SI messages with incorrect neighbouring cells, and to send self-crafted or old PWS messages”.

However, replay attacks are hard to defeat and multiple solutions in TR 33.809 indicate this:

- Clause 6.7.3: “Given the fact that replay attack cannot being 100 % eliminated, the method given by this solution helps to mitigate the reply attack.”

- Clause 6.11.3.4: “Editor's Note: How the MitM attack is addressed is FFS.”

- Clause 6.12.3.3: “Editor's Note: How the MitM attack is addressed is FFS.”

- Clause 6.14.2: “Editor’s Note: It is FFS how the MitM attack is addressed.”

- Clause 6.20.2.6.1: “Mitigating Replay Attacks”

- Clause 6.21.2.2.1: “Editor's Note: It is FFS how a message encrypted by gNB is replay protected”

- Clause 6.27.2.2: “Replay mitigation”

Proposal 3: Agree to work on further measures to mitigate replay attacks in the context of KI#2 during normative phase.

Observation 4 (In the context of Proposal 2.2): there are multiple solutions with different types of digital signature algorithms. Different use cases may require different digital signature algorithms.

Proposal 4: Further analyse the impact of supporting multiple digital signing algorithms.

Observation 5 (In the context of Proposal 2.2): The continuous broadcasting of security information, e.g., every 160 ms, ensures that a UE can directly verify the system information in MIB/SIB1; but it leads to a high communication overhead. In other use cases, an on-demand approach for the delivery of security information used to protect system information can be more suitable.

Proposal 5: Consider support of both continuous and on-demand delivery of security information.

Observation 6 (In the context of Proposal 2.2): Quantum-resistant digital signature algorithms are currently being standardized, and thus, it may be meaningful to consider support for such algorithms. However,

- most of those algorithms have long public keys and/or long signatures.

- there are open calls for the standardization of new digital signature algorithms with shorter keys.

Proposal 6: Focus initially on classical digital signature algorithms, e.g., ECDSA, but consider requirements to facilitate future migration.

Observation 7 (In the context of Proposal 2.2): There are multiple solutions relying on digital signatures. Which digital signature is used and how it is used may influence, e.g., the battery life of a UE, communication (e.g., overhead) and security.

Proposal 7.1: Compare the performance of different signature algorithms and make a choice based on the following performance/security indicators:

- Computational complexity for signing

- Computational complexity for verifying

- Signature size

- Public-key size

- Security level

Proposal 7.2: Send LS to ETSI SAGE requesting feedback on potential choices for digital signature algorithms.

Proposal 7.3: Send LS to RAN2 asking (preferred) options regarding the distribution and acquisition of security information protecting system information -- from RAN/performance point of view – to allow a UE to securely connect to the network.