3GPP TSG-SA3 Meeting #110 	S3-231193

- Athens, Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | 0097 | rev | 2 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of hashing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Federal Office for Information Security (BSI), Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSCAS_5G |  |  |  |  |  | Date: |  |  | 2023-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The test case lacks best practice requirements and precise language concerning the hashing of sensitive data. It should be explicitly required to use a non–broken (and) cryptographic hash algorithm since hash algorithm is a broad term and therefore broken and/or non-cryptographic hash algorithms could be used, which is not sufficient for storing sensitive data. It is also best practice to apply a unique salt to every record while hashing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the terms “cryptographic” and “with unique salt per record” to hash algorithm requirement, purpose and expected results. Add checks for correct and salted cryptographic hashing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A data leak would lead to disclosure of sensitive data if a broken or non-cryptographic hash algorithm iss used. If no salt is used one broken record would lead to disclosure of all other records with the same value. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

- 4.2.3.2.3	Protecting data and information in storage

Requirement Name: Protecting data and information in storage

Requirement Description:

For sensitive data in (persistent or temporary) storage read access rights shall be restricted. Files of a system that are needed for the functionality shall be protected against manipulation.

In addition, the folloswing rules apply for:

-	Systems that need access to identification and authentication data in the clear, e.g. in order to perform an authentication. Such systems shall not store this data in the clear, but scramble or encrypt it by implementation-specific means.

-	Systems that do not need access to sensitive data (e.g. user passwords) in the clear. Such systems shall hash this sensitive data with a non-broken cryptographic hash algorithm and unique salt per record.

-	Stored files on the network product: examples for protection against manipulation are the use of checksum or cryptographic methods.

Security Objective references: tba

Test case:

Test Name: TC_PSW_STOR_SUPPORT

Purpose:

Verify that Password storage uses a non-broken one-way cryptographic hash algorithm with a unique salt per record.

Procedure and execution steps:

Pre-Conditions:

-	The tester can access the storage of own user account password.

-	The tester has privileges to change the password.

-	The original password is P1.

-	New passwords are represented by the variable P2.

Execution Steps

1.	The tester accesses the storage where the result of P1 is, and the corresponding hash value is recorded as A

2.	The tester changes the password with P2, then the tester records the storage hash value of the new password as B

3.	The tester repeats the step 2 to get other records with the following requirements for password P2:

-	at least one new password P2 differs from P1 by exactly one bit

-	at least one new password P2 shall be the same as P1

-	at least one new password P2 shall have a different length compared to P1

4. The tester verifies whether all the records comply with the characteristic of a salted one-way cryptographic hash result by checking at least:

a. All collected records contain different hash values, even if the corresponding passwords were identical.

NOTE 1: 	Even if P1 and P2 only differ by one bit, the resulting hash values should differ substantially. (Bit independence criterion)

b. The bit length of the hash values is fixed and independent from the password length.

c. The hash value does not contain any information that could be used for password disclosure. (e.g. contains part of the password in plain text or some sort of password length indicator)

NOTE 2: 	Depending on the implementation the recorded hash values A and B could be stored with their salt combined in some way (e.g. salt is prefix or suffix of hash value). The tester needs to exclude the salt when comparing records

5. The tester verifies, based on his expert knowledge, that the cryptographic hash algorithm used for password storage has no known vulnerabilities and is suitable for password storage.

Expected Results:

All records comply with the characteristic of a salted one-way cryptographic hash result.

Expected format of evidence:

Evidence suitable for the interface, e.g. screenshot containing the operation results.

********** END OF CHANGE **********