3GPP TSG-SA3 Meeting #110 	S3-231241

# Athens, Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 33.501 | CR | 1578 | rev | <Rev#> | Current version: | V18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | R18 Update I.2.2.2.1 for limitations of AAA server (mirror) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-2-8 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | AAA server in clause I.2.2.2 of TS 33.501 has some limitations, which may result in attacks on UE. Details are given as follows. - In PLMN scenarios, only the UE and the HPLMN have the credential (i.e. K). Therefore, UE can ensure it is connected to the right HPLMN if the PLMN can complete the authentication procedure using the credential. - In I.2.2.2, the credential is shared between AAA server and UE. One AAA server may serve multiple NPNs. The AAA server will not authenticate the serving network identity of the UE. - Suppose NPN#1 and NPN#2 are served by the same AAA server, NPN#1 can impersonate NPN#2 by broadcasting the network identity of NPN#2 and use NPN#2 identity to derive keys (e.g., KAMF). Since the AAA server will not authenticate the serving network identity of the UE, the NPN#1 can complete the authentication procedure. UE will connect to the wrong NPN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following limitations about the AAA server are added to I.2.2.2.1 Note: In clause I.2.2.2, the AAA server will not authenticate the serving network name of the UE. And one AAA server may serve multiple NPNs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 3GPP operators will not aware limitations of the AAA server in I.2.2.2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | I.2.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***************Start of the Change ****************

I.2.2.2.1	General

The procedures described in this clause enables UEs to access an SNPN which makes use of a credential management system managed by a credential provider external to the SNPN.

In this scenario the authentication server role is taken by the AAA Server. The AUSF acts as EAP authenticator and interacts with the AAA Server to execute the primary authentication procedure.

The architecture for SNPN access using credentials from a Credentials Holder using AAA Server is described in clause 5.30.2.9.2 of TS 23.501 [2].

Note: In clause I.2.2.2.2, the AAA server will not authenticate the serving network name of the UE. And one AAA server may serve multiple NPNs.

***************End of the Change ****************