3GPP TSG-SA3 Meeting #110 	S3-231264

# Athens, Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1582 | rev | <Rev#> | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add HONTRA procedure in the TS 33.501 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | HN_Auth |  |  |  |  |  | Date: |  |  | 2023-02-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The WID on Security aspect of home network triggered primary authentication is approved. Therefore, a new clause should be added to capture the home network triggered primary authentication procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause to capture the home network triggered primary authentication procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - |  |  |  |  |  |  |  |  |
| Clauses affected: |  | new clause 6.1.X and new clause 14.2.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the 1st Change ****************

6.1.X	Home network triggered primary authentication procedure

6.1.X.1	Introduction

In the 5G system, the home network can initiate the primary authentication for which the UDM is the central point that decides when to trigger the procedure. The UDM can interact with the serving network to request the execution of primary authentication in the following use cases:

- The SoR/UPU Counter Wrap around: When the counters are about to wrap around, the home network can trigger the primary authentication in order to refresh the KAUSF key and safely reset the counters in time.

- The KAKMA refresh: To ensure new generated KAF is different from the previous one, the AAnF can request the home network to refresh KAUSF and KAKMA by running the home network triggered primary authentication.

6.1.X.2	Home network triggered primary authentication procedure

The home network triggered primary authentication procedure is shown in Figure 6.1.X.2-1.

Figure 6.1.X.2-1: Home network triggered primary authentication procedure

0.	Upon receiving the re-authentication indicator, the UDM triggers a home network triggered primary authentication procedure. The re-authentication indicator can be one of the following:

-	The application error COUNTER_WRAP included in the Nausf_SoRProtection_response.

-	The application error COUNTER_WRAP included in the Nausf_UPUProtection_response.

-	The KAKMA refresh indicator send from AAnF.

1.	The UDM sends Re-Authentication Notification, e.g. Nudm_UEAuthentication_ReAuthenticationNotification, (incl. SUPI, Re-Authentication reason) message to the AMF. The UDM provides the reason for sending the reauthentication notification to the serving AMF.  The Re-Authentication reason can be set to one of the following:

-	SoR counter wrap around.

-	UPU counter wrap around.

-	KAKMA refresh.

2.	If the UDM has requested an acknowledgment from the serving AMF, the serving AMF returns the Re-Authentication Notification ACK message to the UDM.

3.	The serving AMF then initiates the primary authentication procedure as defined in clause 6.1.2 and clause 6.1.3.

*************** Start of the 2nd Change ****************

14.2.X Nudm_UEAuthentication_ReAuthenticationNotification service operation

Service operation name: Nudm_UEAuthentication_ReAuthenticationNotification

Description: UDM notifies AMF to initiate the home network triggered primary authentication procedure.

Inputs, Required: SUPI, Re-authentication reason.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

The re-authentication reason can be one of the following:

-	SoR counter wrap around.

-	UPU counter wrap around.

-	KAKMA refresh.

*************** End of the Change ****************