3GPP TSG-SA3 Meeting #110 	S3-231293

# Athens, Greece, 20 - 24 February 2023											(revision of S3-yyxxxx)

# Source:	Ericsson

# Title:	Update for Solution#1

# Document for:	Approval

Agenda Item:	5.15

# 1	Decision/action requested

This contribution provides an update of solution #1.

# 2	References

[1]	3GPP TR 33.890 v0.5.0: Study on security support for Next Generation Real Time Communication services

# 3	Rationale

This contribution provides an update of solution #1.

# 4	Detailed proposal

It is proposed that the following pCR is included in the TR [1].

**** START OF CHANGES ****

#### 6.1.2.1	Solution Description

The Ms reference point as described in TS 24.229 [4] is used to request signing of a SIP Identity header field and verification of a signed assertion in a SIP Identity header field. This enables calling number verification using signature verification and attestation information based on the STIR/SHAKEN framework.

Figure 6.1.2.1-1: Ms reference point operation (see TS 24.229 [XX], Annex V.2)

Here is the SHAKEN Reference Architecture in ATIS-1000074 [5].

Figure 6.1.2.1-2: SHAKEN Reference Architecture

However, securing the display name of a caller was outside the scope of Secure Telephone Identity Revisited (STIR) while draft-ietf-stir-passport-rcd-18 [6] documents an optional mechanism for PASSporT and the associated STIR procedures allowing to sign and verify additional data elements including for example:

-	the name of the calling person or of an entity;

-	the traditional caller ID along with related display information that would be rendered to the called party during alerting;

-	hyperlinks to images, such as logos or pictures of faces, or to similar external profile information;

-	information related to the location of the caller;

-	information related to an organization the caller is associated with, or categories/departments of organizations and institutions;

-	possibly other Rich Call Data (RCD) information elements.

The types of 3rd party user identities as used in IMS need to be aligned with the definitions in [6] and include the calling person's name and job title, information related to the organization the caller is associated with and information related to the caller's location. The overall reference architecture is depicted in Figure 6.X.2.1-1. The 3rd party (Enterprise) network can be connected to the serving IMS network via UNI or NNI interfaces. The serving IMS network handles outbound SIP calls from the Third Party.

There are several options how and where the 3rd party user identities are signed and verified. These options allow for different deployment scenarios, e.g., using UNI or NNI interface between 3rd Party and IMS network, with different levels of impact to the 3rd Party network and the IMS network provider and with different levels of trust relationship between both.

This solution proposes that the originating IMS network invokes the signing on behalf of 3rd Party. In this case the signing AS in Figure 6.1.2.1-1 is invoked by the originating IMS network and the verification of the signature is invoked in the terminating IMS network.

Prerequisites:

1. The 3rd Party specific user identities and the corresponding rich call data information related to each 3rd Party specific user identity that are subject for signing in the originating IMS network are associated to the corresponding IMS identities in a Database. The address of the applicable Database for the user can be stored in HSS.Who owns and how this Database is administered/provisioned is out of scope of the present solution.

NOTE 1: 	In the PBX case, it is assumed that this Database is under control of the 3rd Party (Enterprise) as the enterprise is responsible for assigning the IMS identities which are provided by the IMS operator to their employees and therefore also maintaining the corresponding Rich Call Data information. Otherwise, the Database could be provided by the originating IMS operator and allow certain access to the calling UE via a UE self-management portal. The access to the enterprise administrator (in the PBX case) or the UE (in the single UE case) self-management portal is assumed to be secured. How this is done is out of scope of the present solution.

2. 	Originating IMS network is assumed to have a secure channel to the Database which includes Rich Call Data information. How this secure channel is set-up is out of scope of the present solution.

NOTE 2:  When the Database is located outside the IMS operator domain, the access to the Database can be secured in the same way as the SIP trunk link between the IMS network and the PBX; i.e., using mutual TLS as defined in Clause S.2.2 of TS 23.228 [12].

NOTE 3:  If the user has multiple 3rd party IDs, the RCD data that matches the provided 3rd  party ID for the IMS identity will be selected. If no match is found, a default RCD data record or no RCD data for the IMS identity will be selected depending on the operator policy.

**** END OF CHANGES ****