3GPP TSG-SA3 Meeting #110 	S3-231305

# Athens, Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1588 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [IAB][Rel-17] IAB inter-CU topology adaptation procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When the IAB-MT is migrated from a source IAB-donor-CU to a target IAB-donor-CU, the migrating IAB-node becomes a boundary IAB-node since its IAB-DU retains F1AP with the source IAB-donor-CU while its IAB-MT obtains RRC connectivity with the target IAB-donor-CU detailed in TS 38.401 [78]. In this case, it is required to specify the security procedure to re-establish the F1 security. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The IKEv2 between migrating IAB-node and the source IAB-donor-CU is performed using stored KIAB as PSK. In case of multiple IKEv2, then the migrating IAB-node provides the mapping information of the old and the new IP addresses for the source IAB-donor-CU to identify and use the appropriate KIAB as PSK for IKEv2. In case IPsec tunnel mode is used for F1 interface protection, the migrating IAB-node may use MOBIKE (IETF RFC 4555 [xx]) to migrate the IPsec tunnel to the new IP outer addresses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity in preforming security procedure for IAB inter-CU topology adaptation procedure may make migrating IAB-nodes out-of-service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, M.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************Start of Changes*********************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

……

[skipped for clarity]

[xx]	IETF RFC 4555 (2006-06): "RFC IKEv2 Mobility and Multihoming Protocol (MOBIKE)".

******************Next Change*********************

# M.x	IAB inter-CU topology adaptation and backhaul RLF recovery procedure

In case of the inter-CU migration as specified in clause 8.17.3 and 8.17.4 in TS 38.401 [78], the IAB-MT is migrated from a source IAB-donor-CU to a target IAB-donor-CU. The migrating IAB-node becomes a boundary IAB-node since its IAB-DU retains F1AP with the source IAB-donor-CU after its IAB-MT obtains RRC connectivity with the target IAB-donor-CU (c.f. TS 38.401 [78]).

The F1-C and F1-U connection between the migrating/descendant/recovery IAB-node and the source/Initial IAB-donor-CU are switched to the target path using the new IP address information of the migrating/descendant/recovery IAB-node as specified in TS 38.401 [78]. In this case, if dynamic PSK computation for IKEv2 PSK authentication is supported, then new IKEv2 between migrating/descendant/recovery IAB-node and the source/Initial IAB-donor-CU may be performed using stored KIAB as PSK.

In case IPsec tunnel mode is used for F1 interface protection, the migrating/descendant/recovery IAB-node may use MOBIKE (IETF RFC 4555 [xx]) to migrate the IPsec tunnel to the new IP outer addresses as specified in TS 38.401 [78].

If IPSec transport mode is used and multiple IKEv2 SAs are established between the migrating/descendant/recovery IAB-node and the source IAB-donor-CU (for each assigned TNL addresses of the IAB node), then a new IKEv2 procedures between migrating/descendant IAB-node and the source IAB-donor-CU may be performed using the stored KIAB as the PSK.

- In case of inter-CU topology adaptation, the migrating/descendant IAB-node shall report the mapping of the old and new IP address information it wants to use for F1-C & F1-U traffic to the source IAB-donor-CU, via the gNB-DU CONFIGURATION UPDATE message. Based on the mapping information, the source IAB-donor-CU shall use the KIAB associated with the old IP address as PSK for IKEv2 to establish secure F1 interface for the corresponding new IP address.

- In case of Backhaul RLF recovery, then for F1-C connections, the new IAB-donor-CU shall provide the mapping of the old and new IP address information it wants to use for F1-C traffic to the initial IAB-donor-CU and the recovery IAB-node. Before this, the initial IAB-donor-CU shall provide old IP address of the recovery IAB node to new IAB-donor-CU. For F1-U connections, the recovery IAB-node shall report the mapping of the old and new IP address information it wants to use for F1-U traffic to the initial IAB-donor-CU, via the gNB-DU CONFIGURATION UPDATE message. Based on the mapping information, the initial IAB-donor-CU shall use the KIAB associated with the old IP address as PSK for IKEv2 to establish secure F1 interface for the corresponding new IP address.

*******************End of Changes*********************