3GPP TSG-SA3 Meeting #110 	S3-231355

# Athens, Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1596 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | HONTRA Updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | HN_Auth |  |  |  |  |  | Date: |  |  | 2023-02-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Contribution provides changes related to the home network triggered authentication based on the HONTRA study outcome. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The home network triggered primary authentication related changes are provided in this document. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Home network triggered authentication cannot be supported by the 5GS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.1, 6.1.5, 6.14.2.3, 6.15.2.2, 14.2.6, 14.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***Start of Change 1***

## 5.8	Requirements on the UDM

### 5.8.1	Generic requirements

The long-term key(s) used for authentication and security association setup purposes shall be protected from physical attacks and shall never leave the secure environment of the UDM/ARPF unprotected.

NOTE 1: Security mechanisms for protection of subscription credentials in ARPF are left to implementation.

NOTE 2: Security mechanisms for storage of subscription credentials in the UDR and for the transfer of authentication subscription data (as specified in 3GPP TS 29.505 [70]) between UDR and ARPF are left to implementation.

The UDM shall set the lifetime for the primary authentication based on the operator’s local policy and it determines to trigger a primary (re-)authentication considering the request if any received and the lifetime of the primary authentication.

***End of Change 1***

***Start of Change 2***

### 6.1.5	Home Network Triggered Authentication

The home network triggered authentication enables the UDM to initiate primary (re)authentication of the UE via the AMF/SEAF as shown in Figure 6.1.5-1.

Figure 6.1.5-1: Home Network Triggered Authentication

The home network triggered authentication for the UE works as follows, cf. also Figure 6.1.5-1:

- Following a successful primary authentication of a UE, based on the home operator’s policy the UDM shall set and maintain a lifetime for the primary authentication.

- The AUSF determines the need to request primary authentication from the UDM as follows:

  - If the AUSF determines that the SoR Counter is about to wrap around (as described in Clause 6.14.2.3), (or)

  - If the AUSF determines that the UPU Counter is about to wrap around (as described in Clause 6.15.2.2),

the AUSF sends to UDM, Nudm_UEAuthentication_Trigger  Request message by including SUPI, re-authentication indication, and Cause set as SoR Counter wrap indication (if case a) or UPU Counter wrap indication (if case b).

- The UDM sends to AUSF, a Nudm_UEAuthentication_Trigger  Response message with acknowldgement indication (Ack).

- The UDM on receiving a re-auth indication and Counter wrap indication (i.e., related to UPU/SoR), checks if it is valid based on the lifetime locally configured for the primary authentication related to the SUPI according to the home network operator policy (e.g., checks if a re-authentication request is so early following a recent successful authentication or not, if not can be considered as valid, else use it for recording purposes). The UDM determines to initiate a primary (re-)authentication for the UE based on the Nudm_UEAuthentication_Trigger  Request message received. In a general case, based on home operator policy, the UDM shall determine to initiate a primary (re-)authentication if the lifetime of the primary authentication is about to expire (e.g., to allow the UDM to systematically initiate a re-(authentication) for the UE and to prevent any long term usage of an established security context).

- The UDM sends to AMF/SEAF, Nudm_UEAuthentication Request message with SUPI.

- The The AMF/SEAF, Nudm_UEAuthentication Response message with acknowledgement indication (Ack).

- The AMF/SEAF initiates primary authentication of the UE based on SUPI as described in Clause 6.1.2.

based on any request from the AUSF (i.e., when the UPU Counter or SoR Counter is about to wrap around) or based on the primary authentication lifetime maintained at the UDM.

***End of Change 2***

***Start of Change 3***

#### 6.14.2.3	SoR Counter

The AUSF and the UE shall associate a 16-bit counter, CounterSoR, with the key KAUSF.

The UE shall initialize the CounterSoR to 0x00 0x00 when the newly derived KAUSF is stored (see clause 6.2.2.2). The UE shall store the SoR counter. If the USIM supports both 5G parameters storage and 5G parameters extended storage, then CounterSoR shall be stored in the USIM. Otherwise, CounterSoR shall be stored in the non-volatile memory of the ME

To generate the SoR-MAC-IAUSF, the AUSF shall use the CounterSoR. The CounterSoR shall be incremented by the AUSF for every new computation of the SoR-MAC-IAUSF. The CounterSoR is used as freshness input into SoR-MAC-IAUSF and SoR-MAC-IUE derivations as described in the Annex A.17 and Annex A.18 respectively, to mitigate the replay attack. The AUSF shall send the value of the CounterSoR (used to generate the SoR-MAC-IAUSF) along with the SoR-MAC-IAUSF to the UE. The UE shall only accept CounterSoR value that is greater than stored CounterSoR value. The UE shall store the received CounterSoR, only if the verification of the received SoR-MAC-IAUSF is successful. The UE shall use the stored CounterSoR received from the HPLMN, when deriving the SoR-MAC-IUE for the SoR acknowledgement.

The AUSF and the UE shall maintain the CounterSoR for lifetime of the KAUSF.

The AUSF that supports the control plane solution for steering of roaming shall initialize the CounterSoR to 0x00 0x01 when the newly derived KAUSF is stored (see clause 6.2.2.1). The AUSF shall set the CounterSoR to 0x00 0x02 after the first calculated SoR-MAC-IAUSF, and monotonically increment it for each additional calculated SoR-MAC-IAUSF. The SoR Counter value of 0x00 0x00 shall not be used to calculate the SoR-MAC-IAUSF and SoR-MAC-IUE.

The AUSF shall determine to request UDM to trigger a primary (re-)authenticationfor the UE, if the CounterSoR associated with the KAUSF of the UE, is about to wrap around. The AUSF shall invoke the Nudm_UE Authentication_Trigger Request message by including SUPI, re-authentication indication and cause set as Counter wrap indication. The UDM on receiving the Nudm_UE Authentication_Trigger Request message performs home network triggers primary authentication as described in clause 6.1.5. When a fresh KAUSF is generated for the UE, the CounterSoR at the AUSF is reset to 0x00 0x01 as defined above and the AUSF shall resume the SoR protection service for the UE.

***End of Change 3***

***Start of Change 4***

#### 6.15.2.2	UE Parameters Update Counter

The AUSF and the UE shall associate a 16-bit counter, CounterUPU, with the key KAUSF.

The UE shall initialize the CounterUPU to 0x00 0x00 when the newly derived KAUSF is stored (see clause 6.2.2.2). The UE shall store the UPU counter. If the USIM supports both 5G parameters storage and 5G parameters extended storage, then CounterUPU shall be stored in the USIM. Otherwise, CounterUPU shall be stored in the non-volatile memory of the ME.

To generate the UPU-MAC-IAUSF, the AUSF shall use the CounterUPU. The CounterUPU shall be incremented by the AUSF for every new computation of the UPU-MAC-IAUSF. The CounterUPU is used as freshness input into UPU-MAC-IAUSF and UPU-MAC-IUE derivations as described in the Annex A.19 and Annex A.20 respectively, to mitigate the replay attack. The AUSF shall send the value of the CounterUPU (used to generate the UPU-MAC-IAUSF) along with the UPU-MAC-IAUSF to the UE. The UE shall only accept CounterUPU value that is greater than stored CounterUPU value. The UE shall update the stored CounterUPU with the received CounterUPU, only if the verification of the received UPU-MAC-IAUSF is successful. The UE shall use the CounterUPU received from the UDM, when deriving the UPU-MAC-IUE for the UE Parameters Upadate Data acknowledgement.

The AUSF and the UE shall maintain the CounterUPU for lifetime of the KAUSF.

The AUSF that supports the UE parameters update using control plane procedure shall initialize the CounterUPU to 0x00 0x01 when the newly derived KAUSF is stored (see clause 6.2.2.1). The AUSF shall set the CounterUPU to 0x00 0x02 after the first calculated UPU-MAC-IAUSF, and monotonically increment it for each additional calculated UPU-MAC-IAUSF. The UPU Counter value of 0x00 0x00 shall not be used to calculate the UPU-MAC-IAUSF and UPU-MAC-IUE.

The AUSF shall determine to request UDM to trigger a primary (re-)authentication for the UE, if the CounterUPU associated with the KAUSF of the UE, is about to wrap around. The AUSF shall invoke the Nudm_UE Authentication_Trigger Request message by including SUPI, re-authentication indication and cause set as Counter wrap indication. The UDM on receiving the Nudm_UE Authentication_Trigger Request message performs home network triggers primary authentication as described in clause 6.1.5. When a fresh KAUSF is generated for the UE, the CounterUPU at the AUSF is reset to 0x00 0x01 as defined above and the AUSF shall resume theUE Parameters Update protection service for the UE.

***End of Change 4***

***Start of Change 5***

### 14.2.6	Nudm_UEAuthentication_ Trigger service operation

The following table illustrates the authentication related services for home network triggered primary (re)authentication that UDM provides.

Table 14.2.6-1: NF services for UE Parameters Update provided by AUSF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nudm_UEAuthentication_ Trigger | authentication | Request/Response | AUSF |

Service operation name: Nudm_UEAuthentication_ Trigger.

Description: This service operation allows the NF (i.e., AUSF) to request UDM to trigger a primary (re-)authentication as described in Clause 6.1.5.

Input, Required: SUPI, re-authentication indication, Cause (set as Counter wrap indication by AUSF).

Input, Optional: None.

Output, Required: ACK Indication.

Output, Optional: None.

***End of Change 5***

***Start of Change 6***

### 14.2.7	Nudm_UEAuthentication_ Trigger service operation

The following table illustrates the home network authentication related trigger services provided by the UDM.

Table 14.2.7-1: NF services for UE Parameters Update provided by AUSF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nudm_UEAuthentication | authentication | Request/Response | AMF |

Service operation name: Nudm_UEAuthentication.

Description: This service operation allows the NF (i.e., AMF) to receive the trigger from UDM to initiate a primary (re-)authentication for the UE as described in Clause 6.1.5.

Input, Required: SUPI.

Input, Optional: None.

Output, Required: ACK Indication.

Output, Optional: None.

***End of Change 6***