3GPP TSG-SA3 Meeting #110 	S3-231371

# Athens, Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 33.222 | CR | DRAFT | rev | <Rev#> | Current version: | V17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enable IETF DTLS in Ua protocol |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 4.11 |  |  |  |  |  | Date: |  |  | 2023-2-8 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This contribution enables UE and NAF to leverage DTLS to protect CoAP based Ua protocol. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add DTLS based protocols to clause 5 of TS 33.222. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE and AF cannot leverage DTLS to protect CoAP based Ua protocol. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 5 of TS 33.222 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***************Start of the 1st Change ****************

5	Authentication schemes

5.X  Shared key-based mutual authentication between UE and NAF in DTLS scenarios

5.X.1 Procedures for DTLS 1.2

In step 1, UE sends PSK-based ciphersuits information, in which PSK-based ciphersuits are those supported by the UE, to the AF via DTLS 1.2 ClientHello message [x].

In step 2, If AF chooses to use a specific ciphersuit provided by the UE, AF sends the PSK identity hint (e.g., "3gpp-gba", "3gpp-gba-uicc", and "3gpp-gba- digest") to the UE via DTLS 1.2 ServerKeyExchange message.

In step 3, UE sends GBA PSK identity, which is consisting of PSK identity hint and the B-TID, to the AF via DTLS 1.2 ClientKeyExchange message.

In step 4, AF retrieves the NAF key from 5GC using the B-TID.

The UE and AF realize the mutual authentication based on the NAF key.

5.X.2 Procedures for DTLS 1.3

In step 1, UE sends GBA PSK identities to the AF via ClientHello message of DTLS 1.3 [y]. The PSK identity is consisting of PSK identity hint (e.g., "3gpp-gba", "3gpp-gba-uicc",and "3gpp-gba- digest") and the B-TID.

In step 2, if AF chooses to leverage GBA PSK identity to build the secure connection, AF sends the selected GBA PSK identity to UE via ServerHello message of DTLS 1.3. AF retrieves the NAF key  from 5GC using B-TID.

The UE and AF realize the mutual authentication based on the NAF key.

***************End of the 1st Change ****************

***************Start of the 2nd Change****************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 23.002: "Network architecture".

[2]	3GPP TS 22.250: "IP Multimedia Subsystem (IMS) group management"; Stage 1".

[3]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture".

[4]	Void

[5]	3GPP TS 33.141: "Presence Service; Security".

[6]	Void.

[7]	Void.

[8]	Void.

[9]	IETF RFC 2818 (2000): "HTTP Over TLS".

[10]	Void

[11]	Void

[12]	Void

[13]	3GPP TS 33.210: "3G Security; Network Domain Security; IP network layer security".

[14]	Void.

[15]	 Void.

[16]	3GPP TS 33.221: "Generic Authentication Architecture (GAA); Support for subscriber certificates".

[17]	Void.

[18]	3GPP TS 24.109: "Bootstrapping interface (Ub) and network application function interface (Ua); Protocol details".

[19]	3GPP TS 29.109: "Generic Authentication Architecture (GAA), Zh and Zn Interface based on the Diameter protocol; Stage 3".

[20]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[21]	Void.

[22]	 Void.

[23]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[24]	W3C Working Draft (Jan 22, 2013): "HTML5.1 Nightly – A vocabulary and associated APIs for HTML and XHTML", work in progress, http://dev.w3.org/html5/spec/.

[25]	IETF RFC 5929 (2010): "Channel Bindings for TLS".

[26]	W3C Working Draft (Oct 20, 2011): "File API", work in progress, http://www.w3.org/TR/FileAPI/.

[27]	W3C Candidate Recommendation (Dec 8, 2011): "Web Storage", work in progress, http://www.w3.org/TR/webstorage/

[28]	3GPP TS 33.203: "3G security; Access security for IP-based services".

[29]	IETF RFC 5705 (2010): "Keying Material Exporters for Transport Layer Security (TLS)".

[30]	IETF RFC 8446 (2018): "The Transport Layer Security (TLS) Protocol Version 1.3".

[31]	IETF RFC 7235: "Hypertext Transfer Protocol (HTTP/1.1): Authentication".

[32]	IETF RFC 7616: "HTTP Digest Access Authentication".

[33]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[x]	IETF RFC 6374: "Datagram Transport Layer Security Version 1.2"

[y]	IETF RFC 9146: "The Datagram Transport Layer Security (DTLS) Protocol Version 1.3"

***************End of the 2nd Change****************