3GPP TSG-SA3 Meeting #110 	draft_S3-231407

# Athens, Greece, 20 - 24 February 2023										revision of S3-231121

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1569 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | control on NSSAA procedures for multi registrations in two PLMNs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Multiple simultaneous Network Slice Specific Authentication and Authorization (NSSAA) may get triggered by AMFs of different PLMNs because of following reasons: • UE may initiate a registration to an AMF of the second network, which may trigger a NSSAA on a S-NSSAI, before NSSAA of the S-NSSAI triggered in the first network has completed. If this happens the EAP layer in the UE will not be able to handle parallel EAP authentication with the same EAP server and EAP id. • From the network side, the AAA-S which is authenticating the UE for the network slice in the NSSAA procedure, may initiate reauthentication and reauthorization of the UE, technically anytime after the authentication for any reason. If this happens the NSSAAF may trigger either or both AMFs to initiate new NSSAA procedure(s). This may lead to race conditions in the UE involving AMFs in two networks. Restriction on concurrent NSSAA procedures for multiple accesses is already described in 3GPP specifications, e.g. TS 23.501 clause 5.15.5.2.1, TS 23.502, clause 4.2.2.2.2, and TS 33.501 clause 16.2. However, the issue has not been addressed in AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify behavior of NSSAAF in AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure, especially in multi PLMNs case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NSSAA may be failed in multi-registration scenario . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

################### start the 1st change ##################

## 16.4	AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure

Figure 16.4-1: AAA Server initiated Network Slice-Specific Re-authentication and Re-authorization procedure

0.	The UE is registered in 5GC via an AMF. The AMF ID is stored in the UDM.

1.	The AAA-S requests the re-authentication and re-authorization for the Network Slice specified by the S-NSSAI/ENSI in the Re-Auth Request message, for the UE identified by the GPSI in this message. This message is sent to an AAA-P, if the AAA-P is used (e.g. the AAA Server belongs to a third party), otherwise it may be sent directly to the NSSAAF. If an AAA-P is present, the AAA-P relays the Reauthentication Request to the NSSAAF.

2.	The NSSAAF checks whether the AAA-S is authorized to request the re-authentication and re-authorization by checking the local configuration of AAA-S address per S-NSSAI. If success,the NSSAAF requests UDM for the AMF serving the UE using the Nudm_UECM_Get (GPSI, AMF Registration) service operation. The UDM provides the NSSAAF with the AMF ID of the AMF serving the UE.

3.	The NSSAAF provides an acknowledgement to the AAA protocol Re-Auth Request message. If the AMF is not registered in UDM the procedure is stopped here.

4.	If the AMF is registered in UDM, the NSSAAF requests the relevant AMF to re-authenticate/re-authorize the S-NSSAI for the UE using the Nnssaaf_NSSAA_Re-authenticationNotification service operation. The AMF is implicitly subscribed to receive Nnssaaf_NSSAA_Re-authenticationNotification service operations. The NSSAAF may discover the Callback URI for the Nnssaaf_NSSAA_Re-authenticationNotification service operation exposed by the AMF via the NRF.

The AMF acknowledges the notification of Re-authentication request.

If NSSAAF receives two different AMF addresses from the UDM, the NSSAAF notifies one AMF first, then may notify the other AMF after completed the first re-authentication unsuccessfully.

5.	If the UE is registered with the S-NSSAI in the Mapping Of Allowed NSSAI, the AMF triggers the NSSAA procedure defined in clause 16.3 for the UE identified by the GPSI and the Network Slice identified by the S-NSSAI received from the NSSAAF.

If the UE is registered but the S-NSSAI is not in the Mapping Of Allowed NSSAI, the AMF removes any status of the corresponding S-NSSAI subject to Network Slice-Specific Authentication and Authorization in the UE context it may have kept, so that an NSSAA is executed next time the UE requests to register with the S-NSSAI.

###################  end of the change  ##################