3GPP TSG-SA3 Meeting #110 	S3-231412

# Athens, Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.402 | CR | 0148 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Aligning DNS and ICMP security for non-3GPP access with 3GPP access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Annex L of TS 33.401 has the following recommendation for DNS and ICMP security when using 3GPP access to EPC: "The security measures specified in Annex P of TS 33.501 can be used to protect the DNS and ICMP messages that are carried over the user plane." Annex P of TS 33.501 applies to both 3GPP and non-3GPP access to 5GCN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of the statement that security measures in Annex P of TS 33.501 can be used to protect the DNS and ICMP messages that are carried over the user plane, in a new informative annex. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessarily missing security feature in non-3GPP access while it is applicable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S3-231145 |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	IETF RFC 4877: "Mobile IPv6 Operation with IKEv2 and the Revised IPsec Architecture".

[3]	Void.

[4]	IETF RFC 5778: "Diameter Mobile IPv6: Support for Home Agent to Diameter Server Interaction".

[5]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[6]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[7]	IETF RFC 4187 (January 2006): "Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA)".

[8]	3GPP TS 23.003: "Numbering, addressing and identification".

[9]	3GPP TS 33.234: "3G: security; Wireless Local Area Network (WLAN) interworking security" (Release 12).

[10]	IETF RFC 4072 (August 2005): "Diameter Extensible Authentication Protocol (EAP) Application".

[11] 	3GPP TS 33.102: "3G security; Security architecture".

[12]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[13]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[14]	3GPP TS 23.203: "Policy and charging control architecture".

[15]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRAN); Overall description; Stage 2".

[16]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security Architecture".

[17]	IETF RFC 3344: "IP Mobility Support for IPv4".

[18]	IETF RFC 4555: "IKEv2 Mobility and Multihoming Protocol (MOBIKE)".

[19]	IETF RFC 5295: "Specification for the Derivation of Root Keys from an Extended Master Session Key (EMSK)".

[20]	3GPPP TS 24.303: "Mobility Management based on Dual-Stack Mobile IPv6; Stage 3".

[21]	IETF RFC 4433: "Mobile IPv4 Dynamic Home Agent (HA) Assignment".

[22]	3GPP TS 24.302: "Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP access networks; Stage 3  ".

[23]	IETF RFC 5448: "Improved Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA') ".

[24]	3GPP TS 33.222: "Generic Authentication Architecture (GAA); Access to network application functions using Hypertext Transfer Protocol over Transport Layer Security (HTTPS)".

[25]	3GPP TS 29.109: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Generic Authentication Architecture (GAA); Zh and Zn Interfaces based on the Diameter protocol; Stage 3".

[26] - [28]	Void.

[29]	3GPP TS 33.223: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA) Push function".

[30]	IETF RFC 5996: "Internet Key Exchange Protocol Version 2 (IKEv2)".

[31]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[32]	3GPP TS 29.275: "Proxy Mobile IPv6 (PMIPv6) based Mobility and Tunnelling protocols; Stage 3".

[33]	IETF RFC 4739: "Multiple Authentication Exchanges in the Internet Key Exchange (IKEv2) Protocol".

[34]	3GPP TS 33.203: "Access security for IP-based services".

[35]	IETF RFC 3948: "UDP Encapsulation of IPsec ESP Packets".

[36]	IETF RFC 2616: "Hypertext Transfer Protocol -- HTTP/1.1".

[37]	IETF RFC 6347: "Datagram Transport Layer Security Version 1.2".

[38]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[39]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[40]	Federal Information Processing Standard (FIPS) draft standard: "Advanced Encryption Standard (AES)", November 2001.

[41]	RFC 1421, February 1993: "Privacy Enhancement for Internet Electronic Mail: Part I: Message Encryption and Authentication Procedures".

[42]	Open Mobile Alliance OMA-WAP-OCSP V1.0: "Online Certificate Status Protocol Mobile Profile". URL: http://www.openmobilealliance.org/

[43]	IETF RFC 4806, Februari 2007: Online Certificate Status Protocol ("OCS)P Extensions to IKEv2".

[44]	IETF RFC 4282, December 2005: "The Network Access Identifier", (Obsoletes RFC2486)

[45]	IETF RFC 2865, June 2000: "Remote Authentication Dial In User Service (RADIUS)".

[46]	IETF RFC 6696: "EAP Extensions for the EAP Re-authentication Protocol (ERP)".

[47]	IETF RFC 6942: "Diameter Support for the EAP Re-authentication Protocol (ERP)".

[48]	3GPP TS 24.502: "Access to the 3GPP 5G System (5GS) via non-3GPP access networks; Stage 3".

[xx]	3GPP TS 33.501: "Security architecture and procedures for 5G System"

*** NEXT CHANGES ***

Annex X (informative):
Security Aspects of DNS and ICMP

# X.1	General

The security measures specified in Annex P of TS 33.501 [xx] can be used to protect the DNS and ICMP messages that are carried over the user plane.

*** END CHANGES ***