3GPP TSG-SA3 Meeting #110 	S3-231585

# Athens, Greece. 20th February – 24th February 2023

# Source:	Intel

# Title:	Policy-based C-RNTI and TMSI refresh

# Document for:	Approval

Agenda Item:	5.4

# 1	Decision/action requested

It is requested to approve the solution in TR 33.870.

# 2	References

.

# 3	Rationale

The solution addresses the privacy of users, which can be tracked based on the RRC establishment cause as described in KI #2. The asynchronous relationship between C-RNTI allocation and TMSI allocation is prevented using existing mechanisms as proposed below.

# 4	Detailed proposal

SA3 is kindly requested to agree on the pCR below to TR 33.870

*****Start of Changes*****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 24:501: “Non-Access-Stratum (NAS) protocol for 5G System (5GS)”.

[3]	3GPP TR 33.501: " Security architecture and procedures for 5G system".

[4]	IETF RFC 3629: "UTF-8, a transformation format of ISO 10646".

[5]	IETF RFC 7542: "The Network Access Identifier".

[6]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[XX]	3GPP TR 24.501: " Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

*****Next Changes*****

## 6.X	Solution #X: Policy-based C-RNTI and TMSI refresh

### 6.X.1	Introduction

The solution addresses the privacy of users, which can be tracked based on the RRC establishment cause as described in KI #2. The asynchronous relationship between C-RNTI allocation and TMSI allocation is prevented using existing mechanisms as proposed below.

### 6.X.2	Solution details

The solution proposes two steps, and implementation is left to the operator's policy

Network to perform an intra-cell HO after RRC connection establishment, which can be limited to the use-case described in Key issue 2 (i.e., when Establishment Cause is high priority). The intra-cell HO will re-assign the C-RNTI over RRC, encrypted.

For refreshing GUTI, which can be allocated any time by the Network using "common procedures" as described in 24.501[XX] (c.f.5.4.4). Network can use this common procedure based on operator policy. Here is the text for reference from 24.501[XX]

"If the service request procedure was triggered due to 5GSM downlink signalling pending, the procedure for assigning a new 5G-GUTI can be initiated by the network after the transport of the 5GSM downlink signalling.
The following parameters are supported by the generic UE configuration update procedure without the need to request
the UE to perform the registration procedure for mobility and periodic registration update:
a) 5G-GUTI

"

Editor’s Note: The effectiveness of the solution against tracking priority users is FFS.

Editor’s Note: It is FFS when and how often the described C-RNTI re-assignment procedure is executed.

### 6.X.3	Evaluation

*****End of Changes*****