| 3GPP TS 33.523 V1.1.0 (2023-05) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 5G Security Assurance Specification (SCAS); Split gNB product classes (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	gNB-CU-specific security requirements and related test cases	8

4.1	Introduction	8

4.2	Security functional adaptations of requirements and related test cases	8

4.2.1	Introduction	8

4.2.2	Requirements and test cases deriving from 3GPP specifications	8

4.2.2.1	Security functional requirements on the gNB-CU deriving from 3GPP specifications – TS 33.501 [3]	8

4.2.2.1.1	Security functional requirements inherited from gNB	8

4.2.2.1.2	Control plane data confidentiality protection over N2/Xn/F1 interface	9

4.2.2.1.3	Control plane data integrity protection over N2/Xn/F1 interface	10

4.2.2.1.4	User plane data confidentiality protection over N3/Xn/F1 interface	10

4.2.2.1.5	User plane data integrity protection over N3/Xn/F1 interface	10

4.2.3	Technical Baseline	10

4.2.4	Operating systems	11

4.2.5	Web servers	11

4.2.6	Network devices	11

4.3	Adaptations of hardening requirements and related test cases	11

4.4	Adaptations of basic vulnerability testing requirements and related test cases	11

4.4.1	Introduction	11

4.4.2	Port Scanning	11

4.4.3	Vulnerability scanning	11

4.4.4	Robustness and fuzz testing	11

5	gNB-CU-CP-specific security requirements and related test cases	12

5.1	Introduction	12

5.2	Security functional adaptations of requirements and related test cases	12

5.2.1	Introduction	12

5.2.2	Requirements and test cases deriving from 3GPP specifications	12

5.2.2.1	Security functional requirements on the gNB-CU-CP deriving from 3GPP specifications – TS 33.501 [3]	12

5.2.2.1.1	Security functional requirements inherited from gNB	12

5.2.2.1.2	Control plane data confidentiality protection over N2/Xn/F1/E1 interface	13

5.2.2.1.3	Control plane data integrity protection over N2/Xn/F1/E1 interface	13

5.2.2.1.4	Ciphering of user data based on the security policy sent by the SMF	13

5.2.2.1.5	Integrity of user data based on the security policy sent by the SMF	14

5.2.3	Technical Baseline	15

5.2.4	Operating systems	15

5.2.5	Web servers	15

5.2.6	Network devices	16

5.3	Adaptations of hardening requirements and related test cases	16

5.4	Adaptations of basic vulnerability testing requirements and related test cases	16

5.4.1	Introduction	16

5.4.2	Port Scanning	16

5.4.3	Vulnerability scanning	16

5.4.4	Robustness and fuzz testing	16

6	gNB-CU-UP-specific security requirements and related test cases	16

6.1	Introduction	16

6.2	Security functional adaptations of requirements and related test cases	17

6.2.1	Introduction	17

6.2.2	Requirements and test cases deriving from 3GPP specifications	17

6.2.2.1	Security functional requirements on the gNB-CU-UP deriving from 3GPP specifications – TS 33.501 [3]	17

6.2.2.1.1	Security functional requirements inherited from gNB	17

6.2.2.1.2	Control plane data confidentiality protection over E1 interface	17

6.2.2.1.3	Control plane data integrity protection over E1 interface	17

6.2.2.1.4	User plane data confidentiality protection over N3/Xn/F1 interface	18

6.2.2.1.5	User plane data integrity protection over N3/Xn/F1 interface	18

6.2.2.1.6	Integrity protection of user data between the UE and the gNB-CU-UP	18

6.2.2.1.7	Ciphering of user data between the UE and the gNB-CU-UP	19

6.2.3	Technical Baseline	20

6.2.4	Operating systems	20

6.2.5	Web servers	20

6.2.6	Network devices	20

6.3	Adaptations of hardening requirements and related test cases	20

6.4	Adaptations of basic vulnerability testing requirements and related test cases	20

6.4.1	Introduction	20

6.4.2	Port Scanning	20

6.4.3	Vulnerability scanning	20

6.4.4	Robustness and fuzz testing	20

7	gNB-DU-specific security requirements and related test cases	21

7.1	Introduction	21

7.2	Security functional adaptations of requirements and related test cases	21

7.2.1	Introduction	21

7.2.2	Requirements and test cases deriving from 3GPP specifications	21

7.2.2.1	Security functional requirements on the gNB-DU deriving from 3GPP specifications – TS 33.501 [3]	21

7.2.2.1.1	Control plane data confidentiality protection over F1 interface	21

7.2.2.1.2	Control plane data integrity protection over F1 interface	21

7.2.2.1.3	User plane data confidentiality protection over F1 interface	22

7.2.2.1.4	User plane data integrity protection over F1 interface	22

7.2.3	Technical Baseline	22

7.2.4	Operating systems	22

7.2.5	Web servers	22

7.2.6	Network devices	22

7.3	Adaptations of hardening requirements and related test cases	23

7.4	Adaptations of basic vulnerability testing requirements and related test cases	23

7.4.1	Introduction	23

7.4.2	Port Scanning	23

7.4.3	Vulnerability scanning	23

7.4.4	Robustness and fuzz testing	23

Annex A (informative): Change history	24

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The gNB can be deployed as more than one entity by splitting the gNB into gNB-CU and gNB-DU(s) and possibly further splitting the gNB-CU into gNB-CU-CP and gNB-CU-UP(s) (see TS 38.401 [5]). The present document contains objectives, requirements and test cases that are specific to the various split gNB network product classes. Test cases for such deployments are provided and are based upon and borrow heavily from the specification for the gNB product class (see TS 33.511 [6]). The main differences are the inclusion of cases for the F1 signalling and user plane connection and the E1 signalling connection on the top of the gNB cases as well as some revised cases to account for the split functionality. The present document also refers to the Catalogue of General Security Assurance Requirements (see TS 33.117 [2]) and formulates specific adaptions of the requirements and test cases given there, as well as specifying requirements and test cases unique to the various split gNB network product class.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[3]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[4]	3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

[5]	3GPP TS 38.401: "NG-RAN; Architecture description".

[6]	3GPP TS 33.511: "Security Assurance Specification (SCAS) for the next generation Node B (gNodeB) network product class".

[7]	3GPP TS 23.501: "System Architecture for 5G System (5GS)".

[8]	3GPP TS 38.300: "NR and NG-RAN Overall Description".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

gNB-CU	as defined in TS 38.401 [5]

gNB-CU-CP	as defined in TS 38.401 [5]

gNB-CU-UP	as defined in TS 38.401 [5]

gNB-DU			as defined in TS 38.401 [5]

# 4	gNB-CU-specific security requirements and related test cases

## 4.1	Introduction

gNB-CU specific security requirements include both requirements derived from gNB-CU-specific security functional requirements as well as security requirements derived from threats specific to gNB-CU as described in TR 33.926 [4]. Generic security requirements and test cases common to other network product classes have been captured in TS 33.117 [2] and are not repeated in the present document.

## 4.2	Security functional adaptations of requirements and related test cases

### 4.2.1	Introduction

The present clause contains gNB-CU-specific security functional adaptations of requirements and related test cases. Many of the security functional requirements are directly inherited from the gNB product class.

### 4.2.2	Requirements and test cases deriving from 3GPP specifications

#### 4.2.2.1	Security functional requirements on the gNB-CU deriving from 3GPP specifications – TS 33.501 [3]

Editor's Note: The 'W' in the clauses for the references to threats will need to be aligned with the final Annex allocation in TR 33.926.

##### 4.2.2.1.1	Security functional requirements inherited from gNB

The following security functional requirements in clause 4.2.2.1 of TS 33.511 [6] apply to the gNB-CU by changing the gNB to gNB-CU for the entity under test in the test cases and with the below change to threat references:

4.2.2.1.1	Integrity protection of RRC-signalling

Threat References: TR 33.926 [4], clause W.2.2.2 – Control plane data integrity protection.

4.2.2.1.2	Integrity protection of user data between the UE and the gNB

Threat References: TR 33.926 [4], clause W.2.2.4 – User plane data integrity protection.

4.2.2.1.4	RRC integrity check failure

Threat References: TR 33.926 [4], clause W.2.2.2 – Control plane data integrity protection.

4.2.2.1.5	UP integrity check failure

Threat References: TR 33.926 [4], clause W.2.2.4 – User plane data integrity protection.

4.2.2.1.6	Ciphering of RRC-signalling

Threat References: TR 33.926 [4], clause W.2.2.1 – Control plane data confidentiality protection.

4.2.2.1.7	Ciphering of user data between the UE and the gNB

Threat References: TR 33.926 [4], clause W.2.2.3 – User plane data confidentiality protection at gNB.

4.2.2.1.8	Replay protection of user data between the UE and the gNB

Threat References: TR 33.926 [4], clause W.2.2.4 – User plane data integrity protection.

4.2.2.1.9	Replay protection of RRC-signalling

Threat References: TR 33.926 [4], clause W.2.2.2 – Control plane data integrity protection.

4.2.2.1.10	Ciphering of user data based on the security policy sent by the SMF

Threat References: TR 33.926 [4], clause W.2.2.8 – Security Policy Enforcement.

4.2.2.1.11	Integrity of user data based on the security policy sent by the SMF

Threat References: TR 33.926 [4], clause W.2.2.8 – Security Policy Enforcement.

4.2.2.1.12	AS algorithms selection

Threat References: TR 33.926 [4], clause W.2.2.5 – AS algorithm selection and use.

4.2.2.1.13	Key refresh at the gNB

Threat References: TR 33.926 [4], clause W.2.2.7 – Key Reuse.

4.2.2.1.14	Bidding down prevention in Xn-handovers

Threat References: TR 33.926 [4], clause W.2.2.6 – Bidding Down on Xn-Handover.

4.2.2.1.15	AS protection algorithm selection in gNB change

Threat References: TR 33.926 [4], clause W.2.2.5 – AS algorithm selection and use.

4.2.2.1.18	Key update at the gNB on dual connectivity

Threat References: TR 33.926 [4], clause W.2.2.7 – Key Reuse.

4.2.2.1.19	UP security activation in Inactive scenario

Threat Reference: TR 33.926 [4], clause W.2.2.9 – State transition from inactive state to connected state.

##### 4.2.2.1.2	Control plane data confidentiality protection over N2/Xn/F1 interface

NOTE: 	This is based on the security functional requirement on the gNB given in clause 4.2.2.1.16 of TS 33.511 [6] but modified as the gNB-CU supports the F1 interface.

Requirement Name: Control plane data confidentiality protection over N2/Xn/F1 interface.

Requirement Reference: TS 33.501 [3], clauses 5.3.9, 9.2 and 9.4.

Requirement Description: F1-C interface supports confidentiality, integrity and replay protection, the transport of control plane data over N2 is integrity, confidentiality and replay-protected and the transport of control plane data and user data over Xn is integrity, confidentiality and replay-protected as specified in TS 33.501 [3], clauses 5.3.9, 9.2 and 9.4.

Threat References: TR 33.926 [4], clause W.2.2.1 – Control plane data confidentiality protection.

Test Case: The test case in subclause 4.2.3.2.4 of TS 33.117 [2]

##### 4.2.2.1.3	Control plane data integrity protection over N2/Xn/F1 interface

NOTE: This is based on the security functional requirement on the gNB given in clause 4.2.2.1.17 of TS 33.511 [6] but modified as the gNB-CU supports the F1 interface.

Requirement Name: Control plane data integrity protection over N2/Xn/F1 interface

Requirement Reference: TS 33.501 [3], clauses 5.3.9, 9.2 and 9.4.

Requirement Description: F1-C interface supports confidentiality, integrity and replay protection, the transport of control plane data over N2 is integrity, confidentiality and replay-protected and the transport of control plane data and user data over Xn shall be integrity, confidentiality and replay-protected as specified in TS 33.501 [3], clauses 5.3.9, 9.2 and 9.4.

Threat References: TR 33.926 [4], clause W.2.2.2 – Control plane data integrity protection.

Test Case: The test case in subclause 4.2.3.2.4 of TS 33.117 [2].

##### 4.2.2.1.4	User plane data confidentiality protection over N3/Xn/F1 interface

NOTE: 	This is based on the security functional requirement on the gNB given in clause 4.2.2.1.R of TS 33.511 [6] but modified as the gNB-CU supports the F1 interface.

Editor's Note: The 'R' in the clause in the Note referring to TS 33.511 needs correction once the CR to include this test case in TS 33.511 is approved.

Requirement Name: User plane data confidentiality protection over N3/Xn/F1 interface.

Requirement Reference: TS 33.501 [3], clauses 5.3.9, 9.3 and 9.4.

Requirement Description: The gNB supports confidentiality, integrity and replay protection on the gNB DU-CU F1-U interface for user plane, the transport of user data over N3 is integrity, confidentiality and replay-protected, and the transport of control plane data and user data over Xn is integrity, confidentiality and replay-protected as specified in TS 33.501 [3], clauses 5.3.9, 9.3 and 9.4.

Threat References: TR 33.926 [4], clause W.2.2.3 – User plane data confidentiality protection at gNB.

Test Case: The test case in subclause 4.2.3.2.4 of TS 33.117 [2].

##### 4.2.2.1.5	User plane data integrity protection over N3/Xn/F1 interface

NOTE: 	This is based on the security functional requirement on the gNB given in clause 4.2.2.1.S of TS 33.511 [6] but modified as the gNB-CU supports the F1 interface.

Editor's Note: The 'S' in the clause in the Note referring to TS 33.511 needs correction once the CR to include this test case in TS 33.511 is approved.

Requirement Name: User plane data integrity protection over N3/Xn/F1 interface.

Requirement Reference: TS 33.501 [3], clauses 5.3.9, 9.3 and 9.4.

Requirement Description: The gNB supports confidentiality, integrity and replay protection on the gNB DU-CU F1-U interface for user plane, the transport of user data over N3 is integrity, confidentiality and replay-protected, and the transport of control plane data and user data over Xn shall be integrity, confidentiality and replay-protected as specified in TS 33.501 [3], clauses 5.3.9, 9.3 and 9.4.

Threat References: TR 33.926 [4], clause W.2.2.4 – User plane data integrity protection.

Test Case: The test case in subclause 4.2.3.2.4 of TS 33.117 [2].

### 4.2.3	Technical Baseline

The baseline technical requirements are identical to the ones for the gNB product class given in clause 4.2.3 of TS 33.511 [6].

### 4.2.4	Operating systems

There are no gNB-CU-specific additions to clause 4.2.4 of TS 33.117 [2].

NOTE: 	The ICMP changes applied for a gNB only apply for a DU. In a split deployment where the CU(-CP/UP) is deployed in a data center, the CU(-CP/UP) should be treated as any other IP nodes (e.g., UPF) as the data center nodes are assumed to have connectivity to IP networks whereas DU can be considered like a gNB from ICMP threat perspective.

### 4.2.5	Web servers

There are no gNB-CU-specific additions to clause 4.2.5 of TS 33.117 [2].

### 4.2.6	Network devices

These requirements are identical to the ones for the gNB product class given in clause 4.2.6 of TS 33.511 [6].

## 4.3	Adaptations of hardening requirements and related test cases

These requirements are identical to the ones for the gNB product class given in clause 4.3 of TS 33.511 [6].

## 4.4	Adaptations of basic vulnerability testing requirements and related test cases

### 4.4.1	Introduction

There are no gNB-CU specific additions to clause 4.4.1 of TS 33.117 [2].

### 4.4.2	Port Scanning

There are no gNB-CU specific additions to clause 4.4.2 of TS 33.117 [2].

### 4.4.3	Vulnerability scanning

There are no gNB-CU specific additions to clause 4.4.3 of TS 33.117 [2].

### 4.4.4	Robustness and fuzz testing

The test cases under clause 4.4.4 of TS 33.117 [2] are applicable to gNB-CU.

The interface defined for the gNB-CU are in clause 4.2.3 of TS 23.501 [7] and in clause 4.1 of TS 38.300 [8].

According to clause 4.4.4 of TS 33.117 [2], the transport protocols available on the interfaces providing IP-based protocols need to be robustness tested. Following TCP/IP layer model and considering all the protocols over transport layer, for gNB-CU, the following interfaces and protocols are in the scope of the testing:

For N2: the SCTP and NGAP protocols.

For N3: the UDP and GTP-U protocols.

For Xn: the SCTP and XnAP protocols.

For F1-c: the SCTP and F1AP protocols.

For F1-u: the UDP and GTP-U protocols.

NOTE: There could be other interfaces and/or protocols requiring testing under clause 4.4.4 of TS 33.117 [2].

# 5	gNB-CU-CP-specific security requirements and related test cases

## 5.1	Introduction

gNB-CU-CP specific security requirements include both requirements derived from gNB-CU-CP-specific security functional requirements as well as security requirements derived from threats specific to gNB-CU-CP as described in TR 33.926 [4]. Generic security requirements and test cases common to other network product classes have been captured in TS 33.117 [2] and are not repeated in the present document.

## 5.2	Security functional adaptations of requirements and related test cases

### 5.2.1	Introduction

The present clause contains gNB-CU-CP-specific security functional adaptations of requirements and related test cases. Many of the security functional requirements are directly inherited from the gNB product class.

### 5.2.2	Requirements and test cases deriving from 3GPP specifications

#### 5.2.2.1	Security functional requirements on the gNB-CU-CP deriving from 3GPP specifications – TS 33.501 [3]

Editor's Note: The 'X' in the clauses for the references to threats will need to be aligned with the final Annex allocation in TR 33.926.

##### 5.2.2.1.1	Security functional requirements inherited from gNB

The following security functional requirements from clause 4.2.2.1 of TS 33.511 [6] apply to the gNB-CU-CP by changing the gNB to gNB-CU-CP for the entity under test in the test cases and with the below changes of threat reference:

4.2.2.1.1	Integrity protection of RRC-signalling

Threat References: TR 33.926 [4], clause X.2.2.2 – Control plane data integrity protection.

4.2.2.1.4	RRC integrity check failure

Threat References: TR 33.926 [4], clause X.2.2.2 – Control plane data integrity protection.

4.2.2.1.6	Ciphering of RRC-signalling

Threat References: TR 33.926 [4], clause X.2.2.1 – Control plane data confidentiality protection.

4.2.2.1.9	Replay protection of RRC-signalling

Threat References: TR 33.926 [4], clause X.2.2.2 – Control plane data integrity protection.

4.2.2.1.12	AS algorithms selection

Threat References: TR 33.926 [4], clause X.2.2.3 – AS algorithm selection and use.

4.2.2.1.13	Key refresh at the gNB

Threat References: TR 33.926 [4], clause X.2.2.5 – Key Reuse.

4.2.2.1.14	Bidding down prevention in Xn-handovers

Threat References: TR 33.926 [4], clause X.2.2.4 – Bidding Down on Xn-Handover.

4.2.2.1.15	AS protection algorithm selection in gNB change

Threat References: TR 33.926 [4], clause X.2.2.3 – AS algorithm selection and use.

4.2.2.1.18	Key update at the gNB on dual connectivity

Threat References: TR 33.926 [4], clause X.2.2.5 – Key Reuse.

4.2.2.1.19	UP security activation in Inactive scenario

Threat Reference: TR 33.926 [4], clause X.2.2.7 – State transition from inactive state to connected state.

##### 5.2.2.1.2	Control plane data confidentiality protection over N2/Xn/F1/E1 interface

NOTE: 	This is based on the security functional requirement on the gNB given in clause 4.2.2.1.16 of TS 33.511 [6] but modified as the gNB-CU-CP supports the F1 and E1 interfaces.

Requirement Name: Control plane data confidentiality protection over N2/Xn/F1/E1 interface.

Requirement Reference: TS 33.501 [3], clauses 5.3.9, 5.3.10, 9.2 and 9.4

Requirement Description: F1-C interface supports confidentiality, integrity and replay protection, the E1 interface between CU-CP and CU-UP is confidentiality, integrity and replay protected, the transport of control plane data over N2 is integrity, confidentiality and replay-protected and the transport of control plane data and user data over Xn is integrity, confidentiality and replay-protected as specified in TS 33.501 [3], clauses 5.3.9, 5.3.10, 9.2 and 9.4.

Threat References: TR 33.926 [4], clause X.2.2.1 – Control plane data confidentiality protection.

Test Case: The test case in subclause 4.2.3.2.4 of TS 33.117 [2]

##### 5.2.2.1.3	Control plane data integrity protection over N2/Xn/F1/E1 interface

NOTE: 	This is based on the security functional requirement on the gNB given in clause 4.2.2.1.17 of 
TS 33.511 [6] but modified as the CU-CP supports the F1 and E1 interfaces.

Requirement Name: Control plane data integrity protection over N2/Xn/F1/E1 interface.

Requirement Reference: TS 33.501 [3], clauses 5.3.9, 5.3.10, 9.2 and 9.4.

Requirement Description: F1-C interface supports confidentiality, integrity and replay protection, the E1 interface between CU-CP and CU-UP is confidentiality, integrity and replay protected, the transport of control plane data over N2 is integrity, confidentiality and replay-protected and the transport of control plane data and user data over Xn is integrity, confidentiality and replay-protected as specified in TS 33.501 [3], clauses 5.3.9, 5.3.10, 9.2 and 9.4.

Threat References: TR 33.926 [4], clause X.2.2.2 – Control plane data integrity protection.

Test Case: The test case in subclause 4.2.3.2.4 of TS 33.117 [2].

##### 5.2.2.1.4	Ciphering of user data based on the security policy sent by the SMF

NOTE: 	This is based on the security functional requirement on the gNB given in clause 4.2.2.1.10 of 
TS 33.511 [6] but modified as the gNB-CU-CP informs both the gNB-CU-UP and UE whether to use a non-NULL ciphering algorithm or not.

Requirement Name: Ciphering of user data based on the security policy sent by the SMF.

Requirement Reference: TS 33.501 [3], clause 5.3.2.

Requirement Description: The gNB activates ciphering of user data based on the security policy sent by the SMF as specified in TS 33.501 [3], clause 5.3.2.

Threat References: TR 33.926 [4], clause X.2.2.6 – Security Policy Enforcement.

Test Case:

Test Name: TC-UP-DATA-CIP-SMF_gNB-CU-CP

Purpose: To verify that the user data packets are confidentiality protected based on the security policy sent by the SMF via AMF

Pre-Condition:

-	The gNB-CU-CP network product shall be connected in emulated/real network environments. The UE and the 5GC may be simulated.

-	The tester shall have access to the NG RAN air interface.

-	The tester shall have knowledge of the RRC and UP ciphering algorithm and protection keys and of the security keys, etc., needed to decrypt the messages on the E1 interface.

-	RRC ciphering is already activated at the gNB.

Execution Steps:

1.	The tester triggers PDU session establishment procedure by sending PDU session establishment request message.

2.	Tester shall trigger the SMF to send the UP security policy with ciphering protection "required" or "not needed" to the gNB-CU-CP.

3. 	The tester shall capture the Bearer Context Setup Request message sent to the gNB-CU-UP over the E1 interface.

4. 	The tester shall decrypt the Bearer Context Setup Request message.

5.	The tester shall capture the RRC connection reconfiguration procedure between gNB-CU-CP to UE over NG RAN air interface. And filter the RRC connection reconfiguration message sent by gNB-CU-CP to UE.

6.	The tester shall decrypt the RRC connection Reconfiguration message and retrieve the UP ciphering protection indication presenting in the decrypted message.

7.	The tester shall verify if the UP ciphering policy received at gNB-CU-CP is same as the UP ciphering protection indication notified by the gNB-CU-CP to the UE in the RRC connection Reconfiguration message and the gNB-CU-UP in the Bearer Context Setup Request message.

Expected Results:

Both the messages indicate that ciphering is to be used inline with the received policy.

Expected format of evidence:

Evidence suitable for the interface, e.g. Screenshot containing the operational results.

##### 5.2.2.1.5	Integrity of user data based on the security policy sent by the SMF

NOTE: 	This is based on the security functional requirement on the gNB given in clause 4.2.2.1.11 of TS 33.511 [6] but modified as the gNB-CU-CP informs both the gNB-CU-UP and UE whether to use a non-NULL integrity algorithm or not.

Requirement Name: Integrity of user data based on the security policy sent by the SMF.

Requirement Reference: TS 33.501 [3], clause 5.3.2.

Requirement Description: The gNB  activates integrity protection of user data based on the security policy sent by the SMF as specified in TS 33.501 [3], clause 5.3.2.

Threat References: TR 33.926 [4], clause X.2.2.6 – Security Policy Enforcement.

Test Case:

Test Name: TC-UP-DATA-INT-SMF_gNB-CU-CP

Purpose: To verify that the user data packets are integrity protected based on the security policy sent by the SMF.

Pre-Condition:

-	 The gNB-CU-CP network product shall be connected in emulated/real network environments. The UE and the 5GC may be simulated.

-	The tester shall have access to the NG RAN air interface.

-	The tester shall have knowledge of the integrity algorithm and protection keys and of the security keys, etc., needed to decrypt the messages on the E1 interface.

-	RRC integrity and cipher are already activated at the gNB.

Execution Steps:

1.	The tester triggers PDU session establishment procedure by sending PDU session establishment request message.

2.	Tester shall trigger the SMF to send the UP security policy with integrity protection is "required" or "not needed" to the gNB.

3. 	The tester shall capture the Bearer Context Setup Request message sent to the gNB-CU-UP over the E1 interface.

4. 	The tester shall decrypt the Bearer Context Setup Request message.

5.	The tester shall capture the RRC connection reconfiguration message sent by gNB to UE over NG RAN air interface.

6.	The tester shall decrypt the RRC connection reconfiguration message and retrieve the UP integrity protection indication presenting in the decrypted message.

7.	Tester shall check whether UP integrity policy received at gNB-CU-UP is same as the UP integrity protection indication notified by the gNB-CU-CP to the UE in the RRC connection reconfiguration message and the gNB-CU-UP in the Bearer Context Setup Request message.

Expected Results:

Both the messages indicate that integrity is to be used inline with the received policy.

Expected format of evidence:

Evidence suitable for the interface, e.g. Screenshot containing the operational results.

### 5.2.3	Technical Baseline

The baseline technical requirements are identical to the ones for the gNB product class given in clause 4.2.3 of TS 33.511 [6].

### 5.2.4	Operating systems

There are no gNB-CU-CP specific additions to clause 4.2.4 of TS 33.117 [2].

### 5.2.5	Web servers

There are no gNB-CU-CP specific additions to clause 4.2.5 of TS 33.117 [2].

### 5.2.6	Network devices

These requirements are identical to the ones for the gNB product class given in clause 4.2.6 of TS 33.511 [6] except the GTP-U Filtering case in clause 4.2.6.2.4 of TS 33.117 [2] as the gNB-CU-CP does not support user plane interfaces.

## 5.3	Adaptations of hardening requirements and related test cases

These requirements are identical to the ones for the gNB product class given in clause 4.3 of TS 33.511 [6].

## 5.4	Adaptations of basic vulnerability testing requirements and related test cases

### 5.4.1	Introduction

There are no gNB-CU-CP specific additions to clause 4.4.1 of TS 33.117 [2].

### 5.4.2	Port Scanning

There are no gNB-CU-CP specific additions to clause 4.4.2 of TS 33.117 [2].

### 5.4.3	Vulnerability scanning

There are no gNB-CU-CP specific additions to clause 4.4.3 of TS 33.117 [2].

### 5.4.4	Robustness and fuzz testing

The test cases under clause 4.4.4 of TS 33.117 [2] are applicable to gNB-CU-CP.

The interface defined for the gNB-CU-CP are in clause 4.2.3 of TS 23.501 [7] and in clause 4.1 of TS 38.300 [8].

According to clause 4.4.4 of TS 33.117 [2], the transport protocols available on the interfaces providing IP-based protocols need to be robustness tested. Following TCP/IP layer model and considering all the protocols over transport layer, for gNB-CU-CP, the following interfaces and protocols are in the scope of the testing:

For N2: the SCTP and NGAP protocols.

For Xn: the SCTP and XnAP protocols.

For F1-c: the SCTP and F1AP protocols.

For E1: the SCTP and E1AP protocols.

NOTE: There could be other interfaces and/or protocols requiring testing under clause 4.4.4 of TS 33.117 [2].

# 6	gNB-CU-UP-specific security requirements and related test cases

## 6.1	Introduction

gNB-CU-UP specific security requirements include both requirements derived from gNB-CU-UP-specific security functional requirements as well as security requirements derived from threats specific to gNB-CU-UP as described in TR 33.926 [4]. Generic security requirements and test cases common to other network product classes have been captured in TS 33.117 [2] and are not repeated in the present document.

## 6.2	Security functional adaptations of requirements and related test cases

### 6.2.1	Introduction

The present clause contains gNB-CU-UP-specific security functional adaptations of requirements and related test cases. Many of the security functional requirements are directly inherited from the gNB product class.

### 6.2.2	Requirements and test cases deriving from 3GPP specifications

#### 6.2.2.1	Security functional requirements on the gNB-CU-UP deriving from 3GPP specifications – TS 33.501 [3]

Editor's Note: The 'Y' in the clauses for the references to threats will need to be aligned with the final Annex allocation in TR 33.926.

##### 6.2.2.1.1	Security functional requirements inherited from gNB

The following security functional requirements from clause 4.2.2.1 of TS 33.511 [6] apply to the gNB-CU-UP by changing the gNB to gNB-CU-UP for the entity under test in the test cases and with the below changes of threat reference:

4.2.2.1.5	UP integrity check failure

Threat References: TR 33.926 [4], clause Y.2.2.4 – User plane data integrity protection.

4.2.2.1.8	Replay protection of user data between the UE and the gNB

Threat References: TR 33.926 [4], clause Y.2.2.4 – User plane data integrity protection.

##### 6.2.2.1.2	Control plane data confidentiality protection over E1 interface

NOTE: 	This is based on the security functional requirement on the gNB given in clause 4.2.2.1.16 of TS 33.511 [6] but modified as the gNB-CU-UP only supports the E1 interface.

Requirement Name: Control plane data confidentiality protection over E1 interface

Requirement Reference: TS 33.501 [3], clauses 5.3.10.

Requirement Description: The E1 interface between CU-CP and CU-UP is confidentiality, integrity and replay protected as specified in TS 33.501 [3], clauses 5.3.10.

Threat References: TR 33.926 [4], clause Y.2.2.1 – Control plane data confidentiality protection.

Test Case: The test case in subclause 4.2.3.2.4 of TS 33.117 [2].

##### 6.2.2.1.3	Control plane data integrity protection over E1 interface

NOTE: 	This is based on the security functional requirement on the gNB given in clause 4.2.2.1.17 of TS 33.511 [6] but modified as the gNB-CU-UP only supports the E1 interface.

Requirement Name: Control plane data integrity protection over E1 interface

Requirement Reference: TS 33.501 [3], clauses 5.3.10.

Requirement Description: The E1 interface between CU-CP and CU-UP is confidentiality, integrity and replay protected as specified in TS 33.501 [3], clauses 5.3.10.

Threat References: TR 33.926 [4], clause Y.2.2.2 – Control plane data integrity protection.

Test Case: The test case in subclause 4.2.3.2.4 of TS 33.117 [2].

##### 6.2.2.1.4	User plane data confidentiality protection over N3/Xn/F1 interface

NOTE : 	This is based on the security functional requirement on the gNB given in clause 4.2.2.1.R of TS 33.511 [6] but modified as the gNB-CU-UP supports the F1 interface.

Editor's Note: The 'R' in the clause in the Note referring to TS 33.511 needs correction once the CR to include this test case in TS 33.511 is approved.

Requirement Name: User plane data confidentiality protection over N3/Xn/F1 interface.

Requirement Reference: TS 33.501 [3], clauses 5.3.9, 9.3 and 9.4.

Requirement Description: The gNB supports confidentiality, integrity and replay protection on the gNB DU-CU F1-U interface for user plane, the transport of user data over N3 is integrity, confidentiality and replay-protected, and the transport of control plane data and user data over Xn is integrity, confidentiality and replay-protected as specified in TS 33.501 [3], clauses 5.3.9, 9.3 and 9.4.

Threat References: TR 33.926 [4], clause Y.2.2.3 – User plane data confidentiality protection at gNB.

Test Case: The test case in subclause 4.2.3.2.4 of TS 33.117 [2].

##### 6.2.2.1.5	User plane data integrity protection over N3/Xn/F1 interface

NOTE: 	This is based on the security functional requirement on the gNB given in clause 4.2.2.1.S of TS 33.511 [6] but modified as the gNB-CU-UP supports the F1 interface.

Editor's Note: The 'S' in the clause in the Note referring to TS 33.511 needs correction once the CR to include this test case in TS 33.511 is approved.

Requirement Name: User plane data integrity protection over N3/Xn/F1 interface.

Requirement Reference: TS 33.501 [3], clauses 5.3.9, 9.3 and 9.4.

Requirement Description: The gNB supports confidentiality, integrity and replay protection on the gNB DU-CU F1-U interface for user plane, the transport of user data over N3 is integrity, confidentiality and replay-protected, and the transport of control plane data and user data over Xn is integrity, confidentiality and replay-protected as specified in TS 33.501 [3], clauses 5.3.9, 9.3 and 9.4.

Threat References: TR 33.926 [4], clause Y.2.2.4 – User plane data integrity protection.

Test Case: The test case in subclause 4.2.3.2.4 of TS 33.117 [2].

##### 6.2.2.1.6	Integrity protection of user data between the UE and the gNB-CU-UP

NOTE 1: 	This is based on the security functional requirement on the gNB given in clause 4.2.2.1.2 of TS 33.511 [6] but modified as the gNB-CU-CP informs the gNB-CU-UP to use a non-NULL integrity algorithm.

Requirement Name: Integrity protection of user data between the UE and the gNB-CU-UP.

Requirement Reference: TS 33.501 [2], clause 5.3.3

Requirement Description: The gNB supports integrity protection and replay protection of user data between the UE and the gNB as specified in TS 33.501 [2], clause 5.3.3.

NOTE 2: 	This requirement does not apply to the gNB that is used as a secondary node connecting to the EPC.

Threat References: TR 33.926 [4], clause Y.2.2.4 – User plane data integrity protection.

Test Case:

Test Name: TC-UP-DATA-INT_gNB-CU-UP

Purpose: To verify that the user data packets are integrity protected over the NG RAN air interface.

Pre-Condition:

-	The gNB-CU-UP network product shall be connected in emulated/real network environments. UE may be simulated.

-	Tester shall enable the user plane integrity protection and ensure NIA0 is not used.

-	Tester shall have knowledge of integrity algorithm and integrity protection keys.

-	The tester can capture the message via the NG RAN air interface, or can capture the message at the UE.

Execution Steps:

1. The NIA0 is disabled at UE and gNB-CU-UP.

2. The gNB-CU-UP is sent by the gNB-CU-CP a Bearer Context Setup Request message with integrity protection indication "on".

3. Check any User data sent by gNB-CU-UP after receiving the Bearer Context Setup Request message and before UE enters CM-Idle state is integrity protected.

Expected Results:

Any user plane packets sent between UE and gNB-CU-UP over the NG RAN air interface after gNB-CU-UP receives the Bearer Context Setup Request is integrity protected.

Expected format of evidence:

Evidence suitable for the interface e.g. Screenshot containing the operational results.

##### 6.2.2.1.7	Ciphering of user data between the UE and the gNB-CU-UP

NOTE: This is based on the security functional requirement on the gNB given in clause 4.2.2.1.7 of TS 33.511 [6] but modified as the gNB-CU-CP informs the gNB-CU-UP to use a non-NULL confidentiality algorithm.

Requirement Name: Ciphering of user data between the UE and the gNB-CU-UP

Requirement Reference: TS 33.501 [2], clause 5.3.2

Requirement Description: The gNB supports ciphering of user data between the UE and the gNB as specified in TS 33.501 [2], clause 5.3.2.

Threat References: TR 33.926 [4], clause Y.2.2.3 – User plane data confidentiality protection at gNB

Test Case:

Test Name: TC-UP-DATA-CIP_gNB

Purpose: To verify that the user data packets are confidentiality protected over the NG RAN air interface.

Pre-Condition:

-	The gNB-CU-UP network product shall be connected in emulated/real network environments. The UE may be simulated.

-	The tester shall have access to the NG RAN air interface or can capture the message at the UE.

Execution Steps:

1. The gNB-CU-UP is sent by the gNB-CU-CP a Bearer Context Setup Request message with ciphering protection indication "on".

2. Check any user data sent by the gNB-CU-UP after receiving the Bearer Context Setup Request message and before the UE enters into CM-Idle state.

Expected Results:

The user plane packets sent to the UE after the gNB-CU-UP receives the Bearer Context Setup Request is confidentiality protected.

Expected format of evidence:

Evidence suitable for the interface e.g. Screenshot containing the operational results.

### 6.2.3	Technical Baseline

The baseline technical requirements are identical to the ones for the gNB product class given in clause 4.2.3 of TS 33.511 [6].

### 6.2.4	Operating systems

There are no gNB-CU-UP specific additions to clause 4.2.4 of TS 33.117 [2].

### 6.2.5	Web servers

There are no gNB-CU-UP specific additions to clause 4.2.5 of TS 33.117 [2].

### 6.2.6	Network devices

These requirements are identical to the ones for the gNB product class given in clause 4.2.6 of TS 33.511 [6].

## 6.3	Adaptations of hardening requirements and related test cases

These requirements are identical to the ones for the gNB product class given in clause 4.3 of TS 33.511 [6].

### 6.4	Adaptations of basic vulnerability testing requirements and related test cases

### 6.4.1	Introduction

There are no gNB-CU-UP specific additions to clause 4.4.1 of TS 33.117 [2].

### 6.4.2	Port Scanning

There are no gNB-CU-UP specific additions to clause 4.4.2 of TS 33.117 [2].

### 6.4.3	Vulnerability scanning

There are no gNB-CU-UP specific additions to clause 4.4.3 of TS 33.117 [2].

### 6.4.4	Robustness and fuzz testing

The test cases under clause 4.4.4 of TS 33.117 [2] are applicable to gNB-CU-UP.

The interface defined for the gNB-CU-UP are in clause 4.2.3 of TS 23.501 [7] and in clause 4.1 of TS 38.300 [8].

According to clause 4.4.4 of TS 33.117 [2], the transport protocols available on the interfaces providing IP-based protocols need to be robustness tested. Following TCP/IP layer model and considering all the protocols over transport layer, for gNB-CU-UP, the following interfaces and protocols are in the scope of the testing:

For N3: the UDP and GTP-U protocols.

For F1-u: the UDP and GTP-U protocols.

For E1: the SCTP and E1AP protocols.

NOTE: There could be other interfaces and/or protocols requiring testing under clause 4.4.4 of TS 33.117 [2].

# 7	gNB-DU-specific security requirements and related test cases

## 7.1	Introduction

gNB-DU specific security requirements include both requirements derived from gNB-DU-specific security functional requirements as well as security requirements derived from threats specific to gNB-DU as described in TR 33.926 [4]. Generic security requirements and test cases common to other network product classes have been captured in TS 33.117 [2] and are not repeated in the present document.

## 7.2	Security functional adaptations of requirements and related test cases

### 7.2.1	Introduction

The present clause contains gNB-DU-specific security functional adaptations of requirements and related test cases.

### 7.2.2	Requirements and test cases deriving from 3GPP specifications

#### 7.2.2.1	Security functional requirements on the gNB-DU deriving from 3GPP specifications – TS 33.501 [3]

Editor's Note: The 'Z' in the clauses for the references to threats will need to be aligned with the final Annex allocation in TR 33.926.

##### 7.2.2.1.1	Control plane data confidentiality protection over F1 interface

NOTE: 	This is based on the security functional requirement on the gNB given in clause 4.2.2.1.16 of TS 33.511 [6] but modified as the gNB-DU only supports the F1 interface.

Requirement Name: Control plane data confidentiality protection over F1 interface

Requirement Reference: TS 33.501 [3], clauses 5.3.9.

Requirement Description: F1-C interface supports confidentiality, integrity and replay protection as specified in TS 33.501 [3], clauses 5.3.9.

Threat References: TR 33.926 [4], clause Z.2.2.1 – Control plane data confidentiality protection.

Test Case: The test case in subclause 4.2.3.2.4 of TS 33.117 [2]

##### 7.2.2.1.2	Control plane data integrity protection over F1 interface

NOTE: 	This is based on the security functional requirement on the gNB given in clause 4.2.2.1.17 of TS 33.511 [6] but modified as the gNB-DU only supports the F1 interface.

Requirement Name: Control plane data integrity protection over F1 interface

Requirement Reference: TS 33.501 [3], clauses 5.3.9.

Requirement Description: F1-C interface supports confidentiality, integrity and replay protection as specified in TS 33.501 [3], clauses 5.3.9.

Threat References: TR 33.926 [4], clause Z.2.2.2 – Control plane data integrity protection.

Test Case: The test case in subclause 4.2.3.2.4 of TS 33.117 [2].

##### 7.2.2.1.3	User plane data confidentiality protection over F1 interface

NOTE: 	This is based on the security functional requirement on the gNB given in clause 4.2.2.1.R of TS 33.511 [6] but modified as the gNB-DU only supports the F1 interface.

Editor's Note: The 'R' in the clause in the Note referring to TS 33.511 needs correction once the CR to include this test case in TS 33.511 is approved.

Requirement Name: User plane data confidentiality protection over F1 interface.

Requirement Reference: TS 33.501 [3], clauses 5.3.9.

Requirement Description: The gNB supports confidentiality, integrity and replay protection on the gNB DU-CU F1-U interface for user plane as specified in TS 33.501 [3], clauses 5.3.9.

Threat References: TR 33.926 [4], clause Z.2.2.3 – User plane data confidentiality protection at gNB.

Test Case: The test case in subclause 4.2.3.2.4 of TS 33.117 [2].

##### 7.2.2.1.4	User plane data integrity protection over F1 interface

NOTE: 	This is based on the security functional requirement on the gNB given in clause 4.2.2.1.S of TS 33.511 [6] but modified as the gNB-DU only supports the F1 interface.

Editor's Note: The 'S' in the clause in the Note referring to TS 33.511 needs correction once the CR to include this test case in TS 33.511 is approved.

Requirement Name: User plane data integrity protection over F1 interface.

Requirement Reference: TS 33.501 [3], clauses 5.3.9.

Requirement Description: The gNB supports confidentiality, integrity and replay protection on the gNB DU-CU F1-U interface for user plane as specified in TS 33.501 [3], clauses 5.3.9.

Threat References: TR 33.926 [4], clause Z.2.2.4 – User plane data integrity protection.

Test Case: The test case in subclause 4.2.3.2.4 of TS 33.117 [2].

### 7.2.3	Technical Baseline

The baseline technical requirements are identical to the ones for the gNB product class given in clause 4.2.3 of TS 33.511 [6].

### 7.2.4	Operating systems

These requirements are identical to the ones for the gNB product class given in clause 4.2.4 of TS 33.511 [6].

### 7.2.5	Web servers

There are no gNB-DU specific additions to clause 4.2.5 of TS 33.117 [2].

### 7.2.6	Network devices

These requirements are identical to the ones for the gNB product class given in clause 4.2.6 of TS 33.511 [6].

## 7.3	Adaptations of hardening requirements and related test cases

These requirements are identical to the ones for the gNB product class given in clause 4.3 of TS 33.511 [6].

## 7.4	Adaptations of basic vulnerability testing requirements and related test cases

### 7.4.1	Introduction

There are no gNB-DU specific additions to clause 4.4.1 of TS 33.117 [2].

### 7.4.2	Port Scanning

There are no gNB-DU specific additions to clause 4.4.2 of TS 33.117 [2].

### 7.4.3	Vulnerability scanning

There are no gNB-DU specific additions to clause 4.4.3 of TS 33.117 [2].

### 7.4.4	Robustness and fuzz testing

The test cases under clause 4.4.4 of TS 33.117 [2] are applicable to gNB-DU.

The interface defined for the gNB-DU are in clause 4.1 of TS 38.300 [7].

According to clause 4.4.4 of TS 33.117 [2], the transport protocols available on the interfaces providing IP-based protocols need to be robustness tested. Following TCP/IP layer model and considering all the protocols over transport layer, for gNB-DU, the following interfaces and protocols are in the scope of the testing:

For F1-u: the UDP and GTP-U protocols.

For F1-c: the SCTP and F1AP protocols.

NOTE: There could be other interfaces and/or protocols requiring testing under clause 4.4.4 of TS 33.117 [2].

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-05 | SA3#107-e | S3-221201 |  |  |  | Skeleton (S3-221196) plus S3-220989. | 0.1.0 |
| 2022-09 | SA3#108-e | S3-222321 |  |  |  | Incorporating S3-221824, S3-222309, S3-221310, S3-222312 and S3-222313. | 0.2.0 |
| 2022-11 | SA3#109 | S3-224103 |  |  |  | Incorporating S3-223346, S3-223348, S3-223349, S3-223350, S3-223352, S3-223353 and S3-223354. | 0.3.0 |
| 2023-02 | SA3#110 | S3-230786 |  |  |  | Changing the TS number from TS 33.742 to TS 33.523 due to mis- allocated specification number. | 0.4.0 |
| 2023-02 | SA3#110 | S3-231498 |  |  |  | Incorporating S3-230789, S3-230790, S3-230794, S3-231470, S3-231471 and S3-231472. | 0.5.0 |
| 2023-03 | SA#99 | SP-230128 |  |  |  | Presented for information. | 1.0.0 |
| 2023-05 | SA#99 |  |  |  |  | EditHelp review. | 1.0.1 |
| 2023-05 | SA3#111 | S3-233332 |  |  |  | Incorporating S3-232542, S3-233330 and S3-233331. | 1.1.0 |